3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800150

# San Diego, US, 22 - 26 Jan, 2018

Agenda item:	4.3.2.3.2

Source: 	Intel Corporation

Title: 	MPR evaluation results for FR2

Document for:	Approval

# Introduction

This contribution provides updated MPR values to our previous submitted MPR results in [1].  In last meeting, we encouraged companies to evaluate the current mmWave IBE requirement [2] which provides relaxation on 16QAM and higher modulations but no relaxation on QPSK. In this meeting, we submit another contribution to propose a modified IBE formula [3]. The MPR values we proposed in this contribution are based on modified IBE formula in [3].

# Discussion

In Reno meeting, the WF on pi/2 BPSK spectrum shaping and power class in FR2 [4] has following agreements:

  - The waveform defined by BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0 is the reference waveform with 0dB MPR and is used for the power class definition

  - MPR requirements for waveforms that operate at higher output power than the reference waveform can be defined with a negative value

  - The Pcmax requirement includes an additional term to account for negative MPR in both Pcmax_L and Pcmax_H terms

In this contribution, we use agreed reference waveform for PA calibration. Since for BW=100Mhz, SCS=60Khz, the maximum transmission bandwidth configuration NRB = 132, while reference waveform has only 128 RBs for DFT-S-OFDM modulation, there are 4 unused RBs. Whether or not to check IBE on these 4 unused RBs will impact PA calibration. Without IBE check on reference waveform, we observed there is a significant power discontinuity, around 1.5dB, between reference waveform and the DFT-S-OFDM waveform with 125RB configurations where IBE needs to be checked. In order to avoid this power discontinuity and also to consider future service deployments on these unused RBs, we make the following proposal.

Proposal 1: IBE needs to be checked on any unused RBs, including reference waveform.

Based on current IBE formula [5] in mmWave, which was copied from NR sub-6 and LTE and still within brackets, a few companies claimed IBE is the limiting factor for QPSK waveforms. Since in mmWave, the ACLR requirement has been much relaxed in consideration of analog beamforming in UE co-existence study, IBE relaxation was considered with relaxation accordingly in Table 6.4.2.3.1-1 in [5] where the IBE relaxation has been made from PRB - 30 dB in NR sub-6 to PRB - 17 dB in NR mmWave. This IBE relaxation seems only effective to 16QAM and higher modulations, but higher modulations are normally limited by EVM not IBE. But for QPSK waveforms, PRB - 17 dB does not provide enough relaxation. By observing this, in another Intel submitted contribution [3], we have proposed relaxed IBE in general formula term as well combined with PRB - 17 dB to relax QPSK waveforms.

Proposal 2: mmWave IBE general term needs relaxation by considering mmWave analog beamforming feature in FR2 MPR evaluations.

In this contribution, we generated MPR by following proposals 1 and 2.

We also consider PA calibration gap for FR2 [6]. Since PA calibration gap can provide dedicated air-time for runtime PA calibration, it is expected that PA can achieve better linearity when PA calibration gap is implemented. Otherwise if there is no PA calibration gap, PA may experience certain degree of degradation in linearity. In later case, larger MPR values are needed. In this contribution, we consider these two scenarios, and provide two set of MPR values accordingly.

## Simulation assumptions

Based on our mmWave PA model, the MPR evaluation was performed by using 8 PAs, 4 for each polarizations within an antenna array panel.

Our simulation assumptions are captured below, following the spec [5], otherwise specified:

- EVM requirement: table 6.4.2.1.1-1 [5].

| Parameter | Unit | Average EVM Level (%) |
| --- | --- | --- |
| Pi/2 BPSK | % | 30 |
| Pi/2 BPSK with pulse shaping | % | 30 |
| QPSK | % | [17.5] |
| 16 QAM | % | [12.5] |
| 64 QAM | % | 8 |

- IBE requirement: table 6.4.2.3.1-1 [5], except general term in IBE which followed IBE modifications in [3].

- ACLR = 16dBc: table 6.6.2.3.1-1 [5], since we simulated 40G band.

- The maximum transmission BW configuration NRB:  Table 5.3.2-1 in [5]

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

- ACLR measurement BW is centered at channel center and is determined by Channel BW – 2* the minimum guardband between 60KHz SCS and 120KHz SCS for that channel BW. The minimum guardband in KHz is from table 5.3.3-1 [5].

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

- SEM requirement:  table 6.5.2.1.1-1 in [5], where ΔfOOB is up to 200% channel BW away from +/- channel edge.

| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |

## Simulation results

### MPR values with PA calibration gap

First, we provide MPR values assuming calibration gap exists. In this case, we extract PA characteristics and apply them in the simulations. PA is assumed to be compensated to a better state by using run-time training during calibration gap, e.g. by real-time DPD algorithm. The PA characteristics are assumed no changes during PA operations. A file containing simulation results is attached in the Appendix. We also notice that by real Tx waveform measurement, some MPR values need to be justified in order to close the gap between simulation platform and real system.

The MPR values are provided in a similar way as in sub6 in table 1, but inner and outer allocations are not differentiated. Only the larger MPR values is taken for both RB allocation types for each modulations.

Table 1. Evaluated MPR values for mmWave with PA calibration gap

| WF type | modulation | MPR For all BW and SCS | Limiting Factors |
| --- | --- | --- | --- |
| DFT-s-OFDM | Pulse Shaped Pi/2 BPSK | -0.5 | ACLR/SEM/EVM |
|  | Pi/2 BPSK | 2.0 | SEM |
|  | QPSK | 2.0 | SEM |
|  | 16QAM | 3.0 | EVM |
|  | 64QAM | 5.5 | EVM |
| CP-OFDM | QPSK | 4.0 | EVM |
|  | 16QAM | 4.5 | EVM |
|  | 64QAM | 8.0 | EVM |

### MPR values without PA calibration gap

PA characteristics may change over time, temperature, voltage and among other factors. Without PA calibration gap, DPD coefficients may not compensate well the real-time PA non-linearity. This mismatch will degrade PA performance and further power backoff is needed to meet ACLR/SEM/IBE/EVM and spurious emission requirements. So larger MPR values are expected. The following table contains proposed MPR values when calibration gap does not exist.

Table 2. Evaluated MPR values for mmWave without PA calibration gap

| WF type | modulation | MPR For all BW and SCS |
| --- | --- | --- |
| DFT-s-OFDM | Pulse Shaped Pi/2 BPSK | 0 |
|  | Pi/2 BPSK | 2.5 |
|  | QPSK | 2.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 7.0 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 5.5 |
|  | 64QAM | 9.5 |

### MPR value selection

Consider changing in PA characteristics is a slow process over time, calibration gap can be implemented in a very low duty cycle with negligible impact on the throughput. We have following proposal.

Proposal 3: PA calibration gap should be supported and MPR values with PA calibration gap should be considered.

# Conclusion

In this contribution, we provide our views on FR2 MPR values. We have the following proposals:

Proposal 1: IBE needs to be checked on any unused RBs, including reference waveform.

Proposal 2: mmWave IBE general term needs relaxation by considering mmWave analog beamforming feature in FR2 MPR evaluations.

In addition of proposals 1 and 2, we also provide our MPR values for two scenarios, MPR values with PA calibration gap and MPR values without PA calibration gap. And we have proposal 3 as follow.

Proposal 3: PA calibration gap should be supported and MPR values with PA calibration gap should be considered.

# References

[1] R4-1712330, ‘MPR simulation results for FR2’, Intel Corp.

[2] R4-1714189, ‘UE RF Evening AH’, Qualcomm Incorporated.

[3] R4-1800119, ‘on IBE for FR2’, Intel Corp.

[4] R4-1714445, ‘WF on Pi/2 BPSK spectrum shaping and power class for FR2’, Intel Corp, Apple, Qualcomm, LGE, Nokia, Huawei, Motorola Mobility.

[5] 3GPP TS 38.101-2 V1.0.0 (2017-12)

[6] R4-1712882, ‘PA calibration gap for FR2’, Intel Corp.

# Appendix

Attached is the file containing MPR simulation results with PA calibration gap.