3GPP TSG RAN WG4 Meeting AH-1801	R4-1800151

San Diego, US, 22 - 26 January 2018

Agenda item:	4.6.5.1

Title: 	Simulation results for PBCH Acquisition

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4 #85 the simulation assumptions for PBCH decoding were agreed up on to derive the basic SI reading time [1]. Based on the agreements, companies were encouraged to provide simulation results for PBCH acquisition for 2 cases.

The proposed simulation assumptions are used to derive the basic SI reading (PBCH acquisition) time. Case1: PBCH reading Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH. Case2: PBCH-DMRS time index reading (Without decode PBCH data) DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

In this contribution we present simulation results for PBCH acquisition for sub-6GHz and mmWave.

# Simulation Results

As agreed in the simulation assumption, the PBCH acquisition time for PBCH-DMRS time index reading and PBCH reading will be used to derive the basic SI reading time. The simulation assumptions agreed upon in [1] are listed in the Appendix. Some additional assumptions include:

- 2 RX antenna

- For FR2 results with CDL-C channel model are with ideal Rx beamforming (i.e., no beam sweep)

- Single shot detection – no soft combining of PBCH

## PBCH Reading

The basic SI reading time is the number of SS-Blocks required to decode PBCH with 99% accuracy. The results for PBCH decoding in terms of number of SS-Blocks is presented in Table 1.

Table 1: PBCH reading time

| SCS | Channel | SNR: -10 dB | SNR: -8 dB | SNR: -6 dB | SNR: -4dB | SNR: -2dB | SNR: 0dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz [2 GHz] | EPA5 | 5 | 3 | 2 | 2 | 1 | 1 |
|  | ETU70 | 7 | 3 | 2 | 1 | 1 | 1 |
| 30KHz [2 GHz] | EPA5 | 5 | 3 | 2 | 2 | 1 | 1 |
|  | ETU70 | 10 | 4 | 2 | 1 | 1 | 1 |
| 120 KHz [30 GHz] | TDL-C | 6 | 3 | 2 | 1 | 1 | 1 |
|  | CDL-C | 8 | 4 | 2 | 2 | 1 | 1 |
| 240 KHz [30 GHz] | TDL-C | 8 | 4 | 2 | 1 | 1 | 1 |
|  | CDL-C | 9 | 4 | 2 | 2 | 1 | 1 |

In FR2, the PBCH decoding time also corresponds to the SSB block index detection and can be used to derive TSSB_time_index.

Observation #1: The basic SI reading time is 4 SS-Blocks at -8dB SNR across all scenarios for 2 Rx antenna for FR1 and FR2

Observation #2: The PBCH decoding time is very similar between the 2 SCS in the same frequency range

## PBCH-DMRS Time Index Reading

The DMRS time index reading time is defined as the number of SS-blocks required for 99% successful reading of DMRS time index. The results for DMRS time index acquisition for the base case of 2 Rx with no Rx beamforming in terms of number of SS-bursts are summarized in Table 1.

Table 1: DMRS time index reading time

| SCS | Channel | SNR: -10 dB | SNR: -8 dB | SNR: -6 dB | SNR: -4dB | SNR: -2dB | SNR: 0dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz [2 GHz] | EPA5 | 2 | 2 | 1 | 1 | 1 | 1 |
|  | ETU70 | 2 | 1 | 1 | 1 | 1 | 1 |
| 30KHz [2 GHz] | EPA5 | 2 | 2 | 1 | 1 | 1 | 1 |
|  | ETU70 | 2 | 2 | 1 | 1 | 1 | 1 |
| 120 KHz [30 GHz] | TDL-C | 2 | 1 | 1 | 1 | 1 | 1 |
|  | CDL-C | 2 | 1 | 1 | 1 | 1 | 1 |
| 240 KHz [30 GHz] | TDL-C | 2 | 2 | 1 | 1 | 1 | 1 |
|  | CDL-C | 2 | 2 | 1 | 1 | 1 | 1 |

In FR1 the DMRS time index reading time corresponds to the SS-Block index detection and can be used to derive TSSB_time_index.

.

Observation #3: The DMRS time index reading time is 2 SS-Blocks at -8dB SNR across all scenarios for 2 Rx antenna for FR1 and FR2

Observation #4: The DMRS time index reading time is very similar between the 2 SCS in the same frequency range

# Conclusions

In this paper we present the PBCH and DMRS time index reading time for 2GHz for FR1 and 30GHz for FR2 and have the following observations.

Observation #1: The basic SI reading time is 4 SS-Blocks at -8dB SNR across all scenarios for 2 Rx antenna for FR1 and FR2

Observation #2: The PBCH decoding time is very similar between the 2 SCS in the same frequency range

Observation #3: The DMRS time index reading time is 2 SS-Blocks at -8dB SNR across all scenarios for 2 Rx antenna for FR1 and FR2

Observation #4: The DMRS time index reading time is very similar between the 2 SCS in the same frequency range

It is suggested that these results be considered while defining requirements in RAN4.

# References

- R4-1713437, “Updated PBCH Simulation assumptions after redesign”, Huawei, HiSilicon

# Appendix

The simulation assumptions for PBCH SI reading tests are listed in Table below.

Table A-4: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 2GHz; 30GHz |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 2GHz; 120 kHz; 240 kHz for 30GHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2, 4 (uncorrelated. Option 1: With receiver beamforming Option 2: Without receiver beamforming) |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 0 |
| PBCH symbols within the SS block |  | According to the RAN1 agreements |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition | - | EPA5, ETU70 for 2GHz CDL/TDL-C with desired RMS delay spread 100ns Note1 for 30GHz |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) FFS Soft- combining |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. |  |  |