3GPP TSG RAN WG4 Meeting AH-1801	R4-1800152

San Diego, US, 22 - 26 January 2018

Agenda item:	4.6.5.1

Title: 	On Cell Identification delay requirements without gap

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In TS 38.133 there are 2 sets of requirements for cell identification for intra-frequency measurements without gap [1].

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if the UE has already identified an SS block with different time index and the same physical cell ID, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index Tidentify_intra_without_index = TPSS/SSS_sync + T SSB_measurement_period ms Tidentify_intra_with_index = TPSS/SSS_sync + T SSB_measurement_period + TSSB_time_index TPSS/SSS_sync is the PSS/SSS detection delay T SSB_measurement_period is the measurement period of SSB based measurement TSSB_time_index is the SSB Index acquisition delay

In this contribution we present our views on cell identification requirements. All the requirements are assuming no configured SCell or E-UTRA Scell.

# Discussion

## PSS/SSS Detection

In RAN4 #85 companies submitted simulation results for PSS/SSS detection and an agreement was reached to define the requirement in FR1 as:

TPSS/SSS_sync = max(600ms,[5 or 6]x SMTC period) ms

Based on results in [2] and considering implementation and RF margin we recommend that the requirement be set as:

TPSS/SSS_sync = max(600ms,6* SMTC period) ms

Proposal#1: In FR1 the PSS/SSS detection requirement be set as TPSS/SSS_sync = max(600ms,6* SMTC_period) ms

In [3] we present results for PSS/SSS detection in FR2. The results show that 2 SS-Blocks are required for PSS/SSS detection at -6dB SINR. These results are with ideal Rx beamforming and don’t consider delay due to beam sweep procedure.  Considering the delay in beam sweeping, and implementation margin, we recommend that the requirement in FR2 be set as:

TPSS/SSS_sync = max(600ms,[3*N]x SMTC_period) ms

We propose that N be set as 8.

Proposal#2: In FR2 the PSS/SSS detection requirement be set as TPSS/SSS_sync = max(600ms,3*N* SMTC_period) ms, where N=8

## SSB Index Detection

In RAN4#85 [4] the simulations assumptions to derive the SSB Index detection requirements were agreed upon and companies were encouraged to provide simulation results in order to derive the requirement for TSSB_time_index . In FR1 the SSB Index detection time can be derived from the DMRS time index reading time and in FR2 the SSB Index detection time corresponds to the basic SI reading time.

Based on results provided in [5], 2 SS-blocks are required for DMRS TI detection with 99% accuracy at -8dB. For FR1 we recommend that TSSB_time_index be set as 2*SMTC_period

Proposal#3: In FR1 the SSB Index detection requirements be set as TSSB_time_index = 2*SMTC_period

In FR2, assuming ideal Rx beamforming, the number of SS-Blocks required for PBCH decoding with 99% success rate is 4 at -8dB. Considering the processing delay for beam sweep, we recommend that the requirement in FR2 be set as 4*N*SMTC_period. We recommend that N be set as 8.

Proposal#4: In FR2 the SSB Index detection requirements be set as TSSB_time_index = 4*N*SMTC_period, where N=8

## SSB Measurement Period

In RAN4#85 the measurement period for intra-frequency measurements was discussed and an agreement was reached to set the requirement in FR1 as:

T SSB_measurement_period = max(200,[5] x SMTC period) ms

Proposal#5: In FR1 the measurement period for intra-frequency measurements be set as TSSB_measurement_period = max(200, 5*SMTC_period) ms

In [6] we present results for SSB based RSRP measurement in FR2. These results are with ideal Rx beamforming and don’t consider the delay in beam sweep. Based on the results, we recommend setting the measurement period requirement in FR2 as T SSB_measurement_period = max(200, 3*N*SMTC_period) ms. We recommend that N be set as 8.

Proposal#6: In FR2 the measurement period for intra-frequency measurements be set as TSSB_measurement_period = max(200, 3*N*SMTC_period) ms , where N=8

# Conclusion

Based on the discussion presented above, we have the following proposals for delay requirements in FR1 and FR2:

Proposal#1: In FR1 the PSS/SSS detection requirement be set as TPSS/SSS_sync = max(600ms,6* SMTC_period) ms Proposal#2: In FR2 the PSS/SSS detection requirement be set as TPSS/SSS_sync = max(600ms,3*N* SMTC_period) ms, where N=8 Proposal#3: In FR1 the SSB Index detection requirements be set as TSSB_time_index = 2*SMTC_period Proposal#4: In FR2 the SSB Index detection requirements be set as TSSB_time_index = 4*N*SMTC_period, where N=8 Proposal#5: In FR1 the measurement period for intra-frequency measurements be set as TSSB_measurement_period = max(200, 5*SMTC_period) ms Proposal#6: In FR2 the measurement period for intra-frequency measurements be set as TSSB_measurement_period = max(200, 3*N*SMTC_period) ms , where N=8

# References

- TS 38.133, “NR; Requirements for support of radio resource management”

- R4-1800173, “Updated PSS SSS detection results for NR link level simulation results in FR1”, Intel

- R4-1800174, “PSS SSS detection results for NR link level simulation results in FR2”, Intel

- R4-1714251, “Way forward on the SSB time index acquisition time”, Huawei, HiSilicon, Ericsson, Intel, MediaTek

- R4-1800151, “Simulation Results for PBCH acquisition”, Intel

- R4-1800340, “SS block RSRP link level simulation result based on beamforming”, Intel