3GPP TSG-RAN WG4 Meeting AH-1801                                 R4-1800203

San Diego, Califonia, USA, 22nd – 26th Jan, 2018

Agenda item:			4.3.3.1

Source:			Samsung

Title:	Discussion on UE REFSENS and updated simulation results

Document for:		Discussion

# 1	Introduction

During RAN4 #85, the SNR for the NR UE REFSENS requirements was further discussed, and the following WF [1] was approved with simulation assumption for alignment purpose as below, which is supplementary to the WF agreed in NR Ad-Hoc #3 [2] and WF agreed in RAN4 #84Bis [3].

Background: - The simulation assumptions were agreed as following from previous meeting. - Channel bandwidth and SCS - Provide REFSENS SNR analysis for at least for the following CBW/SCS combinations - Set 1: FR1, 10 MHz CBW + 15 kHz SCS (52 PRBs) - Set 2: FR1, 50 MHz CBW + 30 kHz SCS (133 PRBs) - Set 3: FR2, 50 MHz CBW + 60 kHz SCS (66 PRBs) - Interested companies are encouraged to provide results for following combinations - FR1 - FR2 - Following the previous slides as agreed simulation assumption - DL control channel configuration: - 3 symbols per-slot are used for DL control channel for CBW=5MHz - 2 symbols per-slot are used for DL control channel for CBW>5MHz - SS Block configuration: - Schedule PDSCH in slots without SS blocks - TRS and CSI-RS configuration - Schedule PDSCH transmissions in the slots without TRS / CSI-RS. - Practical channel and noise estimation are captured in the REFSENS SNR term. Online agreements made in RAN4#85: - The following agreements were made and captured in the chairman’s notes. - Use CR=1/3 for the REFSENS SNR simulation assumption. - Use PRB bundling size 2 for the REFSENS SNR simulation assumption. - Still use -1dB SNR value for the NR UE REFSENS analysis target - Agree on the following REFSENS definition procedure - Use tentative SNR = -1 dB as the assumption for NR RF REFSENS definition - Define REFSENS FRC and define corresponding SNR value (SNRFinal) as per RAN1 work progress (final SNR value may be different from tentative SNR) - Replace the tentative SNR value with final SNR value once it is finalized as follows: REFSENSFinal = REFSENS + SNRFinal – SNR WF for further simulation assumptions - Take the following configuration as the working assumption unless further issues are identified. - DMRS configuration: - Option 1: Front loaded Type 1, DL-DMRS-len = 1, DL-DMRS-add-pos = 2 - Option 2: Front loaded Type 1, DL-DMRS-len = 1, DL-DMRS-add-pos = 3 - Option 3: Front loaded Type 1, DL-DMRS-len = 1, DL-DMRS-add-pos = 1 - PTRS configuration - FR1: No PTRS - FR2: - Option 1: No PTRS - Option 2: PTRS is configured - KPTRS = 2 (transmitted in every 2nd RB) - LPTRS = 1 (transmitted in each OFDM symbol) - Other options are not precluded in case issues are identified by companies. - Both ideal and practical channel estimations results are to be provided for alignment purposes. - Keep previous agreement: Practical channel and noise estimation are captured in the REFSENS SNR term. WF for IM - Companies are encouraged to bring more input in next meeting for components contributing to IM. - FR1 - Remaining impairments are taken into account in the IM term - FR2 - Remaining impairments are added on top of SNR.

In this contribution, we provide our views on the SNR FRC parameters and our further updated simulation results based upon our previous contribution [4].

# Simulation Results Update

#### 2.1 Simulation Assumption

The following are the simulation assumptions and FRC used in our simulations:

Table 2-1: Simulation assumptions for required SNR

| Parameters | Assumption |
| --- | --- |
| Propagation condition | AWGN |
| Antenna configuration | 1x1 |
| Modulation | QPSK |
| Coding rate | 1/3 |
| Transmission mode | 1 layer |
| PDCCH symbol | 2 |
| PT-RS | No PT-RS configuration |
| HARQ | None |

Table 2-2: FRC used in the simulation

|  |  | Set1 | Set2 | Set3 |
| --- | --- | --- | --- | --- |
| CBW(MHz) |  | 10 | 50 | 50 |
| SCS(kHz) |  | 15 | 30 | 60 |
| RB |  | 52 | 133 | 66 |
| Modulation order |  | QPSK | QPSK | QPSK |
| Channel Bits Length |  |  |  |  |
| DMRS Option-1 (DL-DMRS-add-pos = 2) |  | 13728 | 35112 | 17424 |
| DMRS Option-2 (DL-DMRS-add-pos = 3) |  | 13104 | 33516 | 16632 |
| DMRS Option-3 (DL-DMRS-add-pos = 1) |  | 12480 | 31920 | 15840 |
| Effective Transmission Bits Size |  |  |  |  |
| MCS 4 | DMRS Option-1 | 4504 | 11592 | 5656 |
|  | DMRS Option-2 | 3384 | 8776 | 4248 |
|  | DMRS Option-3 | 3384 | 8776 | 4248 |
| MCS 5 | DMRS Option-1 | 5528 | 14152 | 7064 |
|  | DMRS Option-2 | 4120 | 10568 | 5272 |
|  | DMRS Option-3 | 4120 | 10568 | 5272 |
| MCS 6 | DMRS Option-1 | 6552 | 11324 | 8216 |
|  | DMRS Option-2 | 4888 | 12616 | 6296 |
|  | DMRS Option-3 | 4888 | 12616 | 6296 |
| Effective Coding Rate |  |  |  |  |
| MCS 4 | DMRS Option-1 | 0.3281 | 0.3301 | 0.3246 |
|  | DMRS Option-2 | 0.2582 | 0.2618 | 0.2554 |
|  | DMRS Option-3 | 0.2712 | 0.2749 | 0.2682 |
| MCS 5 | DMRS Option-1 | 0.4027 | 0.4031 | 0.4054 |
|  | DMRS Option-2 | 0.3144 | 0.3153 | 0.3170 |
|  | DMRS Option-3 | 0.3301 | 0.3311 | 0.3328 |
| MCS 6 | DMRS Option-1 | 0.4773 | 0.4838 | 0.4715 |
|  | DMRS Option-2 | 0.3730 | 0.3764 | 0.3785 |
|  | DMRS Option-3 | 0.3917 | 0.3952 | 0.3975 |
| CFI |  | 2 | 2 | 2 |
| CRC for TB |  | 24 | 24 | 24 |
| CRC for CB |  | 0 | 24 | 0 |
| # of CB |  | 1 | 2 | 1 |
| Base Graph Type |  | BG1 | BG1 | BG1 |

As agreed in previous RAN4 meeting, the target coding rate should be 1/3; therefore, based on the FRC table above which is calculated with TBSize based on latest approved TS 38.214[5], in each DMRS option, we choose the highlight-yellow code rates which are close to 1/3 for evaluation purpose in each set, e.g., for Set 1 DMRS Option-1 (DL-DMRS-add-pos = 2), the resultant coding rates for MCS 4, 5, 6 will be 0.3281, 0.4027 and 0.4773, among which the former two are simulated and the last one is not due to its being far from 1/3 target coding rate.

#### 2.2 Simulation Results

Following are the simulation results for cases of three sets on three DMRS configuration options.

- Set 1:

Figure 1: Simulation results for Set1 Option1 cases			Figure 2: Simulation results for Set1 Option2 cases

Figure 3: Simulation results for Set1 Option3 cases

- Set 2:

Figure 4: Simulation results for Set2 Option1 cases		  Figure 5: Simulation results for Set2 Option2 cases

Figure 6: Simulation results for Set2 Option3 cases

- Set 3:

Figure 7: Simulation results for Set3 Option1 cases			Figure 8: Simulation results for Set3 Option2 cases

Figure 9: Simulation results for Set3 Option3 cases

Considering the agreement from RAN4 #85, i.e., “Still use -1dB SNR value for the NR UE REFSENS analysis target”, we propose to firstly down select the MCS for Set 1-3 and DMRS Option 1-3 to make sure the -1dB SNR be the closest, and based on this principle, the following table can be reached.

Observation 1: Based on TS 38.213’s definition, the MCS down-selection can be obtained for 1/3 Target Code Rate and tentative SNR target -1dB:

Table 2-3 Summary of MCS Down-selection (for 1/3 Target Code Rate and SNR= -1dB target)

| Scenarios | DMRS pattern |  |  |
| --- | --- | --- | --- |
|  | Option 1 (DL-DMRS-add-pos = 2) | Option 2 (DL-DMRS-add-pos = 3) | Option 3 (DL-DMRS-add-pos = 1) |
| Set 1 | MCS4 | MCS5 | MCS5 |
| Set 2 | MCS4 | MCS5 | MCS5 |
| Set 3 | MCS4 | MCS5 | MCS5 |

Observation 2: Based on the above MCS down-selection, the required SNR values for 5% BLER (with practical channel estimation) can be summarized in the following table.

Table 2-4 Summary of SNR values (for 5% BLER), based on above MCS down-selection

| Scenarios | DMRS pattern |  |  |
| --- | --- | --- | --- |
|  | Option 1 (DL-DMRS-add-pos = 2) | Option 2 (DL-DMRS-add-pos = 3) | Option 3 (DL-DMRS-add-pos = 1) |
| Set 1 | -0.8 | -1.2 | -1.3 |
| Set 2 | -0.8 | -1.3 | -1.2 |
| Set 3 | -0.9 | -1.1 | -1.2 |

Based on the above analysis, we would like to provide our preference among DMRS options, i.e., DMRS option-1 (DL-DMRS-add-pos = 2), which can both test the practical channel estimation performance and SNR values closer to target -1dB.

Proposal 1: Choose DMRS Option-1 (DL-DMRS-add-pos = 2) for REFSENS FRC.

# Discussion on Implementation Margin

Based on our understanding, the implementation margin part discussed here is only targeted to cover the baseband part while the RF margin will be tackled by RF experts separately. Considering the implementation margin usually observed between floating and fixed point implementation, and also practical tracking for baseband part (including AGC, sync-tracking loop etc.), we believe 2.5dB baseband implementation margin is reasonable option to be adopted. .

Proposal 2: Choose baseband implementation margin IM=2.5dB.

# Conclusion

In this paper, we provided our views on the SNR FRC parameters and our further updated simulation results as follow:

Observation 1: Based on TS 38.213’s definition, the MCS down-selection can be obtained for 1/3 Target Code Rate and tentative SNR target -1dB:

Table 2-3 Summary of MCS Down-selection (for 1/3 Target Code Rate and SNR= -1dB target)

| Scenarios | DMRS pattern |  |  |
| --- | --- | --- | --- |
|  | Option 1 (DL-DMRS-add-pos = 2) | Option 2 (DL-DMRS-add-pos = 3) | Option 3 (DL-DMRS-add-pos = 1) |
| Set 1 | MCS4 | MCS5 | MCS5 |
| Set 2 | MCS4 | MCS5 | MCS5 |
| Set 3 | MCS4 | MCS5 | MCS5 |

Observation 2: Based on the above MCS down-selection, the required SNR values for 5% BLER (with practical channel estimation) can be summarized in the following table.

Table 2-4 Summary of SNR values (for 5% BLER), based on above MCS down-selection

| Scenarios | DMRS pattern |  |  |
| --- | --- | --- | --- |
|  | Option 1 (DL-DMRS-add-pos = 2) | Option 2 (DL-DMRS-add-pos = 3) | Option 3 (DL-DMRS-add-pos = 1) |
| Set 1 | -0.8 | -1.2 | -1.3 |
| Set 2 | -0.8 | -1.3 | -1.2 |
| Set 3 | -0.9 | -1.1 | -1.2 |

Proposal 1: Choose DMRS Option-1 (DL-DMRS-add-pos = 2) for REFSENS FRC.

Proposal 2: Choose baseband implementation margin IM=2.5dB.

# Reference

[1] R4-1714028, “WF on UE REFSENS simulation assumptions”, Ericsson.

[2] R4-1709398, “Way forward on NR REFSENS SNR simulation assumptions”, Huawei, Intel.

[3] R4-1711625, “WF on NR UE REFSENS SNR simulation assumptions”, Intel Corporation, Ericsson, Huawei.

[4] R4-1712238, “Discussion on UE REFSENS and simulation results”, Samsung.

[5] 3GPP TS 38.214 V2.0.0: "NR; Physical layer procedures for data”