3GPP TSG- RAN Working Group 4 (Radio) meeting AH-1801	R4-1800258

# San Diego, USA, 22 - 26 January 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for completed EN-DC of LTE 1CC + NR 1band for TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To specify completed DC configurations into TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add below DC configurations. DC_1A_n77A DC_1A_n78A DC_1A_n79A DC_1A_n257A DC_3A_n77A DC_3A_n78A DC_3A_n79A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A DC_19A_n77A DC_19A_n78A DC_19A_n79A DC_19A_n257A DC_20A_n78A DC_21A_n77A DC_21A_n78A DC_21A_n79A DC_21A_n257A DC_28A_n77A DC_28A_n78A DC_28A_n79A DC_28A_n257A DC_42A_n77A DC_42A_n78A DC_42A_n79A DC_42A_n257A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | These DC configurations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change - Unchanged section Omitted>

5.3.2	EN-DC configurations

Editors note: Explanatory text is needed

5.3.2.1	Intra-band contiguous EN-DC configurations

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.101 and for NR operating bands in TS 38.101-1.

5.3.2.1-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.2	Intra-band non-contiguous EN-DC configurations

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.101 and for NR operating bands in TS 38.101-1.

5.3.2.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.3	Inter-band EN-DC configurations

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 and for NR operating bands and CA configurations in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

5.3.2.3.1			Inter-band EN-DC configurations (two bands)

5.3.2.3.1-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n77A | DC_1A_n77A | 1 | 77 |
| DC_1A_n78A | DC_1A_n78A | 1 | 78 |
| DC_1A_n79A | DC_1A_n79A | 1 | 79 |
| DC_1A_n257A | DC_1A_n257A | 1 | 257 |
| DC_3A_n77A | DC_3A_n77A | 3 | 77 |
| DC_3A_n78A | DC_3A_n78A | 3 | 78 |
| DC_3A_n79A | DC_3A_n79A | 3 | 79 |
| DC_3A_n257A | DC_3A_n257A | 3 | 257 |
| DC_5A_n78A | DC_5A_n78A | 5 | 78 |
| DC_5A_n257A | DC_5A_n257A | 5 | 257 |
| DC_7A_n78A | DC_7A_n78A | 7 | 78 |
| DC_7A_n257A | DC_7A_n257A | 7 | 257 |
| DC_19A_n77A | DC_19A_n77A | 19 | 77 |
| DC_19A_n78A | DC_19A_n78A | 19 | 78 |
| DC_19A_n79A | DC_19A_n79A | 19 | 79 |
| DC_19A_n257A | DC_19A_n257A | 19 | 257 |
| DC_20A_n78A | DC_20A_n78A | 20 | 78 |
| DC_21A_n77A | DC_21A_n77A | 21 | 77 |
| DC_21A_n78A | DC_21A_n78A | 21 | 78 |
| DC_21A_n79A | DC_21A_n79A | 21 | 79 |
| DC_21A_n257A | DC_21A_n257A | 21 | 257 |
| DC_28A_n77A | DC_28A_n77A | 28 | 77 |
| DC_28A_n78A | DC_28A_n78A | 28 | 78 |
| DC_28A_n79A | DC_28A_n79A | 28 | 79 |
| DC_28A_n257A | DC_28A_n257A | 28 | 257 |
| DC_42A_n77A | N/A | 42 | 77 |
| DC_42A_n78A | N/A | 42 | 78 |
| DC_42A_n79A | DC_42A_n79A | 42 | 79 |
| DC_42A_n257A | DC_42A_n257A | 42 | 257 |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.3.2			Inter-band EN-DC configurations (three bands)

5.3.2.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3-SUL_n78-n80 | DC_3_SUL_n78-n80 | 3 | SUL_n78-n80 |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.3.3			Inter-band EN-DC configurations (four bands)

5.3.2.3.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.3.4			Inter-band EN-DC configurations (five bands)

5.3.2.3.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

5.3.2.3.5			Inter-band EN-DC configurations (six bands)

5.3.2.3.5-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

< Unchanged section Omitted>

6.2.2	Transmitter power for EN-DC

6.2.2.1	Transmitter power for intra-band EN-DC

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least [one sub frame].

Table 6.2.2.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| DC configuration |  | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- |
| DC_(n)71B |  | [23] | [+2/-3] |

6.2.2.2	Transmitter power for inter-band EN-DC

6.2.2.2.1	Transmitter power for inter-band EN-DC of LTE and NR in FR1

For inter-band EN-DC of LTE and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2.2.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration |  | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- |
| DC_1A_n77A |  | [23] | [+2/-3] |
| DC_1A_n78A |  | [23] | [+2/-3] |
| DC_1A_n79A |  | [23] | [+2/-3] |
| DC_3A_n77A |  | [23] | [+2/-3] |
| DC_3A_n78A |  | [23] | [+2/-3] |
| DC_3A_n79A |  | [23] | [+2/-3] |
| DC_5A_n78A |  | [23] | [+2/-3] |
| DC_7A_n78A |  | [23] | [+2/-3] |
| DC_19A_n77A |  | [23] | [+2/-3] |
| DC_19A_n78A |  | [23] | [+2/-3] |
| DC_19A_n79A |  | [23] | [+2/-3] |
| DC_20A_n78A |  | [23] | [+2/-3] |
| DC_21A_n77A |  | [23] | [+2/-3] |
| DC_21A_n78A |  | [23] | [+2/-3] |
| DC_21A_n79A |  | [23] | [+2/-3] |
| DC_28A_n77A |  | [23] | [+2/-3] |
| DC_28A_n78A |  | [23] | [+2/-3] |
| DC_28A_n79A |  | [23] | [+2/-3] |
| DC_42A_n77A |  | N/A | N/A |
| DC_42A_n78A |  | N/A | N/A |
| DC_42A_n79A |  | [23] | [+2/-3] |

6.2.2.2.2	Transmitter power for inter-band EN-DC of LTE and NR in FR2

For inter-band EN-DC of LTE and NR in FR2, the UE shall meet each transmitter power requirement specified in clause 6.2.2 of TS 36.101 and clause 6.2.1 TS 38.101-2 independently.

< Unchanged section Omitted>

6.2.5	Configured maximum output power for EN-DC

6.2.6		ΔTIB,c for EN-DC

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

6.2.6.1	ΔTIB,c for EN-DC in two bands

Table 6.2.6.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n77A | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1A_n78A | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_3A_n77A | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3A_n78A | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5A_n78A | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7A_n78A | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_19A_n77A | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19A_n78A | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20A_n78A | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21A_n77A | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21A_n78A | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_28A_n77A | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28A_n78A | 28 | 0.5 |
|  | n78 | 0.8 |

6.2.6.2	ΔTIB,c for EN-DC three bands

6.2.6.3	ΔTIB,c for EN-DC four bands

6.2.6.4	ΔTIB,c for EN-DC five bands

< Unchanged section Omitted>

6.5	Output RF spectrum emissions

6.5.1	Output RF spectrum emissions for carrier aggregation

Editor’s note: carrier aggregation of bands FR1 and FR2

6.5.2	Output RF spectrum emissions for inter-band EN-DC

6.5.2.1	Occupied bandwidth

6.5.2.2	Out-of-band emissions

6.5.2.2.1	General

6.5.2.2.2	Spectrum emissions mask (contiguous sub-blocks)

6.5.2.2.3	Additional spectrum emissions mask (contiguous sub-blocks)

6.5.2.2.4	Adjacent channel leakage ratio (contiguous sub-blocks)

6.5.2.3	Spurious emissions

6.5.2.3.1	Minimum requirements

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [36.101], sub-clause 6.5.3.1 of [38.101-1] and [38.101-2] apply for each component carrier.

6.5.2.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.6.3.2A-0 apply on each component carrier with all component carriers are active.

Table 6.5.2.3.2: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  |  | Frequency range |  | 1915 |  | - |  | 1920 |  | +1.6 |  | 5 |  | 5, 7, 9 |  |
| DC_1A_n79A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_3A_n7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n1, n5, n7, n8, n20, n28, n50, n51, n74, n75, n76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_3A_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_3A_n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_3A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_5A_n257A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_7A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| DC_7A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |
| DC_19A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_19A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_19A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_19A_n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20A_n78A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21A_n77A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_21A_n78A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_21A_n79A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_21A_n257A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28A_n77A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_28A_n78A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_28A_n79A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |
| DC_28A_n257A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42A_n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | [-25] | 1 |  |
| DC_42A_n257A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD - 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. NOTE 9: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

6.5.3	Output RF spectrum emissions for intra-band EN-DC

6.5.3.1	Occupied bandwidth

Editor’s note: if needed for contiguously aggregated sub-blocks in any region

6.5.3.2	Out-of-band emissions

6.5.3.2.1	General

Unless otherwise stated, the OOBE limits specified for the DC combination in this sub-clause apply regardless of any OOBE requirements specified for each sub-block in the respective [36.101] and [38.101-1].

The requirements apply to each antenna connector.

6.5.3.2.2	Spectrum emissions mask (contiguous sub-blocks)

6.5.3.2.3	Additional spectrum emissions mask (contiguous sub-blocks)

6.5.3.2.3.1	DC_(n)71B (contiguous sub-blocks)

When NS_35 is indicated in an E-UTRA sub-block of bandwidth class A and NS_j is indicated in an adjacent NR sub-block of bandwidth class A the requirements in Table 6.5.3.2.3.1-1 apply in the frequency ranges immediately adjacent and outside the aggregation of the said sub-blocks.

Table 6.5.3.2.3.1-1: Additional requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.085 MHz | 0.015 MHz f_offset < 0.1 MHz | -13 | 30 kHz |
| 0.05 MHz f < ENBW – 0.05 MHz | 0.1 MHz f_offset < ENBW MHz | -13 | 100 kHz |
| (ENBW – 0.5) MHz f < (ENBW + 4.5) MHz | ENBW MHz f_offset < (ENBW + 5) MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The measurement filter -3dB point is that closest to the aggregated block. |  |  |  |

6.5.3.2.4	Adjacent channel leakage ratio (contiguous sub-blocks)

For EN-DC operation with an E-UTRA sub-block immediately adjacent to an NR sub-block, the ACLR is defined as the ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW [at nominal channel spacing]. The UE shall meet the ACLR minimum requirement EN-DCACLR specified in Table 6.5.3.2.4-1 with ENBW the sum of the sub-block bandwidths.

Table 6.5.3.2.4-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth |  | [0.95] ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

6.5.3.3	Spurious emissions

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [36.101] and sub-clause 6.x.y.z of [38.101-1] apply beyond any frequencies for which the out-of-band emissions requirements in sub-clause 6.5.2 apply.

< Unchanged section Omitted>

7.2.2		ΔRIB,c for EN-DC

For the UE which supports inter-band EN-DC configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in TS 36.101, Table 7.3-1 in TS 38.101-1 and Table 7.3.1-1 in TS 38.101-2 shall be increased by the amount given in ΔRIB,c in Tables below. Unless otherwise stated, ΔRIB,c is set to zero.

7.2.2.1	ΔRIB,c for EN-DC in two bands

Table 7.2.2.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A_n77A | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1A_n78A | n78 | 0.5 |
| DC_3A_n77A | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3A_n78A | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5A_n78A | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7A_n77A | n78 | 0.5 |
| DC_19A_n77A | n77 | 0.5 |
| DC_19A_n78A | n78 | 0.5 |
| DC_20A_n78A | n78 | 0.5 |
| DC_21A_n77A | n77 | 0.5 |
| DC_21A_n78A | n78 | 0.5 |
| DC_28A_n77A | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28A_n78A | 28 | 0.2 |
|  | n78 | 0.5 |

7.2.2.2	ΔRIB,c for EN-DC three bands

Table 7.2.2.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n80 | 0.2 |

< Unchanged section Omitted>

7.3.2	Reference sensitivity for EN-DC

For EN-DC, E-UTRA and NR single carrier REFSENS requirements defined in [2], [3] and [4] apply to all downlink bands of EN-DC configurations listed in Tables[TBD] unless sensitivity degradation is allowed as defined in clause 7.3.2.1 and 7.3.2.2.

7.3.2.1	Reference sensitivity exceptions for EN-DC in FR1 EN-DC

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

Note: FFS how to clarify the issues of 1Tx may also exist for 2Tx mode, for example harmonic, etc.

7.3.2.1.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3.2.1.1-1 with uplink configuration specified in Table 7.3.2.1.1-2.

Table 7.3.2.1.1-1 MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n771,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n773 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| n80, 3 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  | 17.9 |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |
| 19 | n774,5 |  | [10.4] | [8.9] | [7.8] |  | [4.7] | [3.7] | [3] | [1.7] | [0.7] |
| 28 | n774,5 n784,5 |  | [10.4] | [8.9] | [7.8] |  | [4.7] | [3.7] | [3] | [1.7] | [0.7] |
| 20 | n786,7 |  | 10.8 | 9.1 | 8 |  | 6 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.1.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n77 | 12 | 26 | 39 | 531, 1002 |  |  |  |  |  |  |
| n80, 3 | n78 | 12 | 26 | 39 | 531, 1002 |  |  |  |  |  |  |
| 19 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| 20 | n78 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| 28 | n77 n78 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. |  |  |  |  |  |  |  |  |  |  |  |

7.3.2.1.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3.2.1.2-1 with uplink configuration specified in Table 7.3.2.1.2-2.

Table 7.3.2.1.2-1: MSD due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 282 | 28 | 25 | 23.2 | 22 | N/A | N/A | N/A | N/A | N/A | N/A |
| n79 | 192 | 29.5 | 26.5 | 24.7 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n79 | 213 | 39.3 | 36.3 | 34.5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.1.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 | N/A | N/A | N/A | N/A | N/A | N/A |
| n79 | 19 | 15 | 25 | 50 | 75 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n79 | 21 | 15 | 25 | 50 | 75 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

7.3.2.1.3	Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by close proximity of an UL of another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3.2.1.3-1 with uplink configuration specified in Table 7.3.2.1.3-2.

Table 7.3.2.1.3-1: Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.1.3-2: Uplink configuration for reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |

7.3.2.1.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same DC configuration due to cross band isolation issues. Reference sensitivity exceptions are specified in Table 7.3.2.1.4-1 with uplink configuration specified in Table 7.3.2.1.4-2.

Table 7.3.2.1.4-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.1.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |

7.3.2.1.5	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC in NR FR1 configurations indicated by column single uplink allowed in Table 7.3.2.1.5-1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference to its own downlink band if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are below 1 GHz or between 1695 MHz – 2690 MHz. In case for the EN-DC in NR FR1 configurations listed in Table 7.3.2.1.5-1 the intermodulation products caused by the dual uplink operation do not interfere with the own downlink transmission as defined in Annex-A the UE is mandated to operate in dual uplink mode.

For EN-DC in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3.2.1.5-1 the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.2.1.5-1. For these test points the reference sensitivity requirement specified clause 7.3.2.1 of TS 38.101-1 is relaxed by the amount of the parameter MSD given in Table 7.3.2.1.5-1.

Table 7.3.2.1.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | [29.8] | FDD | IMD23 |  |
|  |  |  |  |  |  | [32.54] |  |  |  |
|  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |  |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | [8.0] | FDD | IMD43 |  |
|  |  |  |  |  |  | [10.74] |  |  |  |
|  | n77 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | [8.0] | FDD | IMD43 |  |
|  |  |  |  |  |  | [10.74] |  |  |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |  |
|  | n7 | 2535 | 10 | 52 | 2655 | [10.25] | FDD | IMD4 |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD23 |  |
|  |  |  |  |  |  | [28.74] |  |  |  |
|  | n77, n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | [8.0] | FDD | IMD43 |  |
|  |  |  |  |  |  | [10.74] |  |  |  |
|  | n77, n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_3A-SUL_n78A-n80A | 3 | 1740 | 5 | 25 | 1835 | [26] | FDD | IMD24 |  |
|  |  |  |  |  |  | [28.75] |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_3A-SUL_n78A-n80A | 3 | 1765 | 5 | 25 | 1860 | [8.0] | FDD | IMD44 |  |
|  |  |  |  |  |  | [10.75] |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
| DC_20A_n78A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |  |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | [18.4] | FDD | IMD3 |  |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | TDD | N/A |  |
| CA_28A_n77A CA_28A_n78A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |  |
|  | n77, n78 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

7.3.2.2	Reference sensitivity exceptions for EN-DC in NR FR2

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

Table 7.3.2.2-1 Reference sensitivity exceptions for band combinations of E-UTRA + NR FR2

| Band combination | Band | 50 MHz (dBm) | 100 MHz (dBm) | 200 MHz (dBm) | 400 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| CA_XA-YA | Y |  |  |  |  |

Table 7.3.2.2-2: UL configuration for reference sensitivity

| Band combination | Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_XA-YA | X |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  |  |  |  |  |  |  |  |  |  |  |

7.3.2.2.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR2

Sensitivity degradation is allowed for a band in frequency range 2 if it is impacted by UL harmonic interference from the band in frequency range 1 of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3.2.2.1-1 with uplink configuration specified in Table 7.3.2.2.1-2.

Table 7.3.2.2.1-1: Reference sensitivity exceptions due to UL harmonic for EN-DC in NR FR2

| UL Band | DL Band | 50 MHz (dBm) | 100 MHz (dBm) | 200 MHz (dBm) | 400 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |

Table 7.3.2.1.2-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |

<End of Change - Unchanged section Omitted>