3GPP TSG-RAN WG4 AH-1801 Meeting                                R4-1800260

San Diego, California, 22 to 26 January 2018

Agenda item:	4.2.1.3

Source: 	ZTE

Title: 	On minimum guardband of SS/PBCH block with SCS 240 kHz for FR2

Document for:	Approval

Introduction

RAN4#85 meeting in November 2017 made the following agreements on NR spectrum utilization [1]:

A SS/PBCH block could be transmitted with SCS 240 kHz in FR2 [2] [3], and it’s possible that the SS/PBCH block is placed at the edge of a channel BW. In this contribution, we give our views on the minimum guardband of SS/PBCH block with SCS 240 kHz for FR2.

Minimum guardband of SS/PBCH block with SCS 240 kHz

In RAN4#85 meeting the maximum transmission bandwidth configuration NRB for SCSs 60 kHz and 120 kHz combination for FR2 were decided as shown in Table 1 and have been specified in [4].

Table 1: Maximum transmission bandwidth configuration NRB for FR2

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

The minimum guardband for SCSs 60 kHz and 120 kHz were decided as shown in Table 2 and have been specified in [4]. The minimum guardbands have been calculated using the following equation: (CHBW * 1000 (kHz) - NRB * SCS x 12) / 2 - SCS/2, where NRB is from Table 1.

Table 2: Minimum Guardband [kHz] for FR2

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

In the frequency domain, a SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block [3]. To derive the minimum guardband for SS/PBCH block with SCS 240 kHz, in this contribution we assume that this could be done by regarding SS/PBCH block as data block with single numerology SCS 240 kHz in the channel BW. As RAN4 did before for data channel with SCSs 60 kHz and 120 kHz for NR spectrum utilization, we did some PSD simulations for SCS 240 kHz to find the maximum NRB value for each channel BW to meet the unwanted emission requirements [4], and the results are summarized in Table 3. QPSK modulation is used in the simulation since a UE shall assume SS/PBCH block are QPSK modulated [3]. 50 MHz channel BW is too narrow for the 20-RB long SS/PBCH block.

Table 3: NRB for SCS 240 kHz

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 240 | N.A | 32 | 64 | 128 |

The minimum guardbands for SCS 240 kHz could be calculated using the following equation which is used to calculate the minimum guardband for other SCSs such as 60 kHz and 120 kHz in [4]. The results are summarized in Table 4 below.

Minimum guardband = (CHBW * 1000 (kHz) - NRB * SCS * 12) / 2 - SCS/2,

where NRB is from Table 3.

Table 4: Minimum Guardband [kHz] for SCS 240 kHz

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

Proposal 1: When a SS/PBCH block with SCS 240 kHz is placed at the edge of a channel bandwidth and has the same TX power/kHz as the data block, the minimum guardband [kHz] could be considered as below:

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

The TX power that the network is used for SSB transmission is included in SIB1 as below [2]. To improve UE receiving performance of SS/PBCH block, it’s possible that the network uses extra high TX power/kHz for SSB transmission than data blocks. In such a case, the necessary guardband for SS/PBCH block needs to be increased. How this will be handled in RAN4 should be further discussed and decided.

-- TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power.

-- (see 38.213, section 7.4)

ss-PBCH-BlockPower					INTEGER (-60..50),

Proposal 2: RAN4 should discuss and decide how to handle the guardband for SS/PBCH block when the block is placed at the edge of a channel bandwidth and has extra high power boosting compare to the data block.

Conclusions

Based on the discussions in this contribution, we have the following proposals:

Proposal 1: When a SS/PBCH block with SCS 240 kHz is placed at the edge of a channel bandwidth and has the same TX power/kHz as the data block, the minimum guardband [kHz] could be considered as below:

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

Proposal 2: RAN4 should discuss and decide how to handle the guardband for SS/PBCH block when the block is placed at the edge of a channel bandwidth and has extra high power boosting compare to the data block.

# References

- RAN4#85 Main Session Chairman note_EOM, December 2017.

- RP-172570 draft TS38.331 RRC protocol, December 2017.

- RP-172284 draft TS 38.211 physical channels and modulation, December 2017.

- RP-172268 draft TS 38.104 Base Station (BS) radio transmission and reception, December 2017.