3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800263

San Diego, California, USA, 22 – 26 January 2018

Agenda Item:	4.4.3.1.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.817-02: NR BS receiver sensitivity requirements for FR2 (10.3)

Document for:	Approval

# 1.	Introduction

The way forward on receiver sensitivity requirements for FR2 was agreed in RAN4#85 [1]. The agreements have not been recorded in the NR BS TR 38.817-02 [2], although some of them have been implemented in the NR BS TS 38.104 [3].

This contribution provides a text proposal to record the agreements in TR 38.817-02 [2] for future reference.

# 2.	Text Proposal

<Start of change>

## 10.3	OTA Reference sensitivity level

The following have been agreed for OTA reference sensitivity level for FR2:

-	Reference sensitivity power level, EISREFSENS [dBm] = [X] – ΔOTAREFSENS

-	Declared by BS manufacturer

-	[X] = -174 + 10*log(BW) + NF + SNR + IM

-	BW is the transmission bandwidth configuration of the wanted signal in Hz

-	NF is the noise figure of the BS (include allowed receiver desensitization for MR and LA BS)

-	SNR is the required SNR to achieve 95% of maximum throughput (obtain from simulation)

-	IM is implementation margin

-	ΔOTAREFSENS

-	[1]dB step within the range [10] to [30]dB for WA

-	[1]dB step within the range [0] to [20]dB for MR

-	[1]dB step within the range [0] to [20]dB for LA

-	Reflect BS implementation parameters like antenna array gain

-	For MR and LA, more analysis need to be done

<End of change>

# References

[1]	R4-1714434, “WF on receiver sensitivity requirements for FR2”, Nokia, Nokia Shanghai Bell.

[2]	R4-1714547, “TR 38.817-02 v0.5.0: General aspects for BS RF for NR”, Ericsson.

[3]	3GPP TR 38.104 v15.0.0, “NR; Base Station (BS) radio transmission and reception”.