3GPP TSG-RAN WG4#AH-1801 	R4-1800285
San Diego, CA, USA Jan. 22 - 26, 2018

Agenda item:	4.3.2.1.5

Source:	Apple Inc., Intel Corporation

Title:	Assumptions for Simulating Spherical Coverage Performance

Document for:	Discussion

# 1	Introduction

During the RAN #73 meeting the study item on New Radio access technology [1] was finalized with its outcome captured in TR 38.803 [2]. During RAN #75 the New Radio Work Item was initiated [3]. With an agreement to define and test RF requirements for NR mm-wave over the air (OTA), the discussion on spherical coverage has evolved over a number of agreements.

Most recently, in RAN4#85, a WF on spherical coverage in FR2 was agreed in [4] and suggests a work plan for forthcoming meetings and provides some simulation assumptions to be discussed as follows:

Work Plan:

- RAN4 NR AH #4 (January ’18)

- Initial discussion of simulation results (Both EIRP CDF and Network) based on the harmonized assumptions in this way forward.

- Propose harmonized NW model assumptions and update based on preliminary analysis.

Assumptions for NW Analysis:

- Operators to provide guidance on deployment scenarios and use cases

  - In absence of guidance TR 38.803 scenarios to be considered

  - UE peak EIRP shall be modified in accordance with outcome of RAN4#85

- The following modeling confirmation or enhancements to 38.803 assumptions are needed to assess impact of spherical coverage on network. Modeling is FFS where indicated

  - ISD (200m for dense urban and Macro, 20m for Indoor Office)

  - UE indoor/outdoor ratio (All UEs indoor for Indoor Office, All UEs outdoor for dense urban and Macro)

  - Coverage definition and coverage requirement (currently outage is defined as min SINR less than -10dB)

  - UE elevation (It is currently fixed at 90 degree)

  - Reference antenna pattern CDFs that parameterizes (percentile, EIRP) the spherical coverage requirements (FFS)

  - Partial resource allocation (Not allocating all PRBs to UE) (FFS)

  - Body Blockage, Handgrip, cover materials (FFS)

- Performance metrics

  - Guidance from operators is requested for given deployment scenarios and use cases

  - UL throughput and outage

In RAN4#85, we shared our views on the EIRP CDF for a modern form factor [5] and compared it to a form factor in which the radiating elements see largely plastic. Our results showed that when the phone model employs features which are common to the modern cellular phone, including a full display OLED/LCD and a metal bezel, spherical CDF performance is significantly compromised relative to that of a generally plastic and benign form factor.  We have presented our view that setting spherical radiation requirements based on a benign form-factor design, will only impede adoption of the technology. Finally, we suggested that uplink and downlink network scenarios be studied through simulation to understand realistic carrier deployment scenarios under the constraint of realistic UE form factor. RAN4#85 adopted a way forward that requires a harmonization on network simulation assumptions for a fair comparison.

In this paper, we describe our view on assumptions for the network simulations and provide some preliminary results. Our goal is to harmonize the assumptions within RAN4 for simulating spherical coverage performance. We reiterate that the assumptions in TR 38.803 [2] serve as a baseline and only necessary deviations be made to understand the impact of spherical CDF performance.

# 2	Discussion

In RAN4#85, we presented network simulation results that demonstrate the impact of peak EIRP on UL throughput and outage probability [6] for the indoor and dense urban deployment scenarios. The simulation parameters and antenna patterns in [6] were prescribed for the coexistence study in Chapter 5 of TR 38.803 [2]. The coexistence study in TR 38.803 has not explicitly considered spherical coverage CDF performance. Such a characterization requires modifications to the TR 38.803 framework. In this section, we review the relevant parameters and scenarios given in TR 38.803 and provide our view on necessary modifications.

## 2.1	Antenna Beam Pattern

In our contribution on spherical coverage CDF performance [5], we presented CDFs for spherical coverage based on different form factor considerations. TR 38.803 also provides a UE antenna pattern B(𝜑, 𝜃) which results in a different spherical  coverage profile. Figure 1 shows the spherical coverage CDF when two panels are implemented in a realistic UE (shown as Model-2 (A+B) in [5]) and also the spherical coverage of the TR 38.803 antenna pattern when the UE has two panels (one panel facing the front and one facing the back). We understand that companies may have different implementations and consequently very different spherical coverage profiles. This presents a serious challenge in harmonizing simulation assumptions and results. The task may be simplified by considering the following observation.

Observation 1: Consider a UE, UE1, with spherical coverage CDF C1 and a different UE implementation, UE2, with spherical coverage CDF C2. It is possible for UE1 and UE2 to have very different beam patterns but similar spherical coverage profiles, i.e., C1 and C2 are equal or approximately equal at each percentile. Furthermore, if the deployment and usage scenario uniformly excites the UE beam pattern over the sphere then both UEs are expected to have similar performance.

### 2.1.1	Spatially Masked Antenna Pattern

Given the baseline beam pattern B(𝜑, 𝜃) defined in TR 38.803, we can define a spatial mask M(𝜑, 𝜃) so that the effective beam pattern for the purpose of simulation is

BM(𝜑, 𝜃) = B(𝜑, 𝜃). M(𝜑, 𝜃)

The spatial mask may be derived so as to approximate a certain spherical coverage profile.  Furthermore, it may be necessary to apply the mask in azimuth, or in elevation or across both to achieve the desired result. Note that based on the simulation conditions in Observation 1, exact choice of mask is irrelevant as long as it results in the appropriate CDF. Figure 1 also demonstrates the resulting CDF by application of a spatial mask to the baseline antenna pattern in TR 38.803. The resulting model is a good approximation of the CDF presented by us in [5].

Proposal 1: A spatial mask may be applied to the baseline UE antenna pattern in TR 38.803 to approximate a desired spherical coverage profile. Companies must share the CDF associated with the simulation results. Specification of spatial mask is not necessary.

Figure 1: EIRP CDF

### 2.1.2	UE Elevation

In TR 38.803, the UE elevation is fixed at 90 degrees. In order to simulate the spherical coverage performance, in line with Observation 1, we must relax this assumption and allow the UE a random orientation. We therefore expand the TR 38.803 assumption to allow the UE elevation to vary uniformly between 0 to 180 degrees. Furthermore, we stress that a UE taking on a random orientation does not necessarily transmit at its peak EIRP value.

Proposal 2: Use random UE elevation uniformly distributed within 0 to 180 degrees.

## 2.2	Deployment Scenarios

TR 38.803 has considered three deployment scenarios: 1- Indoor Hotspot, 2-Dense Urban, and 3- Macro cell [2]. For the spherical coverage study the indoor scenario assumptions described in TR 38.803 are adequate; however, for the dense urban and macro cell deployment scenarios we revisit the critical assumptions below.

### 2.2.1	ISD

We simulated the macro cell deployment from TR 38.803 for the UL-UL scenario at 30 GHz across different bandwidth (BW) allocations.  All UEs in the network are scheduled on the same center frequency with the same BW. This represents the worst-case interference. We set a noise figure of 9 dB and introduce a programmable loss in EIRP to simulate system performance within the range of peak EIRP values (21.2-to-25.2 dBm) agreed in the WF on peak EIRP in RAN4#85 [7]. We used the same antenna pattern in TR 38.803, two panels and introduced the spatial mask described above to mimic the spherical coverage CDF in Figure 1. UE elevation is uniformly distributed within 0 to 180 degrees. Finally, all users are assumed to be outdoors. Clearly adding more indoor users will only increase the outage probability.

Figure 2 shows the outage probability for different ISD values across varying bandwidth allocations. The following observations can be made.

Observation 2: The ISD specified in TR 38.803 is 200 m for the urban macro scenario. The outage probability is less than 5% when the ISD is less than 200 m for the approved range of peak EIRP values and across bandwidths considering a realistic spherical coverage profile.

Observation 3: Outage performance significantly improves at lower bandwidths due to reduction in noise power.

Observation 4: The lowest value within the range of EIRP specified in the WF, i.e., 21.2 dBm, delivers acceptable outage performance for an ISD of 200 m and for a realistic UE spherical coverage profile.

We make the following proposals.

Proposal 3: Limit the ISD to 200 m if the urban macro scenario is to be simulated.

Proposal 4: Coverage limits may be determined based on 20 MHz bandwidth allocation.

Figure 2: Outage probability vs. peak EIRP for the urban macro (UMa) scenario.

We note that the ISD in the dense urban deployment in TR 38.803 is also limited to 200 m. While we have shown that the urban macro scenario is feasible with a realistic spherical coverage profile, the dense urban deployment offers superior coverage. We therefore suggest the following.

Proposal 5: Pick the dense urban scenario to specify the UE spherical coverage requirement.

### 2.2.2	Indoor/Outdoor User Ratio

In order to understand the impact of indoor/outdoor user ratio in the dense urban and urban macro deployments, we simulated both the dense urban and urban macro scenarios with all UEs indoor. The ISD was fixed at 200 m and the other simulation parameters are as described in Section 2.2.1. We simulated with the TR 38.803 antenna pattern (i.e., without mask) to understand the performance limits in these deployment scenarios. Figure 3 shows the outage probability for indoor users in the dense urban and urban macro cell deployments.

Observation 5:  The outage probability for indoor users is greater than 14% in the dense urban scenario and more than 30% in the urban macro cell scenario when BW is 20 MHz and ISD is 200 m. These probabilities increase to 33% and 55% at BW 200 MHz.  We can conclude that supporting indoor users with outdoor base-stations is not a realistic deployment scenario.

Proposal 6: Simulate only outdoor users when base stations are outdoor.

Figure 3: Outage probability vs. peak EIRP for indoor users in the dense urban and urban macro deployments.

## 2.3	Body blockage and Handgrip

The latest WF on spherical coverage [4] suggests further discussions on modeling modifications relating to body blockage, handgrip and cover materials in spherical coverage performance simulations. Although it would be informative to study and model these aspects, arriving at a consensus that can be corroborated via measurements and user behavior and the following simulator calibration is a time-consuming process. We therefore recommend not to include these aspects for the purpose of the baseline spherical coverage study. If RAN4 decides to include these features in the simulations, further discussions on modeling are required.

Proposal 7: Do not consider body blockage and handgrip in spherical coverage simulations.

# 3	Conclusions

In this contribution, we described our view on assumptions for the network simulations and provided some preliminary results. Our goal is to harmonize the assumptions within RAN4 for simulating spherical coverage performance.

The following observations and proposals have been made:

Observation 1: Consider a UE, UE1, with spherical coverage CDF C1 and a different UE implementation, UE2, with spherical coverage CDF C2. It is possible for UE1 and UE2 to have very different beam patterns but similar spherical coverage profiles, i.e., C1 and C2 are equal or approximately equal at each percentile. Furthermore, if the deployment and usage scenario uniformly excites the UE beam pattern over the sphere then both UEs are expected to have similar performance.

Observation 2: The ISD specified in TR 38.803 is 200 m for the urban macro scenario. The outage probability is less than 5% when the ISD is less than 200 m for the approved range of peak EIRP values and across bandwidths considering a realistic spherical coverage profile.

Observation 3: Outage performance significantly improves at lower bandwidths due to reduction in noise power.

Observation 4: The lowest value within the range of EIRP specified in the WF, i.e., 21.2 dBm, delivers acceptable outage performance for an ISD of 200 m and for a realistic UE spherical coverage profile.

Observation 5:  The outage probability for indoor users is greater than 14% in the dense urban scenario and more than 30% in the urban macro cell scenario when BW is 20 MHz and ISD is 200 m. These probabilities increase to 33% and 55% at BW 200 MHz.  We can conclude that supporting indoor users with outdoor base-stations is not a realistic deployment scenario.

Proposal 1: A spatial mask may be applied to the baseline UE antenna pattern in TR 38.803 to approximate a desired spherical coverage profile. Companies must share the CDF associated with the simulation results. Specification of spatial mask is not necessary.

Proposal 2: Use random UE elevation uniformly distributed within 0 to 180 degrees.

Proposal 3: Limit the ISD to 200 m if the urban macro scenario is to be simulated.

Proposal 4: Coverage limits may be determined based on 20 MHz bandwidth allocation.

Proposal 5: Pick the dense urban scenario to specify the UE spherical coverage requirement.

Proposal 6: Simulate only outdoor users when base stations are outdoor.

Proposal 7: Do not consider body blockage and handgrip in spherical coverage simulations.

# 4	References

- RP-161596, “Study on New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #73, September 2016

- TR 38.803, “Study on New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017

- RP-170855, “Work Item on New Radio (NR) Access Technology,” NTT DOCOMO, INC., 3GPP RAN #75, March 2017

- R4-1714455, “WF on spherical coverage in FR2”, Samsung, Apple Inc., LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek, 3GPP RAN4 #85, November 2017

- R4-1713850,	“Consideration of EIRP spherical coverage requirement”, Apple Inc., Intel Corporation, 3GPP RAN4 #85, November 2017

- R4-1714003, “Network performance impact of practical peak EIRP”, Apple Inc., Intel Corporation, 3GPP RAN4 #85, November 2017

- R4-1714445,	“WF on pi/2 BPSK spectrum shaping and power class for FR2”, Intel Corporation, Apple Inc., Qualcomm, IITH, LGE, Nokia, Huawei, Motorola Mobility, 3GPP RAN4 #85, November 2017