3GPP TSG-RAN WG4 Meeting #84Bis						R4-1800290

San Diego, CA, USA, 22 - 26 January, 2018

Source: 	Qualcomm Incorporated

Title: 	mmW MPR

Agenda item:	9.3.2.3.2

Document for:	Approval

# Introduction

In this contribution, we are re-submitting MPR requirements for FR2 using simulated and additional measured data. We propose MPR for both narrow CC’s and wide BW CC’s.

# Discussion

In previous contribution [1], we outlined the MPR requirements for FR2 based on simulations and measurement with appropriate assumptions and at that time, we assumed MPR was BW agnostic. We performed a simulation and measurement campaign with 8 PA mmW device intended to drive 4 antenna elements with two polarizations. The summary of previous results are shown in the appendix, Table A1, A2, and in the 3rd column of Table 1, assuming SCFDMA – QPSK is the reference.

Table 1: Comparison of FR2 MPR (CBW <=200MHz)

|  | QCOM (Measurement) | QCOM sim [1] | Company X [2] | Company Y [3] |
| --- | --- | --- | --- | --- |
| SCFDMA-shaped BPSK | - | -3.0 | - | -3.5 |
| SCFDMA-BPSK | - | -0.5 | 0 | -1.0 |
| SCFDMA-QPSK | 0 | 0 | 0 | 0 |
| SCFDMA-16 QAM | - | 0.5 | 0.5 | 0.5 |
| CP OFDM-QPSK | 2.0 | 1.0 | 1.5 | 1.5 |
| CP OFDM-16 QAM | - | 2.5 | 2.5 | 2.5 |
| SCFDMA-64 QAM | 3.0 | 2.5 | 3.5 | 3.5 |
| CP OFDM-64 QAM | 5.0 | 5.0 | 6.0 | 6.0 |

The MPR limitation criteria and simulation assumptions are also listed in [1] references:

To verify simulations, we performed some measurements for fully allocated 100MHz waveforms and listed the results in column 2 of Table 1. We also listed MPR data from other companies as a side by side comparison. There were some differences between measurement and simulation that we can attribute to practical implementation.

Based on new measured data and previous simulated data, we then formulate an MPR for 50M, 100M, 200M as shown in the 2nd column of Table 2.

Proposal 1: For FR2, for CC BW 50M and 100M, regardless of sub-carrier spacing, we propose MPR as listed in 2nd column of Table 2.

When the CC BW gets large such as in 200M and 400M, we find that FR2 architecture introduces droop effects that involves increasing subsequent stage gains to flatten the response. The amount of droop can be up to 0.5dB for a 200MHz allocation and up to 2dB for a 400MHz allocation. These droop effects come from various sources within the FR2 architecture such as the BB output, IF output and the RF output as shown in Figure 1.

Figure 1:

To account for the practical frequency response, some outer RB allocation will have lower output power than other RB allocations within the middle of CC. To prevent overdriving the latter stages at the outer RB allocation and thereby increasing distortion, we need to limit the power by the amount of the total droop. So, for RB configurations where the Outer RB’s are utilized, we propose to limit power for those cases.

To prevent applying added MPR to all RB allocations within the 200MHz and 400MHz CC’s, we make use of the sub6, FR1 definition of “Inner” and “Outer” MPR. In FR2, we use the same Inner/Outer region to define the boundary to account for the drooping effects for wide CC BW cases.

Examples of Inner and Outer RB allocation are illustrated in Figure 2.

Figure 2:

Observation 1: The occupied transmission BW of the Inner RB allocation of large BW CC will not exceed BW/2, and so the MPR numbers of Inner 200MHz are equivalent to MPR numbers for 50M,100M BW’s.

Observation 2: The Inner and Outer principle can apply to aggregated BW’s >=400MHz such as CA cases, where the drooping effects become more severe. For CA cases, more droop >2dB can be observed.

Proposal 2: For FR2, for CC BW =200MHz and 400MHz, we define Inner and Outer MPR to account for droop effects of up to 1dB and 2dB respectively due to practical frequency response in typical RF architecture and propose MPR listed as shown in the labelled Outer columns of Table 2.

Proposal 3: The PA Calibration reference waveform for MPR0 is 128RB 100MHz 120KHz QPSK waveform.

Table 2: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | MPR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50M/100MHz | 200MHz |  | 400MHz |  |  |
|  |  | Inner | Outer | Inner* | Outer* | Outer |
| SCFDMA-Shaped pi/2 BPSK | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| SCFDMA-pi/2 BPSK | [-0.5] | [-0.5] | [-0.5] | [-0.5] | [-0.5] | [1.5] |
| SCFDMA-QPSK | 0 | 0 | ≤ 1.0 | 0 | ≤ 1.0 | ≤ 2.0 |
| SCFDMA-16 QAM | ≤ 0.5 | ≤ 0.5 | ≤ 1.5 | ≤ 0.5 | ≤ 1.5 | ≤ 2.5 |
| CP OFDM-QPSK | ≤ 2.0 | ≤ 2.0 | ≤ 3.0 | ≤ 2.0 | ≤ 3.0 | ≤ 4.0 |
| CP OFDM-16 QAM | ≤ 2.5 | ≤ 2.5 | ≤ 3.5 | ≤ 2.5 | ≤ 3.5 | ≤ 4.5 |
| SCFDMA-64 QAM | ≤ 3.0 | ≤ 3.0 | ≤ 4.0 | ≤ 3.0 | ≤ 4.0 | ≤ 5.0 |
| CP OFDM-64 QAM | ≤ 5.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.0 | ≤ 6.0 | ≤ 7.0 |

Inner Definition: LCRB <=LCRBmax/2 with RBstart >=LCRB/2 from edge [4].

Outer Definition: all LCRB with RB start < LCRB/2 from edge [4]. Definition applies to 200MHz and 400MHz CC.

The Inner* and intermediate Outer* region of the 400MHz CC is identical to the 200MHz inner and outer region.

# Conclusion

We present MPR based on simulations and additional measurements for narrow and wide BW CC’s. We re-use the Inner/Outer MPR boundary condition as used in FR1 and apply the definition to account for practical droop effects of wide BW CC’s.

Proposal 1: For FR2, for CC BW 50M and 100M, regardless of sub-carrier spacing, we propose MPR as listed in 2nd column of Table 2.

Proposal 2: For FR2, for CC BW =200MHz and 400MHz, we define Inner and Outer MPR to account for droop effects of up to 1dB and 2dB respectively due to practical frequency response in typical RF architecture and propose MPR listed as shown in the labelled Outer columns of Table 2.

Proposal 3: The PA Calibration reference waveform for MPR0 is 128RB 100MHz 120KHz QPSK waveform.

# References

- R4-1711517 ‘mmW MPR’, Qualcomm, Incorporated, 3GPP TSG-RAN WG4 Meeting #84Bis, Dubrovnik, Croatia, 9-13 September 2017

- R4-1712330, ‘MPR Simulation Results for FR2’, Intel Corporation, 3GPP TSG-RAN WG4 Meeting #85, Reno, USA, 27 November - 1 December 2017

- R4-1712265, ‘Discussion on NR FR2 MPR’, Samsung, 3GPP TSG-RAN WG4 Meeting #85, Reno, USA, 27 November - 1 December 2017

- R4-1709944, ‘WF on MPR table and RB allocations sub-6GHz’, 3GPP TSG-RAN WG4 NRAH#3, Nagoya, Japan, 18 - 21 September 2017

# Appendix: Simulated Results

Table A1 DFT-S-OFDM MPR summary

|  | DFT-s-OFDM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mod Order | 64QAM | 16QAM | QPSK | pi/2 BPSK | sh pi/2 BPSK |
| MPR, dB | 6 | 4 | 3.5 | 3 | 0.5 |
| Limitation | EVM | EVM | IBE | SEM | SEM |

Table A2 CP-OFDM MPR summary

|  | CP-OFDM |  |  |
| --- | --- | --- | --- |
| Mod Order | 64QAM | 16QAM | QPSK |
| MPR, dB | 8.5 | 6 | 4.5 |
| Limitation | EVM | EVM | IBE |

Note: EUTRA in legend here means mmW ACLR