3GPP TSG-RAN4 Meeting #AH1801 	R4-1800340

San Diego, USA, 22-26, January, 2018

Agenda Item:	9.7.11.1

Source:	Intel Corporation

Title:	SS block RSRP link level simulation result based on beamforming

Document for:	Discussion

# Introduction

In this contribution, we updated our simulation results for NR-SSS based RSRP for beamforming.

# Simulation results

The detailed simulation parameters can be found in [1]. The simulation results for RSRP based on NR-SSS are given with the CDF of measurement error below for the following simulation cases firstly.

- Subcarrier spacing: 120K/240K

- Carrier frequency: 4G Hz/30G Hz

- Number of SS blocks per SS burst set : K=1

- UE receive antennas: 4

- SNR: -6dB

- Channel model: CDL A

Simulation results are in following figures.

Table 1 and Table 2 are simulation results for 120K Hz and 240K Hz respectively.

Table 3:  simulation results for 120K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| CDL-A | 1 sample | -1 | 2.7 | 2.7 |
|  | 3 sample | 0.5 | 2.1 | 2.1 |

Table 4:  simulation results for 240K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| CDL-A | 1 sample | -1.8 | 1.9 | 1.9 |
|  | 3 sample | -1 | 1.5 | 1.5 |

Observation 1: When beamforming is applied, N=3 samples can achieve the RSRP accuracy requirement.

# Conclusion

In this contribution some NR link level simulation results for NR SS RSRP for beamforming was updated.

Observation 1: When beamforming is applied, N=3 samples can achieve the RSRP accuracy requirement.

# References

[1] R4-1706904, “N Link-level simulation assumptions for SS block measurements”, Ericsson