3GPP TSG-RAN4 Meeting AH-1801	R4-1800383

# San Diego, CA, United States, 22 – 26 January 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 introducing handover and interruption delay for NR mobility. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce UE requirements for handover and interruption delay for NR mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE requirements for handover and interruption delay for NR mobility added in section 6.1.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for handover and interruption delay for NR mobility is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

### 6.1.1	NR Handover

#### 6.1.1.1	Introduction

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers between NR cells.

#### 6.1.1.2	NR Handover

### 6.1.2	NR Handover to other RATs

Editor’s note: intended to capture requirements for handover from NR to EUTRAN in this Rel-15.NR to other RATs may be considered in the future release.

#### 6.1.2.1	Introduction

The handover procedure is initiated from NR with a RRC message that implies a handover as described in TS 38.331 [x].

#### 6.1.2.2	NR – E-UTRAN FDD and NR – E-UTRAN TDD Handover

The purpose of inter-RAT handover from NR to E-UTRAN FDD and TDD is to change the radio access mode from NR to E-UTRAN FDD or TDD.

#### 6.1.2.2.1	Handover delay

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last slot containing the RRC command when UE is configured with normal handover.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause x.y in TS 38.331 [x] plus the interruption time stated in clause 6.1.2.2.2.

#### 6.1.2.2.2	Interruption time

When inter-RAT handover to E-UTRAN is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + [20] ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement, a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 (FDD) of [36.133] and Clause 8.1.2.2.1 (TDD) of [36.133] for intra-frequency handover.

<< End of changes>>