3GPP TR 38.813 V0.2.0 (2018-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

New frequency range for NR (3.3-4.2 GHz)

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

5	NR Frequency band definition	7

6	Band numbering and channel bandwidth	7

6.1	Band numbering	7

6.2	Channel bandwidth	8

6.2.1	Channel bandwidth Set [NR_newRAT]	8

7	NR band 3.3GHz - 3.8GHz specific issues	8

7.1	UE specific	8

7.1.1	Transmitter characteristics	8

7.1.1.1	UE maximum output power	8

7.1.1.2	UE maximum output power for modulation / channel bandwidth	9

7.1.1.3	UE maximum output power with additional requirements	9

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	9

7.1.1.5	Spectrum emission mask	9

7.1.1.6	Spurious emissions	10

7.1.1.7	Tx requirements for UL MIMO with PC2	11

7.1.1.7.1	UE maximum output power	11

7.1.1.7.2	Configured transmitted power	11

7.1.1.7.3	Minimum output power	12

7.1.1.7.4	Transmit OFF power	12

7.1.1.7.5	ON/OFF time mask	12

7.1.1.7.6	Power control	12

7.1.1.7.7	Frequency error for UL-MIMO	12

7.1.1.7.8	Transmit modulation quality	12

7.1.1.7.9	Occupied bandwidth for UL-MIMO	13

7.1.1.7.10	Out of band emission for UL-MIMO	13

7.1.1.7.11	Spurious emission for UL-MIMO	13

7.1.1.7.12	Transmit intermodulation for UL-MIMO	13

7.1.1.7.13	Time alignment error for UL-MIMO	13

7.1.2	Receiver characteristics	14

7.1.2.1	Reference sensitivity	14

7.1.2.2	Adjacent Channel Selectivity (ACS)	15

7.1.2.3	Blocking	16

7.1.2.3.1	Out-of-band blocking	16

7.1.2.4	Rx requirements for UL MIMO with PC2	17

7.1.2.4.1	Reference sensitivity level	17

7.1.2.4.2	Maximum input level	17

7.1.2.4.3	ACS	17

7.1.2.4.4	Blocking	17

7.1.2.4.5	Spurious response	17

7.1.2.4.6	Receiver intermodulation	18

7.2	BS specific	18

7.2.1	Operating band unwanted emissions	18

7.2.2	Additional spurious emissions requirements	20

7.2.3	Co-location with other base stations	21

7.2.4	General blocking requirement	22

7.2.5	Blocking requirement for co-location with other base stations	23

8	NR band 3.3GHz - 4.2GHz specific issues	23

8.1	UE specific	23

8.1.1	Transmitter characteristics	23

8.1.1.1	UE maximum output power	23

8.1.1.2	UE maximum output power for modulation / channel bandwidth	23

8.1.1.3	UE maximum output power with additional requirements	23

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)	24

8.1.1.5	Spectrum emission mask	24

8.1.1.6	Spurious emissions	25

8.1.2	Receiver characteristics	25

8.1.2.1	Reference sensitivity	25

8.1.2.2	Adjacent Channel Selectivity (ACS)	25

8.1.2.3	Blocking	27

8.1.2.3.1	Out-of-band blocking	27

8.2	BS specific	28

8.2.1	Operating band unwanted emissions	28

8.2.2	Additional spurious emissions requirements	31

8.2.3	Co-location with other base stations	31

8.2.3.1	Minimum requirement for BS type 1-C and BS type 1-H	31

8.2.4	General blocking requirement	32

8.2.4.1	Minimum requirement for BS type 1-C and BS type 1-H	32

8.2.4.2	Minimum requirement for BS type 1-O	33

8.2.5	Blocking requirement for co-location with other base stations	33

9	Required changes to NR, E-UTRA, UTRA and MSR specifications	33

Annex <A> (normative): <Normative annex title>	34

Annex B: Change history………………………………………………………………………………35

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for New frequency range for NR (3.3-4.2 GHz). The purpose of this technical report is to provide specification support for NR bands 3.3-3.8 GHz and 3.3-4.2 GHz.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	R4-1706893, “WF on 3.5GHz NR band definition”, CMCC, Vodafone,  Ericsson,  Qualcomm, Skyworks, Huawei, HiSilicon, CATT, ZTE, Telecom Italia, Orange, Deutsche Telekom, BT, Broadcom, China Telecom, China Unicom

[3]	R4-1709181, “WF on band numbering”, NTT DOCOMO

[4]	R4-1708845, “WF on UE mandatory channel bandwidth”, Nokia

[5] 	R4-1711732, “WF on BS channel BW set”, Huawei, Hisilicon, Vodafone, Ericsson

[6]	R4-1710957, “TP to TR 38.817-01: Futher ACLR agreements”, Nokia

[7]	R4-1710962, “TP to TS 38.101-1 Output RF spectrum emissions”, Nokia

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Background

In 3GPP RAN4 #AH2 meeting, it is proposed that new bands for NR are assigned band numbers on a “first come first served” basis in reserved ranges regardless of duplex mode or RAT. Then, in RAN4 #84 meeting, band 3.3-3.8 GHz and band 3.3-4.2GHz are assigned band number n78 and n77 respectively.

5	NR Frequency band definition

In 3GPP, RAN 4 specified two NR operating bands n77 and n78 in 3.3GHz-4.2GHz. A UE supporting n78 band is not required to support n77 band [2].

6	Band numbering and channel bandwidth

6.1	Band numbering

# For 3.3-4.2GHz frequency range the NR frequency bands are defined as in Table 6.1-1.

# Table 6.1-1: NR frequency bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive | Duplex Mode |
| --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n77 | 3300MHz-4200MHz | 3300MHz-4200MHz | TDD |
| n78 | 3300MHz-3800MHz | 3300MHz-3800MHz | TDD |

6.2	Channel bandwidth

6.2.1	Channel bandwidth Set [NR_newRAT]

# The superset of channel bandwidth for UE was agreed in Table 6.2.1-1:

# Table 6.2.1-1 UE channel bandwidth set

| NR Band | Data SCS = 15kHz |  |  |  |  | Data SCS = 30kHz |  |  |  |  |  |  |  | Data SCS = 60kHz (for more than 1GHz bands) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 MHZ | 15 MHz | 20 MHZ | 40 MHz | 50 MHz | 10 MHZ | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 10 MHz (NOTE) | 15 MHz | 20 MHz | 40 MH z | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n77 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| n78 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| NOTE: 90% spectrum utilization may not be achieved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For band n77 and band n78, addition to the UE channel BWs for the band, 30 MHz, 70 MHz and 90 MHz are supported for BS. These new BS CBWs have low priority compared to the UE channel BWs.

7	NR band 3.3GHz - 3.8GHz specific issues

7.1	UE specific

7.1.1	Transmitter characteristics

7.1.1.1	UE maximum output power

The MOP requirements for Band 42 and 43 single-carrier were specified as 23 dBm +2/-3 dB for power class 3 based on simulation results shown in Table 7.1.1.1-1 in August 2011.

# Table 7.1.1.1-1.  Simulation results for combined Band 42 and Band 43 filter [R4-114656]

|  | Bandwidth | Max IL (corner) | 2f0 rejection | 2.7 GHz rejection |
| --- | --- | --- | --- | --- |
| Design 7 | 400 MHz | 1.9 | 20 dB | 30 dB |
| Design 8 | 400 MHz | 2.3 | 15 dB | 10 dB |
| Design 9 | 400 MHz | 2.0 | 20 dB | 15 dB |

When specifying MOP requirements of Band n77 and n78, the delta to be checked is IL impact of BPF due to the extension of pass-bandwidth. It is shown in Table 7.1.1.1-2.

# Table 7.1.1.1-2.  Simulation results for Band n77 and n78

| Parameter | Frequency range | Vendor 1 |  | Vendor 2 |  | Vendor 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF | Band n78 BPF | Band n77 BPF |
| Insertion loss (ETC) | 3300-3400 MHz | 1.05 dB | 1.05 dB | 2.0 dB | 2.0 dB | 1.9 dB | 1.9 dB |
|  | 3400-3800 MHz | 1.0 dB | 1.0 dB | 1.5 dB | 1.5 dB |  |  |
|  | 3800-4200 MHz | - | 1.25 dB | - | 2.0 dB | - |  |
| Attenuation (Typ) | 698-2690 MHz | 41.0 dB | 41.0 dB | 45 dB | 35 dB (ETC) | 46.3 dB | 47.8 dB |
|  | 5150-5925 MHz | 40.2 dB | 40.2 dB | 35 dB | 35 dB (ETC) | 40.8 dB | 35.6 dB |

With comparison between Table 7.1.1.1-1 and Table 7.1.1.1-2, it was concluded that there is no degradation of IL at ETC even with the extension and agreed to specify the same MOP as that of Band 42 and 43 (i.e., 23 dBm +2/-3 dB) for Band n77 and n78 of power class 3.

# Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

7.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

7.1.1.3	UE maximum output power with additional requirements

<Text to be added>

7.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

NR adjacent channel leakage power ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 7.1.1.4-1.

If the measured adjacent channel power is greater than [–50dBm] then the NRACLR shall be higher than the value specified in Table 7.1.1.4-2.

# Table 7.1.1.4-1: NR ACLR measurement bandwidth

|  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR ACLR measurement bandwidth | 9.375 | 14.235 | 19.095 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

# Table 7.1.1.4-2: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

7.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 7.1.1.5-1 for the specified channel bandwidth.

# Table 7.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

7.1.1.6	Spurious emissions

Since Band n77 and/or n78 will be used at least in Japan, bands operated in Japan need to be protected. In addition, protected bands required in Europe, Korea and China regions (i.e., operating bands specified for Band n78 of 3.3-3.8 GHz) should also be added for potential use of these bands. As a consequence, required protected bands for Band n77 and n78 will be the same. Note that it was already agreed not to specify the co-existence requirement between Band n77 and Band n79, which means that the co-existence will be guaranteed by the NR general emission requirements.

# Table 7.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

7.1.1.7	Tx requirements for UL MIMO with PC2

7.1.1.7.1	UE maximum output power

For PC2 UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 7.1.1.7.1-1. For UE supporting UL-MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL-MIMO configurations of using 2-layer UL-MIMO transmission with codebook of. Whether other code book will be introduced in Rel-15 is FFS. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

# Table 7.1.1.7.1-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 |  |  | 26 | +2/-3 |  |  |  |  |
| n78 |  |  | 26 | +2/-3 |  |  |  |  |

# If UE is configured for transmission on single-antenna port, the requirements in Table 7.1.1.7.1-2 shall apply.

# Table 7.1.1.7.1-2: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 |  |  |  |  | 23 | ±2 |  |  |
| n78 |  |  |  |  | 23 | ±2 |  |  |

For a power class 2 capable UE supporting UL MIMO, applicability to certain restriction of uplink/downlink configuration is TBD.

7.1.1.7.2	Configured transmitted power

# For UE supporting UL-MIMO, the transmitted power is configured per each UE.

For PC2 UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 7.1.1.7.2-1. The requirements shall be met with UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# Table 7.1.1.7.2-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c =26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

# If UE is configured for transmission on single-antenna port, the requirements in Table 7.1.1.7.2-2 apply.

# Table 7.1.1.7.2-2: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

7.1.1.7.3	Minimum output power

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1ms). The minimum output power shall not exceed the values specified for single carrier.

# If UE is configured for transmission on single-antenna port, the requirements specified for single carrier shall apply.

7.1.1.7.4	Transmit OFF power

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1ms) excluding any transient periods.

# The transmit OFF power at each transmit antenna connector shall not exceed the values specified for single carrier.

7.1.1.7.5	ON/OFF time mask

# For UE supporting UL-MIMO, the ON/OFF time mask requirements apply at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the general ON/OFF time mask requirements apply.

7.1.1.7.6	Power control

For UE supporting UL-MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified for single carrier apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the requirements for single carrier apply.

7.1.1.7.7	Frequency error for UL-MIMO

For UE(s) supporting UL-MIMO, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of one sub-frame (1 ms) compared to the carrier frequency received from the NR Node B.

7.1.1.7.8	Transmit modulation quality

For UE supporting UL-MIMO, the transmit modulation quality requirements are specified at each transmit antenna connector.

# If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

# The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

7.1.1.7.9	Occupied bandwidth for UL-MIMO

For UE supporting UL-MIMO, the requirements for occupied bandwidth is specified at each transmit antenna connector. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified for single carrier. The requirements shall be met with UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

7.1.1.7.10	Out of band emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified for single carrier apply to each transmit antenna connector. The requirements shall be met with UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

7.1.1.7.11	Spurious emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified for single carrier apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the general requirements specified for single carrier apply.

7.1.1.7.12	Transmit intermodulation for UL-MIMO

For UE supporting UL-MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements specified for single carrier apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1.

# If UE is configured for transmission on single-antenna port, the requirements specified for single carrier apply.

7.1.1.7.13	Time alignment error for UL-MIMO

For UE(s) with multiple transmit antenna connectors supporting UL-MIMO, this requirement applies to frame timing differences between transmissions on multiple transmit antenna connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors.

# For UE(s) with multiple transmit antenna connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

7.1.2	Receiver characteristics

7.1.2.1	Reference sensitivity

Based on Table 7.1.1.1-2 and more simulation results in Table 7.1.2.1-1, 7.1.2.1-2 and 7.1.2.1-3 whose attenuation is at least typ. 35 dB below 2690 MHz and above 5150 MHz,  the following was agreed.

# Table 7.1.2.1-1. Filter simulation results reported by Vendor A in [R4-1711334]

| Filter | Nominal IL [dB] |  | Min Attenuation [dB] @ |  |
| --- | --- | --- | --- | --- |
|  | w/o 30 dB Rejection in Radio Altimeter Band | with 30 dB Rejection in Radio Altimeter Band | 693-2690 MHz | 5150-5925 MHz |
| n77 (3.3-4.2 GHz) | 1.8 | 2.7 | >35 | >35 |
| n78 (3.3-3.8 GHz) | 1.5 | 1.8 | >35 | >35 |

Table 7.1.2.1-2. Filter simulation results reported by Vendor B in [R4-1711334]

| Filter | IL [dB] |  | Min Attenuation [dB] @ |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Nominal | Worst Case | 693-2690 MHz | 5150-5925 MHz | Radio Altimeter |
| n77 (3.3-4.2 GHz) | 1.87 | TBC | 44 | 46 |  |
| n78 (3.3-3.8 GHz) | 1.56 | TBC | 40.5 | 51 |  |

Table 7.1.2.1-3. Filter simulation results reported in [R4-1711334]

| Filter | IL [dB] |  |  |  |  |  | Min Attenuation [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nominal |  |  | Worst case |  |  |  |  |  |  |  |  |
| Frequencies [GHz] | 3.3 | 3.8 | 4.2 | 3.3 | 3.8 | 4.2 | 2.3-2.69 | 0.617-2.2 | 4.4-5.0 | 4.8-5.0 | 5.15-5.925 | 5.250-5.925 |
| n77 (3.3-4.2 GHz) | 2.6 | 1.9 | 2.4 | 2.9 | 2.1 | 2.6 | 35 | 45 | 5 | 18 | 40 | 45 |
| n78 (3.3-3.8 GHz) | 2.1 | 1.8 | - | 2.3 | 2.1 | - | 35 | 40 | 22 | 25 | 40 | 45 |

Agreement: REFSENS for Band n77 at 3.3-3.8 GHz and n78 should be 1 dB larger than that of bands which have NR smallest sensitivity (less RF challenges) such as Band n1. REFSENS for Band n77 at 3.8-4.2 GHz is increased by 0.5 dB compared to that of Band n78.

# Table 7.1.2.1-4. Reference sensitivity for Band n78

| Operating Band | SCS kHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |  |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| n78 | 15 | -95.8 | -94.0 | -92.7 | -89.6 | -88.6 |  |  |  | TDD |
|  | 30 | -96.1 | -94.1 | -92.9 | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  | 60 | -96.5 | -94.4 | -93.1 | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |

7.1.2.2	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

It is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.1.2.2-2 and Table 7.1.2.2-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 38.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in TS 38.101 Annex A.5.1.1/A.5.2.1).

# Table 7.1.2.2-1: Adjacent channel selectivity

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | ACS | dB | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] |

# Table 7.1.2.2-2: Test parameters for Adjacent channel selectivity, Case 1

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |
|  | PInterferer | dBm | REFSENS [+45.5]dB | REFSENS [+45.5] dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

# Table 7.1.2.2-3: Test parameters for Adjacent channel selectivity, Case 2

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz |  | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n78 | Power in Transmission Bandwidth Configuration | dBm | [-56.5] | [-56.5] |  | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] |
|  | PInterferer | dBm | -25 |  |  |  |  |  |  |  |  |
|  | BWInterferer | MHz | 10 | 15 | 20 |  | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 |  | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | NA | NA | NA |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 |  | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 |  | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

7.1.2.3	Blocking

7.1.2.3.1	Out-of-band blocking

Based on filter performance shown in Table 7.1.1.1-2, out-of-band blocking requirement needs to be optimized as with LTE band 42 and 43 as shown in Table 7.1.2.3-1. Wanted signal level is the same as that of LTE refarming bands.

# Agreement: Out-of-band blocking for n78 should be specified as below.

# Table 7.1.2.3.1-1: Out-of-band blocking parameters for n78

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  | 6 | 7 | 9 | 9 | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is TBD NOTE 3: The REFSENS power level is TBD |  |  |  |  |  |  |  |  |  |

Table 7.1.2.3.1-2: Out of band blocking for Band n78

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n78 (NOTE 2) | Finterferer (CW) | MHz | -60 < f – FDL_low < -3CBW or 3CBW < f – FDL_high < 60 | -200 < f – FDL_low -60 or 60 ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – 200 or FDL_high + 200 ≤ f ≤ 12750 |
| n78 (NOTE 3) | FInterferer (CW) | MHz | N/A | -200 < f – FDL_low ≤ -MIN(200,3CBW) or MIN(200,3CBW) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MIN(200,3CBW) or FDL_high + MIN(200,3CBW) ≤ f ≤ 12750 |
| NOTE 1: CBW denotes the channel bandwidth of the wanted signal NOTE 2: For CBW < 20 MHz NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*CBW from the band edge. |  |  |  |  |  |

7.1.2.4	Rx requirements for UL MIMO with PC2

7.1.2.4.1	Reference sensitivity level

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PUMAX is the total transmitter power over the two transmits power over the two transmit antenna connectors.

7.1.2.4.2	Maximum input level

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

7.1.2.4.3	ACS

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

7.1.2.4.4	Blocking

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

7.1.2.4.5	Spurious response

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

7.1.2.4.6	Receiver intermodulation

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified for single carrier shall be met with the UL-MIMO configurations described in sub-clause 7.1.1.7.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

7.2	BS specific

7.2.1	Operating band unwanted emissions

In TS38.104, the operating band unwanted emission (OBUE) limits are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. For Band n78, ΔfOBUE =40 MHz.

# 7.2.1.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted operating band unwanted emissions requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included, and the existing requirements in clause 6.6.4 in TS38.104 can be applied.

- For the basic limits,  the existing tables of requirements for Category A or Category B in sub-clause 6.6.4.2 are applicable .

- For BS type 1-C,  the unwanted emissions in subclause 6.6.4.3 are applicable.

- For BS type 1-H,  the operating band unwanted emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  6.6.4.2, the same scaling factor methods in sub-clause 6.6.4.4 are applied.

# Editor note: The additional unwanted emission requirements shall be added if any.

<< Extract from TS 38.104>>

# 6.6.4.2	Basic limits (FR1)

# 6.6.4.2.2	Basic limits for Wide Area BS (Category A)

# For NR BS operating in Band n78, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.2-2:

Table 6.6.4.2.2-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.3	Basic limits for Wide Area BS (Category B)

# For NR BS operating in Band n78, emissions shall not exceed the maximum levels specified in Tables 6.6.4.2.3-2:

Table 6.6.4.2.3-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -15 dBm/1 MHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.4	Basic limits for Medium Range BS (Category A and B)

For Medium Range BS, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.4-1 and Table 6.6.4.2.4-2.

# Table 6.6.4.2.4-1: Medium Range BS operating band unwanted emission limits, 31< Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Pmax,c-60dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Pmax,c-60dB, -25dBm) (Note 3) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Pmax,c-60dB, -25dBm)/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# Table 6.6.4.2.4-2: Medium Range BS operating band unwanted emission limits, Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 3) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29dBm/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.5	Basic limits for Local Area BS (Category A and B)

# For Local Area BS, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.5-1.

# Table 6.6.4.2.5-1: Local Area BS operating band unwanted emission limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -37 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 10) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

< End of the extract from TS 36.104>>

#### 7.2.1.2	Minimum requirement for BS type 1-O

The OTA operating band unwanted emissions requirements are based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 7.2.1.1.

7.2.2	Additional spurious emissions requirements

# 7.2.2.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted additional spurious emissions requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n78:

- The basic limits are defined in subclause 7.2.2.1.1.

- For BS type 1-C,  the emissions requirements shall be below the emission limits defined in subclause 7.2.2.1.1.

- For BS type 1-H,  the emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  7.2.2.1.1, the same scaling factor methods in sub-clause 6.6.5.4 in TS38.104 are applied.

The additional spurious emission requirement may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. It is proposed that the same limits as E-UTRA (i.e. -52 dBm/MHz) can be applied for basic limits for NR band n78 . 7.2.2.1.1	Basic limits (FR1)

# The power of any spurious emission shall not exceed the limits of Table 7.2.2.1.1-1.

Table 7.2.2.1.1-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n78 | 3.3 – 3.8 GHz | [-52] dBm | 1 MHz | This requirement does not apply to NR BS operating in Band n77 or n78. |

Editor note: The additional spurious emission requirements for band n78 in certain regions shall be added if any.

# 7.2.2.2	Minimum requirement for BS type 1-O

The OTA minimum requirement is based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 7.2.2.1.

7.2.3	Co-location with other base stations

# 7.2.3.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted co-located with other base stations requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n78:

- The basic limits are defined in subclause 7.2.3.1.1.

- For BS type 1-C,  the emissions requirements shall be below the emission limits defined in subclause 7.2.3.1.1.

- For BS type 1-H,  the emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  7.2.3.1.1, the same scaling factor methods in sub-clause 6.6.5.4 in TS38.104 are applied.

The conduct requirements assume a 30dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class. For NR Bands n78, it is proposed that the same limits as E-UTRA can be applied for NR band n78 .

#### 7.2.3.1.1	Basic limits (FR1)

# The power of any spurious emission shall not exceed the limits of Table 7.2.3.1.1-1 to 7.2.3.1.1-3.

# Table 7.2.3.1.1-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA NR Band n78 | 3.3 – 3.8 GHz | [-96] dBm | 100 kHz |  |

# Table 7.2.3.1.1-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| MR NR Band n78 | 3.3 – 3.8 GHz | [-88] dBm | 100 kHz |  |

# Table 7.2.3.1.1-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| LA NR Band n78 | 3.3 – 3.8 GHz | [-91] dBm | 100 kHz |  |

#### 7.2.3.2	Minimum requirement for BS type 1-O

The OTA minimum requirements are based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 7.2.3.1.

7.2.4	General blocking requirement

# 7.2.4.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted general blcoking requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n78:

- The basic limits are defined in subclause 7.2.4.1.1.

- For BS type 1-C,  the minimum conducted requirement shall be below the emission limits defined in subclause 7.2.4.1.1.

- For BS type 1-H, the minimum conducted requirement shall be below the emission limits defined in subclause 7.2.4.1.1.

The blocking requirements apply in the in-band blocking frequency range, which is from 60 MHz below the lowest frequency of the uplink operating band up to 60 MHz above the highest frequency of the uplink operating band for NR Band n78, but excludes the downlink frequency range of the operating band.

7.2.4.1.1	Basic limits (FR1)

For NR Band n78, the wanted and the interfering signal coupled to BS antenna input are specified in Tables 7.2.4.1.1-1 ,7.2.4.1.1-2  and 7.2.4.1.1-3 for general blocking, narrowband blocking and out-of-band blocking requirements, respectively.

# Table 7.2.4.1.1-1: Base Station general blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 10, 15, 20 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | # 7.5+[offset] | 5MHz NR signal |
| 40, 50, 60, 80,100 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 30+[offset] | 20MHz NR signal |

# Table 7.2.4.1.1-2: Base Station narrowband blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 10, 15, 20 | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 | ±(350+[offset]+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB |
| 40, 50, 60, 80,100 | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 | ±(1400+[offset]+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB |

Table 7.2.4.1.1-3: Out-of-band blocking performance requirement for NR

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n78 | 1 (FUL_high +60) | to to | (FUL_low -60) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

# 7.2.4.2	Minimum requirement for BS type 1-O

# Detailed structure of the subclause is TBD.

7.2.5	Blocking requirement for co-location with other base stations

# <Text to be added>

8	NR band 3.3GHz - 4.2GHz specific issues

8.1	UE specific

# For Power Class 2 UE in Band n77 supporting UL MIMO, the requirements in Clause 7.1.1.7 and 7.1.2.4 apply.

8.1.1	Transmitter characteristics

8.1.1.1	UE maximum output power

Based on the same discussion as UE maximum output power of Band n78 described in clause 7.1.1.1, the following was agreed.

# Agreement: MOP is to be specified as 23 dBm +2/-3 dB for Band n77 and n78 of power class 3

8.1.1.2	UE maximum output power for modulation / channel bandwidth

<Text to be added>

8.1.1.3	UE maximum output power with additional requirements

<Text to be added>

8.1.1.4	Adjacent Channel Leakage Ratio (ACLR)

NR adjacent channel leakage power ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 8.1.1.4-1.

If the measured adjacent channel power is greater than [–50dBm] then the NRACLR shall be higher than the value specified in Table 8.1.1.4-2.

# Table 8.1.1.4-1: NR ACLR measurement bandwidth

|  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR ACLR measurement bandwidth | 9.375 | 14.235 | 19.095 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

# Table 8.1.1.4-2: NR ACLR requirement

|  | Power class 1 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NRACLR |  | 31 dB | 30 dB |

8.1.1.5	Spectrum emission mask

The power of any UE emission shall not exceed the levels specified in Table 8.1.1.5-1 for the specified channel bandwidth.

# Table 8.1.1.5-1: NR General spectrum emission mask

| ΔfOOB (MHz) | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± 0-1 | -18 | -20 | -21 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |
| ± 10-15 | -25 |  |  |  |  |  |  |  |  |
| ± 15-20 |  | -25 |  |  |  |  |  |  |  |
| ± 20-25 |  |  | -25 |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  |  |  |  |  |
| ± 30-40 |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  | -25 |  |

8.1.1.6	Spurious emissions

Based on the same discussion as Spurious emissions of Band n78 described in clause 8.1.1.6, the following was agreed.

# Table 8.1.1.6-1: Spurious emission band UE co-existence for Band n77 and n78

| NR band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |
|  | NR Band n257 | 26500 | - | 29500 | [-5] | 100 |  |

Agreement: Protected bands required for Band n77 are to be the same as those for Band n78 (i.e., not only Japanese bands but also ones required for other regions such as Europe, Korea and China)

8.1.2	Receiver characteristics

8.1.2.1	Reference sensitivity

Based on the same discussion as reference sensitivity of Band n78 described in clause 7.1.2.1, the following was agreed.

Agreement: REFSENS for Band n77 at 3.3-3.8 GHz and n78 should be 1 dB larger than that of bands which have NR smallest sensitivity (less RF challenges) such as Band n1. REFSENS for Band n77 at 3.8-4.2 GHz is increased by 0.5 dB compared to that of Band n78.

# Table 8.1.2.1-1. Reference sensitivity for Band n77

| Operating Band | SCS kHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |  |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |
| n77 (3.3 to 3.8 GHz) | 15 | -95.8 | -94.0 | -92.7 | -89.6 | -88.6 |  |  |  | TDD |
|  | 30 | -96.1 | -94.1 | -92.9 | -89.7 | -88.7 | -87.9 | -86.6 | -85.6 |  |
|  | 60 | -96.5 | -94.4 | -93.1 | -89.9 | -88.8 | -88.0 | -86.7 | -85.7 |  |
| n77 (3.8 to 4.2 GHz) | 15 | -95.3 | -93.5 | -92.2 | -89.1 | -88.1 |  |  |  | TDD |
|  | 30 | -95.6 | -93.6 | -92.4 | -89.2 | -88.2 | -87.4 | -86.1 | -85.1 |  |
|  | 60 | -96.0 | -93.9 | -92.6 | -89.4 | -88.3 | -87.5 | -86.2 | -85.2 |  |

8.1.2.2	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

It is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 8.1.2.2-2 and Table 8.1.2.2-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 38.101 Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in TS 38.101 Annex A.5.1.1/A.5.2.1).

# Table 8.1.2.2-1: Adjacent channel selectivity

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n77 | ACS | dB | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] | [33.0] |

# Table 8.1.2.2-2: Test parameters for Adjacent channel selectivity, Case 1

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40MHz | 50 MHz | 60MHz | 80MHz | 100 MHz |
| n77 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |
|  | PInterferer | dBm | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB | REFSENS [+45.5]dB |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

# Table 8.1.2.2-3: Test parameters for Adjacent channel selectivity, Case 2

| NR band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 MHz | 15 MHz | 20 MHz | 40MHz | 50 MHz | 60MHz | 80MHz | 100 MHz |
| n77 | Power in Transmission Bandwidth Configuration | dBm | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] | [-56.5] |
|  | PInterferer | dBm | -25 |  |  |  |  |  |  |  |
|  | BWInterferer | MHz | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 |
|  | FInterferer (offset) For SCS of 15kHz | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 40+0.0125 / -40-0.0125 | 50+0.0025 / -50-0.0025 | N/A | N/A | N/A |
|  | FInterferer (offset) For SCS of 30kHz | MHz | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 40+0.005 / -40-0.005 | 50+0.025 / -50-0.025 | 60+0.015 / -60-0.015 | 80+0.025 / -80-0.025 | 100+0.005 / -100-0.005 |
|  | FInterferer (offset) For SCS of 60kHz | MHz | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 40+0.050 / -40-0.050 | 50+0.010 / -50-0.010 | 60+0.030 / -60-0.030 | 80+0.010 / -80-0.010 | 100+0.050 / -100-0.050 |

8.1.2.3	Blocking

8.1.2.3.1	Out-of-band blocking

Based on the same discussion as out-of-band blocking of Band n78 described in clause 7.1.2.3.1, the following was agreed.

# Agreement: Out-of-band blocking for Band n77 and n78 should be specified as below.

# Table 8.1.2.3.1-1: Out-of-band blocking parameters for Band n77 and n78

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  | 6 | 7 | 9 | 9 | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is TBD NOTE 3: The REFSENS power level is TBD |  |  |  |  |  |  |  |  |  |

Table 8.1.2.3.1-2: Out of band blocking for Band n77 and n78

| NR band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| n77 (NOTE 2) | Finterferer (CW) | MHz | -60 < f – FDL_low < -3CBW or 3CBW < f – FDL_high < 60 | -200 < f – FDL_low -60 or 60 ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – 200 or FDL_high + 200 ≤ f ≤ 12750 |
| n77 (NOTE 3) | FInterferer (CW) | MHz | –N/A | -200 < f – FDL_low ≤ -MIN(200,3CBW) or MIN(200,3CBW) ≤ f – FDL_high < 200 | 1 ≤ f ≤ FDL_low – MIN(200,3CBW) or FDL_high + MIN(200,3CBW) ≤ f ≤ 12750 |
| NOTE 1: CBW denotes the channel bandwidth of the wanted signal NOTE 2: For CBW < 20 MHz NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3*CBW from the band edge. |  |  |  |  |  |

8.2	BS specific

8.2.1	Operating band unwanted emissions

In TS38.104, the operating band unwanted emission (OBUE) limits are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. For Band n77,  ΔfOBUE =40 MHz.

# 8.2.1.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted operating band unwanted emissions requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included, and the existing requirements in clause 6.6.4 in TS38.104 can be applied.

- For the basic limits,  the existing tables of requirements for Category A or Category B in sub-clause 6.6.4.2 in TS38.104 are applicable .

- For BS type 1-C,  the unwanted emissions in subclause 6.6.4.3 in TS38.104 are applicable.

- For BS type 1-H,  the operating band unwanted emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  6.6.4.2 in TS38.104, the same scaling factor methods in sub-clause 6.6.4.4  are applied.

# Editor note: The additional unwanted emission requirements shall be added if any.

<< Extract from TS 38.104>>

# 6.6.4.2	Basic limits (FR1)

# 6.6.4.2.2	Basic limits for Wide Area BS (Category A)

# For NR BS operating in Band n77, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.2-2:

Table 6.6.4.2.2-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/1 MHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.3	Basic limits for Wide Area BS (Category B)

# For NR BS operating in Band  n77, emissions shall not exceed the maximum levels specified in Tables 6.6.4.2.3-2:

Table 6.6.4.2.3-2: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -15 dBm/1 MHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.4	Basic limits for Medium Range BS (Category A and B)

For Medium Range BS, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.4-1 and Table 6.6.4.2.4-2.

# Table 6.6.4.2.4-1: Medium Range BS operating band unwanted emission limits, 31< Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Pmax,c-60dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Pmax,c-60dB, -25dBm) (Note 3) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Pmax,c-60dB, -25dBm)/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

# Table 6.6.4.2.4-2: Medium Range BS operating band unwanted emission limits, Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 3) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29dBm/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap.] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

6.6.4.2.5	Basic limits for Local Area BS (Category A and B)

# For Local Area BS, emissions shall not exceed the maximum levels specified in Table 6.6.4.2.5-1.

# Table 6.6.4.2.5-1: Local Area BS operating band unwanted emission limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Emission limit (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -37 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 10) | 100 kHz |
| [NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz.] [NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap] NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

< End of the extract from TS 36.104>>

# 8.2.1.2	Minimum requirement for BS type 1-O

The OTA operating band unwanted emissions requirements are based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 8.2.1.1.

8.2.2	Additional spurious emissions requirements

# 8.2.2.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted additional spurious emissions requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n77:

- The basic limits are defined in subclause 8.2.2.1.1.

- For BS type 1-C,  the emissions requirements shall be below the emission limits defined in subclause 8.2.2.1.1.

- For BS type 1-H,  the emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  8.2.2.1.1, the same scaling factor methods in sub-clause 6.6.5.4 in TS38.104 are applied.

The additional spurious emission requirement may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. It is proposed that the same limits as E-UTRA (i.e. -52 dBm/MHz) can be applied for basic limits for NR band n77.

8.2.2.1.1	Basic limits (FR1)

# The power of any spurious emission shall not exceed the limits of Table 8.2.2.1.1-1.

Table 8.2.2.1.1-1: BS Spurious emissions limits for E-UTRA BS for co-existence with systems operating in other frequency bands

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Basic limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n77 | 3.3 – 4.2 GHz | [-52] dBm | 1 MHz | This requirement does not apply to NR BS operating in Band n77 or n78. |

# Editor note: The additional unwanted emission requirements shall be added if any.

# 8.2.2.2	Minimum requirement for BS type 1-O

The OTA minimum requirement is based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 8.2.2.1.

8.2.3	Co-location with other base stations

8.2.3.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted co-located with other base stations requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n77:

- The basic limits are defined in subclause 8.2.3.1.1.

- For BS type 1-C,  the emissions requirements shall be below the emission limits defined in subclause 8.2.3.1.1.

- For BS type 1-H,  the emissions requirements are for each TAB connector TX min cell group and each applicable basic limit in subclause  8.2.3.1.1, the same scaling factor methods in sub-clause 6.6.5.4 in TS38.104 are applied.

The conduct requirements assume a 30dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class. For NR Bands n77, it is proposed that the same limits as E-UTRA can be applied for NR band n77.

8.2.3.1.1	Basic limits (FR1)

# The power of any spurious emission shall not exceed the limits of Table 8.2.3.1.1-1 to 8.2.3.1.1-3.

# Table 8.2.3.1.1-1: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA NR Band n77 | 3.3 – 4.2 GHz | [-96] dBm | 100 kHz |  |

# Table 8.2.3.1.1-2: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| MR NR Band n77 | 3.3 – 4.2 GHz | [-88] dBm | 100 kHz |  |

# Table 8.2.3.1.1-3: BS Spurious emissions limits for Medium range BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| LA NR Band n77 | 3.3 – 4.2 GHz | [-91] dBm | 100 kHz |  |

# 8.2.3.2	Minimum requirement for BS type 1-O

The OTA minimum requirements are based on the emission scaling, where the emission limits are defined as basic limit+9dB, where the basic limits are specified in subclause 8.2.3.1.

8.2.4	General blocking requirement

8.2.4.1	Minimum requirement for BS type 1-C and BS type 1-H

For conducted general blcoking requirements, two requirement sets of BS type 1-C and BS type 1-H minimum requirements are included.

# For NR BS operating in Band n77:

- The basic limits are defined in subclause 8.2.4.1.1.

- For BS type 1-C,  the minimum conducted requirement shall be below the emission limits defined in subclause 8.2.4.1.1.

- For BS type 1-H, the minimum conducted requirement shall be below the emission limits defined in subclause 8.2.4.1.1.

The blocking requirements apply in the in-band blocking frequency range, which is from 60 MHz below the lowest frequency of the uplink operating band up to 60 MHz above the highest frequency of the uplink operating band for NR Band n77, but excludes the downlink frequency range of the operating band.

8.2.4.1.1	Basic limits (FR1)

For NR Band n77, the wanted and the interfering signal coupled to BS antenna input are specified in Tables 8.2.4.1.1-1 ,8.2.4.1.1-2 and 8.2.4.1.1-3 for general blocking, narrowband blocking and out-of-band blocking requirements, respectively.

# Table 8.2.4.1.1-1: Base Station general blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 10, 15, 20 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 7.5+[offset] | 5MHz NR signal |
| 40, 50, 60, 80,100 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 30+[offset] | 20MHz NR signal |

# Table 8.2.4.1.1-2: Base Station narrowband blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 10, 15, 20 | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 | ±(350+[offset]+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB |
| 40, 50, 60, 80,100 | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 | ±(1400+[offset]+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB |

Table 8.2.4.1.1-3: Out-of-band blocking performance requirement for NR

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 1 (FUL_high +60) | to to | (FUL_low -60) 12750 | -15 | PREFSENS +6dB* |  | CW carrier |

8.2.4.2	Minimum requirement for BS type 1-O

# Detailed structure of the subclause is TBD.

8.2.5	Blocking requirement for co-location with other base stations

<Text to be added>

9	Required changes to NR, E-UTRA, UTRA and MSR specifications

The required changes to the 3GPP specifications for the NR band 3.3-3.8GHz are summarised in a Table 9-1.

# Table 9-1: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

The required changes to the 3GPP specifications for the NR band 3.3-4.2GHz are summarised in a Table 9-2.

# Table 9-2: Overview of 3GPP specifications with required changes

| Affected existing specifications |  |  |  |
| --- | --- | --- | --- |
| Spec No. | Subject of the CR | Comments | CR/TP (Tdoc) |

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 08/2017 | RAN4#84 | R4-1707861 |  |  | TR skeleton | N/A | 0.0.1 |
| 11/2017 | RAN4#85 | R4-1713002 |  |  | R4-1711816, TP for TR 38.813 UE RF requirements of Band n77 and n78 | 0.0.1 | 0.1.0 |
| 01/2018 | RAN4# AH-1801 | R4-1800511 |  |  | R4-1713002, TR 38.813 v0.1.0 New frequency range for NR 3.3GHz - 4.2GHz R4-1714322, TP to TR38.813: BS specific requirements(Clause 8.2); R4-1714323, TP to TR38.813: BS specific requirements(Clause 7.2);; R4-1712815,TP for TR 38.813 Finalization of Band n77 and n78; R4-1714341 [NR] TP for TR 38.813 UL MIMO UE RF requirements of Band n78 and n77; R4-1714374 TP to TR 38.813 NR band for n77 and n78 | 0.1.0 | 0.2.0 |