3GPP TSG-RAN WG4 Meeting AH-1801                           	  R4-1800551

San Diego , US, 22 - 26 Jan, 2018

Source: 	Apple

Title:           Discussion on How to Implement “Single UL allowed” Indication in TS38.101-3

Agenda Item:	  4.3.3.2.2

Document for:  Approval

# Introduction

To support Single Tx Switched UL, indication of “Single UL allowed” was agreed to add in the reference sensitivity table (Table 7.3.2.1.5-1 in 38.101-3) in RAN4 #85 meeting [1]. However, it’s believed that it is not very appropriate to capture it in this table. In this contribution, we propose to explicitly mark all the difficult band combinations in the band combination table (Section 5.2.2.2 EN-DC) in TS38.101-3.

# Discussion

Firstly, at present the indication of “Single UL allowed” for Single Tx Switched UL is carried in the reference sensitivity table (Table 7.3.2.1.5-1 in TS38.101-3). However, it’s not quite appropriate to mix the Single Tx Switched UL with general MSD table. As well known, MSD is a metric applicable for channel allocations rather than band combination. That is to say, for one band combination, different channel allocations may suffer from different IM orders. To our understanding, it’s not logically straighforward to define one band combination as difficult when some channel allocation of it only suffers IM5, though some other channel allocation may suffer from IM2/3.

Secondly, the IMD issue represented in Table 7.3.2.1.5-1 could happen on either PCell or SCell. While in Single Tx Switched UL case, only IMD issue on PCell is considerd. Using Table 7.3.2.1.5-1 for “Single UL allowed” might lead to confusion on which band actually suffers IMD2/IMD3 issue from dual UL transmission.

Then, one method proposed by companies is to only indicate the “Single UL allowed” for 1LTE band+1NR band case in Table 7.3.2.1.5-1. For other band combinations with morn than one LTE band, both NW and UE refer to the 1+1 case. One problem is that it requires extra text to guide the UE and NW implementation. For example, it should make clear for both UE and NW that when A+C is a difficult BC, any super set of BCs containing A+C are also difficult BCs, e.g., A+B+C.

Observation 1: Using Table 7.3.2.1.5-1 in TS38.101-3 to indicate “Single UL allowed” is not appropriate due to several reasons, e.g., MSD table does not differentiate whether the IMD happens on PCell or SCell.

Another alternative is to creat a new table dedicated for difficult band combinations only. This scheme is simple but not flexible enough to for maintenance. For instance, current difficult band combinations may become not difficult any more someday thanks to technology development. Then, special attention should be paid to remove the specific band combinations from the table. This alternative seems also not flexible enough to reflect the latest agreement in [2] from RAN plenary meeting #78 stating that “For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5) is mandatory with capability signaling”.

- Proposal 1 - Agree to introduce Rel-15 capability signaling to indicate whether the UE supports dynamic EN-DC power sharing - Dynamic power sharing means that the UE can operate with P_LTE + P_NR > P_powerclass configuration - Agree that the intent is to make dynamic EN-DC power sharing mandatory at a future time - Check any possible updates on status above in March - Proposal 2 - For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5) is mandatory with capability signalling - Single UL operation is optional for dynamic power sharing capable UEs

It is observed from the agreement that for UEs without dynamic LTE-NR power sharing capability, Single Tx Switched UL should be always supported, no matter UE is operating on difficult or non-difficult band combinations. In another word, UE should be capable to indicate Single Tx capability to all the LTE-NR band combinations in the capability signaling if UE does not support dynamic LTE-NR power sharing. Therefore, we should ensure the flexibility in specification to configure all the EN-DC band combinations as “Single UL allowed”.

Observation 2: Creating a new table for difficult band combinations only is not flexible enough to reflect the latest agreement from RAN plenary meeting #78 related to UEs without dynamic LTE-NR power sharing.

At last, to solve all the problems mentioned above, we propose to explicitly mark all the difficult band combinations in the band combination table (Section 5.2.2.2 EN-DC) in TS38.101-3. The corresponding draft CR is provided in [3].

Proposal 1: Explicitly mark all the difficult band combinations in the band combintion table (Section 5.2.2.2 EN-DC) in TS38.101-3.

# Conclusion

Observation 1: Using Table 7.3.2.1.5-1 in TS38.101-3 to indicate “Single UL allowed” is not appropriate due to several reasons, e.g., MSD table does not differentiate whether the IMD happens on PCell or SCell.

Observation 2: Creating a new table for difficult band combinations only is not flexible enough to reflect the latest agreement from RAN plenary meeting #78 related to UEs without dynamic LTE-NR power sharing

Proposal 1: Explicitly mark all the difficult band combinations in the band combintion table (Section 5.2.2.2 EN-DC) in TS38.101-3.

# Reference

- R4-1714443  TP to TS 38.101-3: On dual uplink operation for EN-DC in NR FR1 and single uplink  Nokia, Apple    RAN4#85

- RP-172833  Way Forward on NR EN-DC power sharing capability QC  RAN#78

- R4-1800552  Draft CR to 38.101-3: Single Tx Switched UL   Apple  Jan 2018 RAN4 NR Ad Hoc