3GPP TSG-RAN4 NR AH 	R4-1800579

# San Diego, USA, 22-26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to FR1 blocking requirement sections 7.4.2, 10.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR captures some corrections to the conducted and OTA blocking requirements: - Square brackets around bandwidths - Missing table notes - Redundant text about conducted requirements in the OTA section |  |  |  |  |  |  |  |  |
| Summary of change: |  | Square brackets removed, table notes added, redundant text removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Blocking requirements text contains not fully correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2, 10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 7.4.2	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver’s ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Basic limits

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS, the wanted and the interfering signal coupled to BS antenna input are specified in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general and narrowband blocking requirements, respectively. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each channel bandwidth and further specified in Annex A.

The blocking requirements are applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. The blocking requirements apply in the in-band blocking frequency range, which is from 20 MHz below the lowest frequency of the uplink operating band up to 20 MHz above the highest frequency of the uplink operating band for BS type 1-C in an operating band less than 200 MHz wide or BS type 1-H in an operating band less than 100 MHz wide, or is from 60 MHz below the lowest frequency of the uplink operating band up to 60 MHz above the highest frequency of the uplink operating band for BS type 1-C in an operating band more than or equal to 200 MHz wide or BS type 1-H in an operating band more than or equal to 100 MHz wide, but excludes the downlink frequency range of the operating band.

[For a BS operating in non-contiguous spectrum within any operating band, the blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1 and 7.4.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a BS capable of multi-band operation, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1 and 7.4.2.2-3.]

Table 7.4.2.2-1: Base station general blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 7.5 | 5MHz NR signal SCS: 15kHz |
| 25, 30, 40, 50, 60, 70, 80,90, 100 | PREFSENS + 6dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 30 | 20MHz NR signal SCS: 15kHz |
| NOTE: PREFSENS depends on the channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2, 7.2.2-3 |  |  |  |  |

Table 7.4.2.2-2: Base station narrowband blocking requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 (NOTE 1) | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 |
| 25, 30, 40, 50, 60, 70, 80,90, 100 (NOTE 1) | PREFSENS + 6dB | Wide Area: -49 Medium Range: -44 Local Area: -41 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: PREFSENS depends on the channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2, 7.2.2-3 |  |  |

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342,5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 10 | [347.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 15 | [352.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 20 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 25 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 30 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 40 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 50 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 60 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 70 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 80 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 90 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 100 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |

#### 7.4.2.3	Minimum requirement for BS type 1-C

Minimum conducted requirement for BS type 1-C is defined as basic limit at the antenna connector.

#### 7.4.2.4	Minimum requirement for BS type 1-H

Minimum conducted requirement for BS type 1-H is defined as basic limit at the TAB connector.

{Unchanged sections omitted}

### 10.5.2	OTA in-band blocking

#### 10.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 10.5.2.2	OTA in-band blocking for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the OTA interfering signal are specified in tables 10.5.2.1-1, table 10.5.2.1-2 and table 10.5.2.1-3 for general and narrowband blocking requirements, respectively. The reference measurement channel for the wanted signal is identified in tables 10.3.2-1, 10.3.2-2 and 10.3.2-3  for each channel bandwidth and further specified in annex A.

The blocking requirements are applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. The blocking requirements apply in the in-band blocking frequency range, which is from 20 MHz below the lowest frequency of the uplink operating band up to 20 MHz above the highest frequency of the uplink operating band for an operating band less than 100 MHz wide, or is from 60 MHz below the lowest frequency of the uplink operating band up to 60 MHz above the highest frequency of the uplink operating band for an operating band more than or equal to 100 MHz wide, but excludes the downlink frequency range of the operating band.

[For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3.]

Table 10.5.2.2-1: General OTA blocking requirement for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] | OTA Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | EISREFSENS + 6dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 7.5 | 5MHz NR signal SCS: 15kHz |
| 5, 10, 15, 20 | EISMINSENS + 6dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 7.5 | 5MHz NR signal SCS: 15kHz |
| 25,30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 30 | 20MHz NR signal SCS: 15kHz |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISMINSENS + 6dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 30 | 20MHz NR signal SCS: 15kHz |

Table 10.5.2.2-2: OTA narrowband blocking requirement for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 | EISREFSENS + 6dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
| 5, 10, 15, 20 | EISMINSENS + 6dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISMINSENS + 6dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| NOTE: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth |  |  |

Table 10.5.2.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342,5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 10 | [347.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 15 | [352.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 20 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5MHz NR signal, 1 RB SCS: 15kHz |
| 25 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 30 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 40 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 50 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 60 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 70 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 80 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 90 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |
| 100 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20MHz NR signal, 1 RB SCS: 15kHz |

#### 10.5.2.3	OTA in-band blocking for BS type 2-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2 BS, the wanted and the interfering signal coupled to BS antenna input are specified in tables 10.5.2.3-1 for general OTA blocking requirements. The reference measurement channel for the wanted signal is identified in table 10.2.1-1 for each channel bandwidth and further specified in annex A.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. The OTA blocking requirements apply in the in-band blocking frequency range, which is from [XX] MHz below the lowest frequency of the uplink operating band up to [XX] MHz above the highest frequency of the uplink operating band for BS type 2-O in an operating band less than [XX] MHz wide, but excludes the downlink frequency range of the operating band.

[For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 10.5.2.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in table 10.5.2.3-1.]

Table 10.5.2.3-1: General OTA blocking requirement for BS type 2-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA wanted signal mean power [dBm] | OTA interfering signal mean power [dBm] | OTA interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6dB | EISREFSENS + 33dB | 75 | 50MHz NR signal 60kHz SCS |