3GPP TSG-RAN4 NR AH 	R4-1800580

# San Diego, USA, 22-26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ACS requirement sections 3.1, 7.4.1.2, 10.1, 10.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR corrects some errors in the ACS sections: A definition of minSENS is currently missing. The EIS values should refer to minSENS. OTA FR1 wanted signal level incorrect The current specification text contains some redundant text about REFSENS for ACS, which is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minsense definition added. OTA FR1 wanted signal level corrected. Square brackets removed from list of bandwidths. Missing note added to table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACS definition incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.4.1.2, 10.1, 10.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.1	Definitions

{Unchanged definitions omitted}

EISminSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement.

{Unchanged section omitted}

## 7.4	In-band selectivity and blocking

### 7.4.1	Adjacent Channel Selectivity (ACS)

#### 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.4.1.2	Basic limits

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS, the wanted and the interfering signal coupled to the BS antenna input are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each channel bandwidth and further specified in Annex A.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For a BS operating in non-contiguous spectrum within any operating band, the ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.4.1.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a BS capable of multi-band operation, the ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 7.4.1.2-1: Base station ACS requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 (NOTE 1) | PREFSENS + 6dB | Wide Area: -52 Medium Range: -47 Local Area: -44 |
| 25, 30, 40, 50, 60, 70, 80, 90, 100(NOTE 1) | PREFSENS + 6dB | Wide Area: -52 Medium Range: -47 Local Area: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: PREFSENS depends on the channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2, 7.2.2-3 |  |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [2.5025] | 5MHz NR signal SCS: 15kHz |
| 10 | [2.5075] | 5MHz NR signal SCS: 15kHz |
| 15 | [2.5125] | 5MHz NR signal SCS: 15kHz |
| 20 | [2.5025] | 5MHz NR signal SCS: 15kHz |
| 25 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 30 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 40 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 50 | [10.0025] | 20MHz NR signal SCS: 15kHz |
| 60 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 70 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 80 | [10.0025] | 20MHz NR signal SCS: 15kHz |
| 90 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 100 | [10.0075] | 20MHz NR signal SCS: 15kHz |

#### 7.4.1.3	Minimum requirement for BS type 1-C

Minimum conducted requirement for BS type 1-C is defined as basic limit at the antenna connector.

#### 7.4.1.4	Minimum requirement for BS type 1-H

Minimum conducted requirement for BS type 1-H is defined as basic limit at the TAB connector.

{Unchanged sections omitted}

## 10.1	General

This subclause describes any general aspects of receiver characteristics and relations between requirements.

Editor’s note: Possible content of this sub clause are reference points for requirement, necessary declarations, etc. Related information will be covered in other parts of the specification, such as BS architecture in 4.3.

Each requirement shall be met over the RoAoA specified.

For FR1 requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) dB for the reference direction

and

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS)  dB for all other directions

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS

{Unchanged sections omitted}

### 10.5.1	OTA adjacent channel selectivity

#### 10.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 10.5.1.2	Minimum requirement for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and

the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 10.5.1.2-1 and table 10.5.1.2-2 for ACS. The reference measurement channel for the OTA wanted signal is  specified in annex A.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.2-1. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For multi-band RIBs, the OTA ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 10.5.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 10.5.1.2-1: OTA ACS requirement for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 (NOTE 1) | EISminSENS + 6dB | Wide Area: -52 – ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| 25, 30, 40, 50, 60, 70, 80,90, 100(NOTE 1) | EISminSENS + 6dB | Wide Area: -52– ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: EISminSENS depends on the channel bandwidth |  |  |

Table 10.5.1.2-2: OTA ACS interferer frequency offset for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [2.5025] | 5MHz NR/E-UTRA signal SCS: 15kHz |
| 10 | [2.5075] | 5MHz NR/E-UTRA signal SCS: 15kHz |
| 15 | [2.5125] | 5MHz NR/E-UTRA signal SCS: 15kHz |
| 20 | [2.5025] | 5MHz NR/E-UTRA signal SCS: 15kHz |
| 25 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 30 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 40 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 50 | [10.0025] | 20MHz NR signal SCS: 15kHz |
| 60 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 70 | [10.0075] | 20MHz NR signal SCS: 15kHz |
| 80 | [10.0025] | 20MHz NR signal SCS: 15kHz |
| 90 | [10.0125] | 20MHz NR signal SCS: 15kHz |
| 100 | [10.0075] | 20MHz NR signal SCS: 15kHz |

#### 10.5.1.3	Minimum requirement for BS type 2-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 10.5.1.3-1 and table 10.5.1.3-2 for ACS. The reference measurement channel for the OTA wanted signal is specified in annex A.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For multi-band RIBs, the OTA ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 10.5.1.3-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 10.5.1.3-1: OTA ACS requirement for BS type 2-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6dB | EISREFSENS +[28.7 -SINR-IM] (Note 1) EISREFSENS +[28.7 -SINR-IM] (Note 2) |
| Note 1: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz Note 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |

Table 10.5.1.3-2: OTA ACS interferer frequency offset for BS type 2-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 | [25,01] | 50MHz NR signal 60kHz SCS |
| 100 | [25.03] | 50MHz NR signal 60kHz SCS |
| 200 | [25.01] | 50MHz NR signal 60kHz SCS |
| 400 | [25.03] | 50MHz NR signal 60kHz SCS |