3GPP TSG-RAN4 NR AH 	R4-1800581

# San Diego, USA, 22-26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to RX IM requirement sections 7.7, 10.8.2, 10.8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IM interferer levels for FR1 are taken from the MSR specification instead of E-UTRA SR. The TR documents that it is agreed to use E-UTRA single RAT levels. Wanted signal level incorrect for FR1 OTA. Interferer signal subcarrier spacing not captured for OTA requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IM levels for FR1 corrected. OTA FR1 wanted signal level corrected. Interferer signal subcarrier spacing added for OTA requirements. Interferer subcarrier spacing described in tables rather than text to improve consistency with the descriptions for the other RX requirements. Editorial corrections to table notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrectly specified RX IM requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.2, 10.8.2, 10.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.7	Receiver intermodulation

### 7.7.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

### 7.7.2	Minimum requirement for BS type 1-C and BS type 1-H

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS antenna connector or BS antenna TAB connector, with the conditions specified in tables 7.7.2-1 and 7.7.2-2 for intermodulation performance and in tables 7.7.2-3, and 7.7.2-4 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 and table 7.2.2-3 for each channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the NR interfering signal in table 7.7.2-3, 7.7.2-4 or 7.7.2-5. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

[For a BS capable of multi-band operation or multi-band TAB connectors, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

[For a BS capable of multi-band operation or multi-band TAB connectors, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 7.7.2-3, 7.7.2-4 and 7.7.2-5. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 | PREFSENS +6 dB | See Table 7.7.2-2 |
| Medium Range BS | -47 | PREFSENS +6 dB |  |
| Local Area BS | -44 | PREFSENS +6 dB |  |
| NOTE 1: PREFSENS depends on the BS class and on the channel bandwidth, see subclause 7.2. |  |  |  |

Table 7.7.2-2: Interfering signals for intermodulation requirement

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±7.5] | CW |
|  | [±17.5] | 5MHz NR signal |
| 10 | [±7.45] | CW |
|  | [±17.5] | 5MHz NR signal |
| 15 | [±7.43] | CW |
|  | [±17.5] | 5MHz NR signal |
| 20 | [±7.38] | CW |
|  | [±17.5] | 5MHz NR signal |
| 30 | [±7.43] | CW |
|  | [±25] | 5MHz NR signal |
| 25 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |
| 40 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |
| 50 | [±7.35] | CW |
|  | [±25] | 20MHz NR signal |
| 60 | [±7.49] | CW |
|  | [±25] | 20MHz NR signal |
| 70 | [±7.42] | CW |
|  | [±25] | 5MHz NR signal |
| 80 | [±7.44] | CW |
|  | [±25] | 20MHz NR signal |
| 90 | [±25] | CW |
|  | [±7.43] | 5MHz NR signal |
| 100 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |

Table 7.7.2-3: Narrowband intermodulation performance requirement in FR1

| BS type | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6dB (NOTE 1) | -52 | See Table 7.7.2-4 |
| Medium Range BS | PREFSENS + 6dB (NOTE 2) | -47 |  |
| Local Area BS | PREFSENS + 6dB (NOTE 3) | -44 |  |
| NOTE 1: PREFSENS depends on the channel bandwidth as specified in table 7.2-1. NOTE 2: PREFSENS depends on the channel bandwidth as specified in table 7.2-2. NOTE 3: PREFSENS depends on the channel bandwidth as specified in table 7.2-3. |  |  |  |

Table 7.7.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±360] | CW |
|  | [±1420] | 5MHz NR signal, 1 RB (NOTE 1) |
| 10 | [±325] | CW |
|  | [±1780] | 5MHz NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | [±380] | CW |
|  | [±1600] | 5MHz NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | [±345] | CW |
|  | [±1780] | 5MHz NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | [±325] | CW |
|  | [±1990] | 20MHz NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | [±320] | CW |
|  | [±1990] | 20MHz NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | [±310] | CW |
|  | [±2710] | 20MHz NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | [±330] | CW |
|  | [±3250] | 20MHz NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | [±350] | CW |
|  | [±3790] | 20MHz NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | [±400] | CW |
|  | [±4870] | 20MHz NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | [±390] | CW |
|  | [±4870] | 20MHz NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

{Unchanged sections omitted}

### 10.8.2	Minimum requirement for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the within the minSENS RoAoA.

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB. with the conditions specified in tables 10.8.2-1 and 10.8.2-2 for intermodulation performance and in tables 10.8.2-3 and 10.8.2-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 10.3.2-1, table 10.3.2-2 and table 10.3.2-3 for each channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the NR interfering signal in tables 10.8.2-2 and 10.8.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

[For multi-band RIBs, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

[For multi-band RIBs, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in tables 10.8.2-1 and 10.8.2-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Table 10.8.2-1: General intermodulation requirement

| BS class | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 - ΔOTAREFSENS | EISREFSENS +6 dB | See Table 10.8.2-2 |
|  | -52 - ΔminSENS | EISminSENS +6 dB |  |
| Medium Range BS | -47 - ΔOTAREFSENS | EISREFSENS +6 dB |  |
|  | -47 - ΔminSENS | EISminSENS +6 dB |  |
| Local Area BS | -44 - ΔOTAREFSENS | EISREFSENS +6 dB |  |
|  | -44 - ΔminSENS | EISminSENS +6 dB |  |
| NOTE 1: EISREFSENS depends on the BS class and on the channel bandwidth, see subclause 7.2. |  |  |  |

Table 10.8.2-2: Interfering signals for intermodulation requirement

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±7.5] | CW |
|  | [±17.5] | 5MHz NR signal |
| 10 | [±7.45] | CW |
|  | [±17.5] | 5MHz NR signal |
| 15 | [±7.43] | CW |
|  | [±17.5] | 5MHz NR signal |
| 20 | [±7.38] | CW |
|  | [±17.5] | 5MHz NR signal |
| 30 | [±7.43] | CW |
|  | [±25] | 5MHz NR signal |
| 25 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |
| 40 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |
| 50 | [±7.35] | CW |
|  | [±25] | 20MHz NR signal |
| 60 | [±7.49] | CW |
|  | [±25] | 20MHz NR signal |
| 70 | [±7.42] | CW |
|  | [±25] | 5MHz NR signal |
| 80 | [±7.44] | CW |
|  | [±25] | 20MHz NR signal |
| 90 | [±25] | CW |
|  | [±7.43] | 5MHz NR signal |
| 100 | [±7.45] | CW |
|  | [±25] | 20MHz NR signal |

Table 10.8.2-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (NOTE 1) | -52 - ΔOTAREFSENS | See Table 10.8.2-4 |
|  | EISminSENS + 6 dB (NOTE 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (NOTE 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (NOTE 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (NOTE 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB(NOTE 1) | -44 - ΔminSENS |  |
| NOTE 1: EISREFSENS / EISminSENS depends on the channel bandwidth, see subclause 7.2 |  |  |  |

# Table 10.8.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| E-UTRA channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±360] | CW |
|  | [±1420] | 5MHz NR signal, 1 RB (NOTE 1) |
| 10 | [±325] | CW |
|  | [±1780] | 5MHz NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | [±380] | CW |
|  | [±1600] | 5MHz NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | [±345] | CW |
|  | [±1780] | 5MHz NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | [±325] | CW |
|  | [±1990] | 20MHz NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | [±320] | CW |
|  | [±1990] | 20MHz NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | [±310] | CW |
|  | [±2710] | 20MHz NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | [±330] | CW |
|  | [±3250] | 20MHz NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | [±350] | CW |
|  | [±3790] | 20MHz NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | [±400] | CW |
|  | [±4870] | 20MHz NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | [±390] | CW |
|  | [±4870] | 20MHz NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

### 10.8.3	Minimum requirement for BS type 2-O

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS antenna connector or BS antenna TAB connector, with the conditions specified in tables 10.8.3-1 and 10.8.3-2. All of the OTA test signals arrive from the save direction, and the requirement is valid if the signals arrive from any direction within the OTA REFSENS RoAoA. The reference measurement channel for the wanted signal is identified in [table 10.2.1-1] for each channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

[For a multi-band RIBs, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

Table 10.8.3-1: General intermodulation requirement

| Mean power of interfering signals [dBm] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- |
| EISREFSENS + 25dB | EISREFSENS + 6dB | See Table 10.8.3-2 |

Table 10.8.3-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | [±7.5] | CW |
|  | [±40] | 50MHz NR signal |
| 100 MHz | [±6.88] | CW |
|  | [±40] | 50MHz NR signal |
| 200 MHz | [±5.64] | CW |
|  | [±40] | 50MHz NR signal |
| 400 MHz | [±6.02] | CW |
|  | [±45] | 50MHz NR signal |