3GPP TSG-RAN4 NR AH 	R4-1800583

# San Diego, USA, 22-26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to frequency limits for unwanted emissions section 6.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During RAN4#84bis, an agreement was reached on the size of the transition between in band unwanted emissions and out of band spurious emissions, dependent on the width of the band. The transition size differs between the AAS BS types (1-H and 1-O) and the non-AAS type 1-C. In the current version of 38.104, this agreement is not captured properly because the transition is stated to be 40MHz for all bands above 100MHz in a table that is also applicable to 1-C. Furthermore, the table does not state that the agreement is applicable for FR1 only, which was clearly the intention of the agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Out of band emissions transition corrected to correspond to agreement (table inserted). Transition clarified to occur for FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrectly specified out of band emissions transition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 6.6	Unwanted emissions

### 6.6.1	General

Unwanted emissions consist of out-of-band emissions and spurious emissions according to ITU definitions [2]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The out-of-band emissions requirement for the BS transmitter is specified both in terms of Adjacent Channel Leakage power Ratio (ACLR) and operating band unwanted emissions (OBUE). The Operating band unwanted emissions define all unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

In FR1, the maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The value of ΔfOBUE is defined in table 6.6.1-1 for the NR operating bands, and may differ between BS type 1-C, 1-H and 1-O.

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band for FR1

| Operating band | ΔfOBUE / BS type |  |  |
| --- | --- | --- | --- |
|  | 1-C | 1-H | 1-O |
| Bands with FDL_high – FDL_low < 100 MHz |  | 10 MHz |  | 10MHz |  | 10MHz |  |
| Bands with 100Mz ≥ FDL_high – FDL_low < 200 MHz |  | 10MHz |  | 40MHz |  | 40MHz |  |
| Bands with FDL_high – FDL_low ≥ 200 MHz |  | 40 MHz |  | 40MHz |  | 40MHz |  |

[The BS type 1-H unwanted emission level limit of a TAB connector TX min cell group is in general defined by the unwanted emission basic limit which is the same as the corresponding applicable BS type 1-C per transmitter requirement and its scaling by NTXU,countedpercell, in certain regions.] The unwanted emission requirements are applied per the TAB connector TX min cell groups for all the configurations supported by the BS. The basic limits and corresponding scaling are defined in each relevant subclause.

There is in addition a requirement for occupied bandwidth.