3GPP TSG-RAN4 NR AH 	R4-1800584

# San Diego, USA, 22-26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to OTA reference sensitivity section 10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the OTA reference sensitivity, sensitivity should be specified as an EIS and not Prefsens as for the conducted requirement. FRCs for FR1 25 and 30MHz incorrectly related to 5MHz FRC, not 20MHz FRC. Also, numerical values for the sensitivity are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference sensitivity updated to be EIS and values are added. 25 and 30MHz moved to 20Mz FRC. The values for FR2 sensitivity have been rounded to the nearest integer to preserve the granularity tentatively agreed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect terminology for OTA reference sensitivity. No numerical requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

### 10.3.2 	Minimum requirement for BS type 1-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-1: Wide Area BS OTA reference sensitivity levels

| NR Channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity EIS level, EISEEFSENSR [dBm] |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G- FR1-A1-1 | [-101.1]- ΔOTAREFSENS |
| 5, 10, 15 | 30 | G- FR1-A1-2 | [-101.3] - ΔOTAREFSENS |
| 10, 15 | 60 | G- FR1-A1-3 | [-98.3] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | [-94.7] - ΔOTAREFSENS |
| 20, 25, 30,40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | [-95.0] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | [-95.0] - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full channel bandwidth. |  |  |  |

Table 10.3.2-2: Medium Range BS OTA reference sensitivity levels

| NR Channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity EIS level, EISREFSENS [dBm] |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G- FR1-A1-1 | [-96.1]- ΔOTAREFSENS |
| 5, 10, 15 | 30 | G- FR1-A1-2 | [-96.3] - ΔOTAREFSENS |
| 10, 15 | 60 | G- FR1-A1-3 | [-93.3] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | [-89.7] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | [-90] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | [-90] - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full channel bandwidth. |  |  |  |

Table 10.3.2-3: Local Area BS OTA reference sensitivity levels

| NR Channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | Reference sensitivity EIS level, EISRFSENS [dBm] |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G- FR1-A1-1 | [-93.1] - ΔOTAREFSENS |
| 5, 10, 15 | 30 | G- FR1-A1-2 | [-93.3] - ΔOTAREFSENS |
| 10, 15 | 60 | G- FR1-A1-3 | [-90.3] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | [-86.7] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | [-87.0] - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | [-87.0] - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full channel bandwidth. |  |  |  |

### 10.3.3	Minimum requirement for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and annex A when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

For wide area BS, the OTA reference sensitivity level for FR2 is an [integer] value in the range [-95] to [-115] dBm. The specific value is declared by the vendor.

For medium range BS, the OTA reference sensitivity level for FR2 is an [integer] value in the range [-90] to [-100] dBm. The specific value is declared by the vendor.

For local area area BS, the OTA reference sensitivity level for FR2 is an [integer] value in the range [-87] to [-97] dBm. The specific value is declared by the vendor.