3GPP TSG-RAN WG4 NR AH	R4-1800589
San Diego, USA, 22 -26 January, 2018

Source:	Ericsson

Title:	pCR to TR 38.817-2: Corrections to frequency limits for conducted unwanted emissions

Agenda item:	4.4.1.1

Document for:	Approval

# 1. Background

During RAN4#84bis, an agreement was reached on the size of the transition between in band unwanted emissions and out of band spurious emissions, dependent on the width of the band [1]. The transition size differs between the AAS BS types (1-H and 1-O) and the non-AAS type 1-C:

|  |  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 36.104/141 (non-AAS type) | 37.105/145-1/145-2 (AAS type) | 38.104/141 (non-AAS type(1-C)) | 38.104/141 (AAS type(1-H/1-O)) |
| ΔfUEM | Operating band <100MHz | 10MHz | 10MHz | 10MHz | 10MHz |
|  | 100MHz≦Operating band ≦200MHz | 10MHz | 40MHz | 10MHz | 40 MHz |
|  | 200MHz<Operating band ≦900MHz |  |  | 40MHz | 40MHz |
|  | 900MHz<Operating band |  |  |  |  |

In the current version of 38.817, this agreement is not captured properly for the conducted section because the transition is stated to be 40MHz for all bands above 100MHz in a table that is also applicable to 1-C.

This contribution proposes a correction in order to align the NR specification with the agreements in [1]. Furthermore, it is proposed to remove the sentence implying that there is some difference to LTE, since the agreement covers both LTE and NR and the eAAS specification will be updated accordingly.

[1] NTT DoCoMo, Nokia, Ericsson, Huawei, ZTE, “WF on in-band and out-of-band boundary”, R4-1711779, RAN4#84bis, October 2017

# 2. Proposal

It is proposed that the attached correction TP is adopted

TEXT PROPOSAL:

### 6.4	Operating band unwanted emissions

#### 6.6.4.1	General

A set of band-centric masks are defined for NR in Frequency Range 1, based on the same principles as for LTE. Details of the masks are as follows:

-	The UEM should be defined up to an offset ΔfUEM from the band edge, where ΔfUEM =40 MHz for NR bands equal to and wider than 100 MHz for AAS BS type 1-H and type 1-O, ΔfUEM =40 MHz for NR bands equal to and wider than 200 MHz for non-AAS BS type 1-C or ΔfUEM =10 MHz in all other cases. This serves as a baseline for the boundary between UEM and spurious emission for both Category A and Category B spurious emission limits.

-	The emission levels for the UEM will be the same as for LTE (5 MHz and larger channel BW) in the full frequency range (FDL_low– ΔfUEM) ~ (FDL_high+ΔfUEM).

-	The same UEM tables will be used for the different ΔfUEM.  For Category B spurious emissions, the limits will be taken from LTE Category B Option 1.

-	The following variations for the Unwanted Emissions Mask (UEM) tables should be used for sub-6 GHz:

–	BS classes

–	Category of emission (A/B)

–	Frequency range (above/below 1 GHz)

–	Power levels (for some BS classes)

The value of ΔfUEM determines the extent of the mask outside the operating band, but it also defines the spurious emissions requirements. It is a parameter that will vary between bands and is therefore best documented in a table, listing its value for different bands. Since it is common to spurious emissions and UEM requirements, the table should be placed in the general section of Unwanted emissions (6.6.1).

Specification of UEM relies on a few common parameters defining the frequency offset. These can largely be re-used from LTE:

-	Since the limits will be the same as for LTE, we also need to consider re-using the concept of two definitions of offset from the channel edge: Δf defining offset to the edge of the measurement filter and f_offset defining the offset to the centre of the filter. Both should be kept, since the difference (half the measurement bandwidth) is not negligible.

-	Keeping both Δf and f_offset, we also need to define two maximum values setting the maximum offset to which the UEM extends: fmax and f_offsetmax.

-

For Wide Area BS, the UEM level in the spurious domain is aligned with ITU-R recommendation SM.329. Consequently, there is a need to distinguish between Category A and B emission limits and also between frequencies below and above 1 GHz, since they have different limits and/or measurement bandwidths. For this reason, Category A and B should be in separate subclauses:

Wide Area BS, Category A (subclause)

Table: Category A (<1 GHz)

Table: Category A (1 GHz < f < 6 GHz)

Wide Area BS, Category B (subclause)

Table: Category B (<1 GHz)

Table: Category B (1 GHz < f < 6 GHz)

Medium Range BS (subclause)

Table: Category A & B

Local Area BS (subclause)

Table: Category A & B

Editor’s note: Operation in Non-contiguous spectrum or Multi-band BS are both integral part of the LTE specifications. If no additional aspect to such operation is discovered during the NR Work Item, it would be possible to re-use the LTE text regarding non-contiguous spectrum. Initially, text concerning these two features can be introduced in TS 38.104 in brackets and be for further study.

#### 6.6.4.2	Basic limits

Table with basic limits for NR BS in FR1.

#### 6.6.4.3	non-AAS conducted OBUE requirement

Minimum conducted requirement for 1-C type NR BS in FR1 is defined as basic limit, i.e. no scaling and no antenna considered.

#### 6.6.4.4	AAS conducted OBUE requirement

Minimum conducted requirement for 1-H type NR BS in FR1 is defined based on basic limit with the emissions scaling applied.