3GPP TSG-RAN WG4 Meeting #AH-NR-1801	R4-1800598

San Diego, California, USA, 22nd-26th January 2018

Source: 	Ericsson

Title:  	Draft pCR to TS 38.133 v15.0.0: Additional synch/synch requirement for NR DC

Agenda Item:	4.6.11.2

Document for:	Approval

# Background

Synchronous and asynchronous Dual connectivity requirements in terms of maximum receive timing difference (MRTD) and maximum transmission timing difference (MTTD) are specified in TS 38.133 [1] for Rel-15 LTE-NR combinations.

In a companion contribution, we have described some additional requirements that need to be defined to complete MRTD and MTTD requirements in Rel-15.

In this contribution, we propose a draft pCR for these additional requirements in 38.133.

# Proposal

We propose to adopt the text in this contribution to TR 38.133.

# References

TS 38.133 v15.0.0

R4-1800597, “Further discussions on synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations”, Ericsson

# Text proposal for TS 38.133

# <Start of draft pCR>

## 7.5	Maximum Transmission Timing Difference

### 7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated E-UTRA-NR dual connectivity.

### 7.5.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band E-UTRA-NR dual connectivity, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1.

Table 7.5.2-1: Maximum uplink transmission timing difference requirement for asynchronous operation

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For intra-band FDD-FDD E-UTRA-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.5.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [16].

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.5.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [16].

Table 7.5.2-2: Maximum uplink transmission timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |

For intra-band FDD-FDD E-UTRA-NR dual connectivity with collocated deployment, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [TS 38.331].

For intra-band TDD-TDD E-UTRA-NR dual connectivity with collocated deployment, only synchronous and collocated operation is allowed, thus no uplink transmission timing difference is applicable.

## 7.6	Maximum Receive Timing Difference

### 7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated for E-UTRA-NR dual connectivity.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of different carriers to be aggregated NR carrier aggregation.

### 7.6.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band E-UTRA-NR dual connectivity, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1 below.

# Table 7.6.2-1: Maximum receive timing difference requirement for asynchronous operation

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For intra-band FDD-FDD E-UTRA-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.6.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [16].

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell- E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.6.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [16].

Table 7.6.2-2: Maximum receive timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

For intra-band FDD-FDD E-UTRA-NR dual connectivity with collocated deployment, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [16].

For intra-band E-UTRA-NR dual connectivity with collocated deployment, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-3 provided the UE indicates that it is only capable of synchronous dual connectivity [16].

Table 7.6.2-3: Maximum receive timing difference requirement for synchronous operation in intra-band collocation scenario

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

### 7.6.3	Minimum Requirements for NR Carrier Aggregation

For intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.3-1 below.

Table 7.6.3-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| DL Sub-carrier spacing (kHz) | Maximum receive timing difference (µs) for FR1 | Maximum receive timing difference (µs) for FR2 |
| --- | --- | --- |
| 15 | TBD | N.A. |
| 30 | TBD | N.A. |
| 60 | TBD | TBD |
| 120 | N.A. | TBD |

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.3-2 below.

# Table 7.6.3-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| DL Sub-carrier spacing (kHz) | Maximum receive timing difference (µs) for FR1 | Maximum receive timing difference (µs) for FR2 |
| --- | --- | --- |
| 15 | TBD | N.A. |
| 30 | TBD | N.A. |
| 60 | TBD | TBD |
| 120 | N.A. | TBD |
| Note: For inter-band NR carrier aggregation between FR1 and FR2 Maximum receive timing difference is TBD µs. |  |  |

# <End of draft pCR>