3GPP TSG-RAN WG4-AH-1801 Meeting					 R4-1800652

San Diego, USA, 22nd January– 26th January, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS36.133 for NR PSCell Addition and Release Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-12-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CR R4-1714483 was agreed in RAN4#85 meeting with many TBD values. This CR replaces the TBD values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR replaces the TBD values of the agreed CR R4-1714483. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are TBD values for PSCell delay requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.X	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity

### 7.X.1	Introduction

This section defines requirements for the delay within which the UE shall be able to configure an NR PSCell in E-UTRA – NR dual connectivity. The requirements in this section apply for UE capable of E-UTRA-NR dual connectivity with PCell which can be FDD or TDD.

### 7.X.2	NR PSCell Addition Delay Requirement

The requirements in this section shall apply for the UE which is configured with PCell, and may also be configured with one or more SCells.

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n + Tconfig PSCell:

Where:

Tconfig_PSCell = TRRC_delay + Tactivation_time + TSFN_acquisition + TPCell_ DU + TPSCell_ DU

TRRC_delay is the RRC procedure delay as specified in [2].

Tactivation_time is the NR PSCell activation delay. If the NR PSCell is known, then Tactivation_time is [6 SMTC period]. If the NR PSCell is unknown, then Tactivation_time is [7 SMTC period] provided the NR PSCell can be successfully detected on the first attempt.

TPCell_ DU is the delay uncertainty due to PCell PRACH preamble transmission. TPCell_ DU is up to 20 ms if NR PSCell activation is interrupted by an PCell PRACH preamble transmission, otherwise it is 0.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU is up to [1 PRACH occasion period] as defined in [2].

TSFN_acquisition is the time used for SFN acquisition. If the SFN is unknown, TSFN_acquisition is [2 SMTC period] provided that the SFN can be successfully decoded on the first attempt. If the SFN is known, TSFN_acquisition is 0 ms.

In FR1, NR PSCell is known if it has been meeting the following conditions:

During the last [5] seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the NR PSCell being configured remains detectable according to the cell identification conditions specified in section Y.Y of [TS 38.133],

-	NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section Y.Y of [TS 38.133].

otherwise it is unknown.

The PCell interruption specified in section Z.Z is allowed only during the RRC reconfiguration procedure [2].

Editor’s note: Condition of known NR PSCell in FR2 is FFS;

Editor’s note: It is FFS whether SRS carrier based switching during NR PSCell addition procedure impacts NR PSCell addition delay requirement.

### 7.X.3	NR PSCell Release Delay Requirement

The requirements in this section shall apply for a UE which is configured with PCell and NR PSCell, and may also be configured with one or more SCells and/or NR SCells.

Upon receiving NR PSCell release in subframe n, the UE shall accomplish the release actions specified in [2] no later than in subframe n+ TRRC_delay:

Where

TRRC_delay is the RRC procedure delay as specified in [2].

The PCell interruption specified in section Z.Z is allowed only during the RRC reconfiguration procedure [2].

Editor’s note: It is FFS whether SRS carrier based switching during NR PSCell release procedure impacts NR PSCell release delay requirement.