3GPP TSG-RAN WG4 Meeting AH-1801							   R4-1800661

San Diego, USA, 22 – 26 January, 2018

Title: 	Discussion on CSI-RS measurement bandwidth

Source: 		Huawei, HiSilicon

Agenda item:		4.6.5

Document for:			Discussion

# Introduction

Definition of CSI-RS based measurement was discussed in RAN1 and RAN4. The following agreement was finally captured in RAN1 #89:

Agreements: - Following CSI-RS properties for RRM measurement for L3 mobility are supported in NR - Periodic CSI-RS transmission with configurable periodicity - Note that CSI-RS transmission can be turned on and off - FFS: candidate periodicity values - Configurable transmission/measurement bandwidth - FFS: whether/how transmission and/or measurement bandwidth is indicated to UE - Configurable CSI-RS time/frequency resource - Note that this property is the same with that for beam management based on CSI-RS - Note that the numerology for CSI-RS should consider neighbor cell measurement aspects

In this contribution we provide some initial considerations on this topic.

# Discussion on CSI–RS measurements

In NR, UE may need to evaluate multiple CSI-RS based measurements in one slot for variety proposes. Due to the different distance between UE and cells, the arrival timing of the measurement resources for each CSI-RS measurement may be different. It is very likely that the timing difference between the measurement resources from different target cells exceeds the length of CP, as depicted in fig.1. As a result, it is not possible to process multiple measurement resources simultaneously by single FFT. To improve the accuracy of CSI-RS measurements. It is desired to evaluate each measurement according to its timing. Then it will bring an impact on the hardware implementation of UE.

Figure 1. Demonstration of the timing difference between different CSI-RS measurement resources

## The parallel structure

There are two main ways to evaluate multiple CSI-RS measurements. First way is to handle each CSI-RS measurement by a separate FFT module, as depicted in fig. 2. One FFT module is allocated for each CSI-RS measurement. As a result, all measurements can be evaluated in parallel. The benefit of parallel processing is lower processing latency. However, the main drawback of the parallel structure is that multiple FFT modules consumes more hardware resources.

Figure 2 Demonstration of the parallel structure

Observation 1: Using multiple FFTs to handle CSI-RS measurements in parallel is hardware-consuming.

## The serial structure

The second way is to use a buffer to storage all sample values in a buffer and use a sliding FFT window to do all the measurements, as depicted in fig.2. UE uses a buffer to store all the sampling values. By using serial structure UE needs only one FFT module. However, if UE wants to store all samples in the buffer, the size of buffer may become a problem. On one hand, the samples stored for each measurement is directly determined by the measurement bandwidth of the CSI-RS measurement resources. On the other hand, buffer size is also related to the number of CSI-RS based measurements. If the buffer is too large, it will consume too many hardware resources, and if the cache is too small, some samples will be lost and UE will suffer from the performance degradation.

Figure 3 Demonstration of the serial structure

Observation 2: The size of buffer depends on both the measurement bandwidths of CSI-RS based measurements and the number of CSI-RS based measurements to evaluate in one slot.

# Proposals

From the implementation point of view, large measurement bandwidth will bring high sampling rate, it is a challenge to UE processing capacity no matter parallel structure or serial structure is applied. The configuration of the measurement bandwidth is specified in [2] as follows:

CSI-RS-ResourceConfig-Mobility ::= SEQUENCE { -- MO specific values csi-rs-MeasurementBW SEQUENCE { csi-rs-measurementBW-size ENUMERATED { size24, size48, size96, size192, size268}, csi-rs-measurement-BW-start ENUMERATED {ffsTypeAndValue}, associated-SSB ENUMERATED {ffsTypeAndValue}, qcled-SSB BOOLEAN, isServingCellMO BOOLEAN } -- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB subcarrierSpacing SubcarrierSpacing, -- List of resources csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility }

So the measurement bandwidth of CSI-RS in NR can be as high as 268 RBs. Furthermore, the SCS of CSI-RS can be up to 240 KHz in FR2. So the maximum measurement bandwidth of CSI-RS is 771.840MHz which is too big. So it is needed to limit the configurable measurement bandwidth values and the maximum number of CSI-RS based measurements to evaluate in one slot for each CSI-RS SCS.

Based on above discussion, we have the following proposals.

Proposal 1: The configurable measurement bandwidth values of CSI-RS measurement under each SCS value should be limited.

Proposal 2: The maximum number of CSI-RS measurements to be evaluate in one slot should be limited.

In our opinion, it makes sense to make the maximum measurement bandwidth of CSI-RS based measurement be comparable with the maximum measurement bandwidth of SSB based measurements. The maximum measurement bandwidth of SSB based measurement UE needs to support is 57.6MHz (20RB, 240KHz), it will be no big problem to support a measurement bandwidth of 69.12MHz (24RB, 240KHz)。Use this value as a reference, we propose that:

Proposal 3: The configurable measurement bandwidth values of CSI-RS measurement under each SCS value are given by the following table.

| Subcarrier spacing of CSI-RS (in KHz) | measurement bandwidth values for CSI-RS based measurements (in RBs) |
| --- | --- |
| 15 | 24, 48, 96, 192, 268 |
| 30 | 24, 48, 96, 192 |
| 60 | 24, 48, 96 |
| 120 | 24, 48 |
| 240 | 24 |

Table 1. Configurable CSI-RS measurement bandwidth for each SCS

As mentioned in previous section, the buffer size is related to both bandwidth and the number of measurements to be evaluated in one slot. So the number of measurements to evaluate in one slot should be smaller for bigger bandwidth. We propose that

Proposal 4: For each CC, the maximum number of CSI-RS measurements to evaluate in one slot for is given by the following table.

| SCS (KHz) Bandwidth (RB) | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- |
| 24 | 4 | 4 | 4 | 2 | 1 |
| 48 | 4 | 4 | 2 | 1 | N/A |
| 96 | 4 | 2 | 1 | N/A | N/A |
| 192 | 2 | 1 | N/A | N/A | N/A |
| 268 | 1 | N/A | N/A | N/A | N/A |

Table 2. Maximum number of CSI-RS measurement in one slot for each CC

# Conclusion

In this contribution we provide our view on bandwidth of CSI-RS measurements in NR. After discussion, the following observations and conclusions are made:

Observation 1: Using multiple FFTs to handle CSI-RS measurements in parallel is hardware-consuming.

Observation 2: The size of buffer depends on both the measurement bandwidths of CSI-RS based measurements and the number of CSI-RS based measurements to evaluate in one slot.

Proposal 1: The configurable measurement bandwidth values of CSI-RS measurement under each SCS value should be limited.

Proposal 2: The maximum number of CSI-RS measurements to be evaluate in one slot should be limited.

Proposal 3: The configurable measurement bandwidth values of CSI-RS measurement under each SCS value are given by the following table.

| Subcarrier spacing of CSI-RS (in KHz) | measurement bandwidth values for CSI-RS based measurements (in RBs) |
| --- | --- |
| 15 | 24, 48, 96, 192, 268 |
| 30 | 24, 48, 96, 192 |
| 60 | 24, 48, 96 |
| 120 | 24, 48 |
| 240 | 24 |

Table 1. Configurable CSI-RS measurement bandwidth for each SCS

Proposal 4: For each CC, the maximum number of CSI-RS measurements to evaluate in one slot for is given by the following table.

| SCS (KHz) Bandwidth (RB) | 15 | 30 | 60 | 120 | 240 |
| --- | --- | --- | --- | --- | --- |
| 24 | 4 | 4 | 4 | 2 | 1 |
| 48 | 4 | 4 | 2 | 1 | N/A |
| 96 | 4 | 2 | 1 | N/A | N/A |
| 192 | 2 | 1 | N/A | N/A | N/A |
| 268 | 1 | N/A | N/A | N/A | N/A |

Table 2. Maximum number of CSI-RS measurement in one slot for each CC

# Reference

[1] RAN1 #89 Chairmen Report.

[2] 38.331