3GPP TSG-RAN4 Meeting AH-1801	R4-1800710

# San Diego, CA, United States, 22 – 26 January 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 introducing number of carriers in SA Connected mode. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce UE requirements for number of carriers in SA Connected mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE requirements for number of carriers in SA Connected mode added in section 9.1.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for number of carriers in SA Connected mode is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

##### 9.1.3.2	Maximum allowed layers for multiple monitoring

9.1.3.2.1	Maximum allowed layers for multiple monitoring in NSA

If a UE is configured with E-UTRA-NR dual connectivity operation, the UE shall be capable of monitoring at least:

-	Depending on UE capability, [7] NR inter-frequency carriers configured by PScell, and

-	Depending on UE capability, [7] NR inter-RAT carriers configured by E-UTRA PCell [15], and

-	Depending on UE capability, [6] E-UTRA TDD inter-frequency carriers configured by E-UTRA PCell [15], and

-	Depending on UE capability, [6] E-UTRA FDD inter-frequency carriers configured by E-UTRA PCell [15],

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [13] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD and E-UTRA TDD layers. The UE shall be capable of monitoring a total of at least [7] effective NR carrier frequency layers configured by E-UTRA PCell and/or PScell.

Note: The E-UTRA-NR dual connectivity capable UE configured with PSCell shall fulfil the requirements defined in only one of Section 9.1.3.2 and Section 8.2.1.1b.1 of [15].

9.1.3.2.2	Maximum allowed layers for multiple monitoring in SA

The UE shall be capable of monitoring at least:

-	[7] NR inter-frequency carriers, and

-	Depending on UE capability, [6] E-UTRA TDD carriers, and

-	Depending on UE capability, [6] E-UTRA FDD carriers,

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [13] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD and E-UTRA TDD layers.

<< End of changes>>