3GPP TSG-RAN WG4 AH-1801	R4-1800727

San Diego, USA, 22 – 26 January 2018

Source:	Ericsson

Title:	TP to TR 37.863-02-01: TP for DC_7C_n78

Agenda item:	4.1.2.3

Document for:	Approval

# 1. Introduction

This is a text proposal for 37.863-02-01 for DC_7C_n78.

# 2. Text Proposal for TR 37.863-02-01

##### ---Start of changes---

## 6.x		DC_7C_n78A

## 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7C_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  | 140 |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Tables 6.x.3-1 lists Band 7C + Band n78A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.x.3-1: Band 7 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

Based on Table 6.x.3-1;

- There was no harmonic issues at legacy E-UTRA bands and NR new band 3.3~4.2GHz and 4.4~5GHz

- 2nd order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B67, B68 and TDD B44, B46 and B47.

- 3rd order IMD may also fall into B3, B11, B21, B24, B32 and TDD B45, n77

- 4th order IMD may also fall into B1, B2, B3, B4, B9, B10, B11, B21, B23, B24, B25, B30, B32, B65, B66, TDD B33, B34, B35, B36, B37, B38, B39, B40, B41, B43, B45, n77, n78, n79

- 5th order IMD may also fall into B5, B8, B12, B13, B14, B17, B18, B19, B20, B26, B27, B28, B29, B31, B67, B68, TDD B44, B46, B47 and n79.

The 4th IMD will be impact to the own received band n78. However, the NR TDD band is not any impacts from dual uplink transmission since there was no simultaneous Tx/Rx transmission in the n78 NR band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.3-2: 2UL Band 7 +Band n78 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd & 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd & 5th IMD |

Table 6.x.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.x.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7C_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1,2,3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1,2,3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1,2 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 1,3,4 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 1,3,4 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 1,4 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range [TBD – 3792.5] MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range [TBD - 3790] MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to [TBD] RB. |  |  |  |  |  |  |  |

##### ---End of changes---