3GPP TSG-RAN4 Meeting AH-1801	R4-1800735

# San Diego, CA, United States, 22 – 26 January 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 36.133: Updating UE measurement capability with remaining inter-RATs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | GSM and UMTS frequency layer monitoring is not added in UE capability section, which is not aligned with RAN LS (RP-172823). |  |  |  |  |  |  |  |  |
| Summary of change: |  | GSM, UMTS, cdma2000 and HRPD frequency layers are added on RATs to be monitored when UE is operating in NSA. Changes are made on top of changes to 8.1.2.1.1b in CR R4-1713934, which was approved in RAN4#85. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not aligned with RAN guidance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)

The requirements in this section are applicable for E-UTRA-NR dual connectivity capable UE, which is configured with NR PSCell.

When monitoring of multiple inter-frequency E-UTRAN and inter-RAT NR, GSM, UMTS, cdma2000 and HRPD carriers as configured by PCell, and inter-frequency NR carriers as configured by NR PSCell using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (RSRP, RSRQ, RS-SINR, NR SS-RSRP, NR SS-RSRQ, NR SS-SINR, UTRAN TDD P-CCPCH RSCP, UTRAN FDD CPICH  measurements, GSM carrier RSSI etc.) of detected cells on all the layers.

The effective total number of frequencies excluding the frequencies of the PCell, SCells, NR PSCell and NR SCells being monitored is Nfreq, NSA, which is defined as:

Nfreq, NSA = Nfreq_NSA_E-UTRA_inter-freq + Nfreq_NSA_NR + Nfreq_NSA_UTRA + MNSA_gsm + Nfreq_NSA_cdma2000 + Nfreq_NSA_HRPD

where

Nfreq_NSA_E-UTRA_inter-freq is the number of E-UTRA (FDD and TDD) inter-frequency carriers being monitored as configured by PCell

Nfreq_NSA_NR ≤ Nfreq_NSA_NR_inter-RAT + Nfreq_NSA_NR_inter-freq

where

Nfreq_NSA_NR_inter-RAT is the number of NR inter-RAT carriers being monitored as configured by PCell

Nfreq_NSA_NR_inter-freq is the number of NR inter-frequency carriers being monitored as configured by NR PSCell [TS 38.133]

Nfreq_NSA_UTRA is the number of UTRA carriers being monitored (FDD and TDD)

MNSA_GSM is an integer which is a function of the number of GSM carriers on which measurements are being performed. MGSM is equal to 0 if no GSM carrier is being monitored. For a MGRP of 40 ms, MGSM is equal to 1 if cells on up to 32 GSM carriers are being measured. For a MGRP of 80 ms, MGSM is equal to ceil(Ncarriers,GSM /20) where Ncarriers,GSM is the number of GSM carriers on which cells are being measured.

Nfreq_NSA_cdma2000 is the number of cdma2000 1x carriers being monitored.

Nfreq_NSA_HRPD is the number of HRPD carriers being monitored.

8.1.2.1.1b.1 Maximum allowed layers for multiple monitoring for UE in NSA operation

The UE configured with NR PSCell shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, [6] FDD E-UTRA inter-frequency carriers configured by PCell, and

-	Depending on UE capability, [6] TDD E-UTRA inter-frequency carriers configured by PCell, and

-	Depending on UE capability, [7] NR inter-RAT carriers configured by PCell, and

-	Depending on UE capability, [7] NR inter-frequency carriers configured by NR PSCell [TS 38.133].

-	Depending on UE capability, [TBD] FDD UTRA carriers configured by PCell, and

-	Depending on UE capability, [TBD] TDD UTRA carriers configured by PCell, and

-    Depending on UE capability, [TBD] GSM carriers (one GSM layer corresponds to 32 carriers) configured by PCell, and

-	Depending on UE capability, [TBD] cdma2000 1x carriers configured by PCell, and

-	Depending on UE capability, [TBD] HRPD carriers configured by PCell.

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [13] effective carrier frequency layers comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD, NR, UTRA FDD, UTRA TDD, GSM (one GSM layer corresponds to 32 carriers), cdma2000 1x and HRPD layers.

The UE shall be capable of monitoring a total of at least [7] effective NR carrier frequency layers configured by PCell and NR PSCell.

Note: The E-UTRA-NR dual connectivity capable UE configured with NR PSCell shall fulfil the requirements defined in only one of Section 8.2.1.1b.1 and Section 9.1.3.2 of [TS 38.133].