3GPP TSG-RAN4 Meeting AH-1801	R4-1800739

# San Diego, CA, United States, 22 – 26 January 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.133 on Network-indicated measurements on deactivated SCells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce network indicated measurements on deactivated NR SCells instead of UE autonomous interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section is added to describe how to handle measurements on deactivated NR SCells and interruptions related to these. The requirements are common for NSA and SA operation in this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

## 9.X Network-indicated measurements on deactivated SCCs

### 9.X.1	Introduction

This section contains requirements for measurements on deactivated SCells on network-indicated measurement occasions.

The requirements are applicable for a UE which has indicated to the network a need for interruptions during measurements on deactivated SCells.

### 9.X.2	Requirements

### A UE which causes interruptions during measurements on deactivated SCells shall indicate this to the network.

If indicated by the network, the UE shall use the indicated SMTCs for measurements on the deactivated SCells. The UE is not allowed to cause autonomous interruptions due to measurements on the deactivated SCells. The UE is only allowed:

For E-UTRA PCell and E-UTRA SCell

- an interruption of 1 subframe before and 1 subframe after the network indicated measurement specific SMTC on the deactivated SCell

For PCell, PSCell and SCell

- an interruption as defined in Table 9.X.2-1 before and after the network indicated measurement specific SMTC on the deactivated SCell.

Table 9.X.2-1: Interruption duration for PSCell and SCell in SCG.

| SCS (kHz) | Interruption duration (slots) |
| --- | --- |
| 15 | 1 |
| 30 | 1 |
| 60 | 1 |
| 120 | 2 |
| 240 | 2 |

Editor’s note: Measurement requirements for deactivated SCell shall be defined in this section once intra-frequency measurement requirements are finalized.

<< End of changes>>