3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800874

San Diego, US, January 22-26, 2018

Agenda Item:	4.2.2.3

Source:	Huawei, HiSilicon

Title:	Examining the mapping of SS blocks to the GSCN

Document for:	Approval

# Introduction

During RAN4#85 meeting, many agreements were made on the GSCN. This document identifies some possible misalignment between the agreements and the specifications.

# Discussion

In section 5.4.3.1 of [1], there is a statement regarding the frequency ordering of GSCN. Specifically,

The GSCN are numbered in increasing frequency order

There appears to be an inconsistency with that statement and with Table 5.4.3.1-1 with regards the values of the range of GSCN and frequency. For example, the value of 8832 corresponds to 2649.605 MHz. However, the value 8833 corresponds to 2400 MHz. The GSCN is not ordered according to frequency. Some clarification is needed.

- The statement “The GSCN are numbered in increasing frequency order” should only apply to the frequency range 0-2650 MHz. (most likely)

- The statement could apply if the frequency ranges do not overlap (meaning the ranges need to be change)

Proposal 1: Statement “The GSCN are numbered in increasing frequency order” in section 5.4.3.1 needs further clarification.

For the remainder of the contribution, it is assumed the entries in Table 5.4.3.1-1 are correct.

In the LS [2], the GSCN indicates

- Sync raster will indicate the position of RE=#0(subcarrier #0) of RB#10 of the SS block

From this definition, the GSCN should be inside the band. From the values listed in Table 5.4.3.3-1, the lower value for many range entries aligns with the band edge.

Consider band n1 that spans 2110 to 2170 MHz on the DL. Since 15 kHz SCS is used for the SS block, the span for 10 RBs is 15 kHz/RE  12 (REs/RB)  10 RBs equals 1.8 MHz. From this, the GSCN should indicate frequencies from 2110+1.8 to 2170-1.8 MHz (=2111.8 to 2168.2 MHz). Using the formula in Table 5.4.3.1-1, the computed values of GSCN are listed below

Table 1. Band n1

| GSCN | Frequency, MHz | Comment |
| --- | --- | --- |
| 7033 | 2110.495 | Current value; corresponds to band edge |
| 7039 | 2112.295 | Lowest value of GSCN that is 1.8 MHz greater than band edge |
| 7224 | 2167.205 | Current value |
| 7227 | 2168.105 | Largest value of GSCN that is 1.8 MHz less than band edge |
| 7233 | 2169.905 | Corresponds to band edge |

Table 5.4.3.3-1 lists the GSCN as “7033 – <1> – 7224” which would correspond to 2110.495 to 2167.205. The value of 7033 is the lower band edge of band n1. Based on the agreement, the center of the SS block should not be at the band edge.

Proposal 2: A set of revised values is proposed for Table 5.4.3.3-1 which accounts for the offset from the band edge.

The GSCN for band n7 is inconsistent. The current value has the lowest GSCN using the formula from the first row of for Table 5.4.3.1-1 but the highest GSCN is based on the second row. Because the frequency ranges in Table 5.4.3.1-1 overlap, there is ambiguity in the GSCN and frequency values. The decision on the frequency solution for band n7 is needed and the GSCN should be modified accordingly. Several options are possible:

- Expand the frequency span in the first row of Table 5.4.3.1-1 to cover band n7

- Ensure the frequency ranges for GSCN do not overlap in Table 5.4.3.1-1

- Use middle row of Table 5.4.3.1-1 for band n7

Proposal 3: the GSCN for band n7 should be revised to avoid ambiguity in frequency ranges

============================== Start of text ===================================

# Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1)

| NR Operating Band | SS Block SCS | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- |
| n1 | 15kHz | 7039 – <1> – 7227 |
| n2 | 15kHz | 6439 – <1> – 6627 |
| n3 | 15kHz | 6022 – <1> – 6258 |
| n5 | 15kHz | 2902 – <1> – 2973 |
|  | 30kHz | 2908 – <1> – 2967 |
| n7 | 15kHz | 8740– <1> – |
| n8 | 15kHz | 3088 – <1> – 3192 |
| n20 | 15kHz | 2641 – <1> – 2730 |
| n28 | 15kHz | 2533 – <1> – 2670 |
| n38 | 15kHz | 8572 – <1> – 8727 |
| n41 | 15kHz | 8901 – <1> – 9033 |
| n50 | 15kHz | 4780 – <1> – 5049 |
| n51 | 15kHz | 4762 – <1> – 4767 |
| n66 | 15kHz | 7039 – <1> – 7326 |
|  | 30kHz | 7045 – <1> – 7320 |
| n70 | 15kHz | 6655 – <1> – 6726 |
| n71 | 15kHz | 2062 – <1> – 2166 |
| n74 | 15kHz | 4921 – <1> – 5052 |
| n75 | 15kHz | 4780 – <1> – 5049 |
| n76 | 15kHz | 4762 – <1> – 4764 |
| n77 | 30kHz | 9460 – <1> – 10081 |
| n78 | 30kHz | 9460 – <1> – 9803 |
| n79 | 30kHz | 10224 – <TBD> – 10637 |

# Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR2)

| NR Operating Band | SS Block SCS | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- |
| n258 | 120 kHz | 24008 – <1> – 24194 |
| n257 | 120 kHz | 24139 – <1> – 24309 |
|  | 240 kHz | 24139 – <2> – 24309 |
| n260 | 120 kHz | 24746 – <1> – 24917 |
|  | 240 kHz | 24746 – <2> – 24917 |

============================== End of text ===================================

# Conclusions

This contribution examined some possible misalignment between the agreements and the specifications in the GSCN sections of 38.104.

Proposal 1: Statement “The GSCN are numbered in increasing frequency order” in section 5.4.3.1 needs clarification.

Proposal 2: A set of revised values is proposed for Table 5.4.3.3-1 which accounts for the offset from the band edge.

Proposal 3: the GSCN for band n7 should be revised to avoid ambiguity in frequency ranges

# References

TS38.104, V1.0, Dec. 2017.

R4-1714295, “LS on channel raster and SS raster”, Huawei, RAN4#85, Nov. 27 - Dec. 1, 2017.