3GPP TSG-RAN WG4 Meeting RAN4-AH-1801                                                  R4-1800888

San Diego, California, USA 22 - 26 January 2018

Source:	Sony, Ericsson

Title:	UE Spherical coverage at mmWave 28GHz

Agenda item:	4.3.2.1.4

Document for:	Discussion

# Background

The impact on spherical coverage of a realistic design has been discussed for several meetings. Detailed simulation assumptions were given in a WF on spherical coverage [2] and a WF on power class was agreed in [1].

In this discussion paper we use the assumption 3 and 4 [2]. Assumption 3 and 4 stipulates top + bottom antenna modules and full (assumption 3) versus partial full (assumption 4) display. For assumption 3 one of the antenna arrays is mounted on the edge covered by plastic (i.e. no metal frame covering the antenna).

# Antenna environmental simulation

We have studied two antenna configurations:

- Assumption 4 [2]: 4x1 linear patch arrays, bottom-back + top-front with partial full display (10mm clearance)

- Assumption 3 [2]: 4x1 linear dipole array mounted in the top-edge covered by plastic (1.5mm clearance between display and back) + 4x1 linear patch array mounted in the bottom-back. Full display on front.

## Top + Bottom Patch Arrays

4x1 linear patch arrays in top-front and bottom-back was simulated with display and glass back cover and with display and plastic back cover. The display was semi full with 10 mm clearance in the top. The top-front antenna is covered by glass in both cases. As a reference a simulation without either display or back cover was also performed.

Simplified layout and respectively total scan pattern is shown in Figure 1 and the spherical coverage as CDF curve is plotted in Figure 2. The result from this simplified implementation shows a gain difference between peak EIRP and 20%-tile of 9dB. The gain difference between peak EIRP and 50%-tile is 6dB. Note that the referred peak EIRP value comes from the ideal simulation result.

Figure 1. Schematic picture of simulation assumptions and total scan pattern, top + bottom patch arrays

Figure 2. CDF curve of spherical coverage, top + bottom patch arrays

## Top Dipole Array + Bottom Patch Array

A 4x1 linear dipole array on the top-edge and a 4x1 linear patch array on bottom-back was simulated with display and glass back cover and with display and plastic back cover. In this simulation the display was covering the full front. A plastic frame is assumed, i.e. the edge (frame) mounted dipole array is covered by plastic. As a reference also a simulation without either display or back cover was performed.

The simplified layout and corresponding total scan pattern is shown in Figure 1 and the spherical coverage plotted as a CDF curve is shown in Figure 2. For this simplified implementation the gain difference between peak EIRP and 20%-tile is 10dB. The gain difference between peak EIRP and 50%-tile is 6dB. Note that the referred peak EIRP value comes from the ideal simulation result.

Figure 3. Schematic picture of simulation assumptions and total scan pattern, top dipole array + bottom patch array

Figure 4. CDF curve of spherical coverage, top + bottom patch arrays

# Impact of cover material

From this simplified simulation a difference between peak EIRP and the 20%-tile is in the order of 10dB. Measurements on real prototypes is needed to verify the results.

Observation 1. The difference between peak EIRP and the 20%-tile is in the order of 10dB.

# Conclusion

The impact of material in the back cover has been studied. A simplified two-antenna-panel layout has been simulated in accordance to the WF from the RENO meeting [2]. The following observation is made:

Observation 1. The difference between peak EIRP and the 20%-tile is in the order of 10dB.

Measurements on real prototypes is needed to verify the results.

# References

- R4-1714445	“WF on pi/2 BPSK spectrum shaping and power class in FR2”, Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia , Huawei, Motorola Mobility

- R4-1714355	“WF on spherical coverage in FR2”, Samsung, Apple, LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek