3GPP TSG-RAN WG4 Meeting RAN4-AH-1801                                                  R4-1800889

San Diego, California, USA 22 - 26 January 2018

Source:	Sony, Ericsson

Title:	Impact of UE back cover material on peak EIRP at mmWave

Agenda item:	4.3.2.1.3

Document for:	Discussion

# Background

The impact on spherical coverage of a realistic design has been discussed for several meetings. Detailed simulation assumptions were given in a WF on spherical coverage [2]. Another WF on power class was agreed in [1].

In this discussion paper we investigate the assumption 2 of the WF [2] with glass respectively plastic back cover.

Assumption 2 stipulates one antenna module. In our understanding one antenna module will not give the expected spherical performance, however, the aim with this document is to study the impact of cover material on the peak EIRP and therefore the simpler simulation model is chosen.

# Antenna environmental simulation

We have studied two antenna types:

- 4x1 linear patch array mounted in the back

- 4x1 linear dipole array, edge mounted

## Patch Antenna

A linear 4x1 patch array was simulated with display and glass back cover, with display and plastic back cover and without either display or back cover (Figure 1).

Total scan pattern for respectively assumption is shown in Figure 1. A horizontal plane total scan pattern is shown in Figure 2 and numerical result is shown in Table 1. It can be seen from Figure 1 and Figure 2 that the casing act as a lens and will scatter the signal compared to no casing. The glass will scatter more than plastic. This means that some directions could be almost unaffected or even amplified but other direction are heavily attenuated compared to the ideal simulation without casing material. In general there is some attenuation due to the cover. I.E TRP is affected about 1dB for 28GHz. For 39GHz the general attenuation is expected to be less than 1.5dB.

Figure 1. Schematic picture of simulation assumptions and total scan pattern, patch antennas.

Figure 2. Total scan pattern of linear patch antenna array

| Antenna gain | -45 beam | 0 beam | 45 beam |
| --- | --- | --- | --- |
| No cover, no display | 10.1 dBi | 11.9 dBi | 8.3 dBi |
| Display + plastic cover | 8.0 dBi | 12.1 dBi | 9.8 dBi |
| Display + glass cover | 3.5 dBi | 14.1 dBi | 5.5 dBi |

Table 1. Comparison with and w/o casing for three different beam directions with patch antennas.

## Dipole Antenna

A similar simulation as was done for the patch antennas was done for the dipole antenna array. The clearance between display and back cover was 1.5mm according to [2]. The number of beams was 7, also in accordance with [2]. Total scan pattern for respectively assumption is shown in Figure 3. Total scan pattern on horizontal plane is shown in Figure 4. From the figures below it can be seen that the dipole antenna array is less prone to scattering due to the back cover material but there is still some attenuation in particular for the more deflected beams. Note that the antenna array is mounted on the edge covered by plastic (i.e. no metal frame covering the antenna).

Figure 3. Schematic picture of simulation assumptions and total scan pattern, dipole antennas.

Figure 4. Total scan pattern of linear dipole antenna array

| Antenna gain | -45 beam | 0 beam | 45 beam |
| --- | --- | --- | --- |
| No cover, no display | 9.2 dBi | 7.3 dBi | 8.7 dBi |
| Display + plastic cover | 6.6 dBi | 7.1 dBi | 6.8 dBi |
| Display + glass cover | 6 dBi | 8.1 dBi | 6.3 dBi |

Table 2. Comparison with and w/o casing for three different beam directions with dipole antennas.

# Impact of cover material

It can be seen that the peak value is only affected marginally but the lower percentile could be degraded substantially.

Observation 1. Implementation loss due to cover material for peak EIRP calculation could be less than 1dB for 28GHz band and less than 1.5dB for 39GHz band.

# Conclusion

The impact of material in the back cover has been studied. It is observed that the material will scatter the signal i.e. the material act as a lens. The peak value is thus affected only marginally but the lower percentile could be degraded substantially. The general attenuation of the signal is less than 1dB for 28GHz and less than 1.5dB for 39GHz and those values could be used in the peak EIRP (power class) calculation.

Observation 1. Implementation loss due to cover material for peak EIRP calculation could be less than 1dB for 28GHz band and less than 1.5dB for 39GHz band.

# References

- R4-1714445	“WF on pi/2 BPSK spectrum shaping and power class in FR2”, Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia , Huawei, Motorola Mobility

- R4-1714355	“WF on spherical coverage in FR2”, Samsung, Apple, LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek