3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800895

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: relations between single core and separate conducted/OTA test requirements (4.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-12-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During previous RAN4#85 meeting, TP to TS 38.104 was proposed in R4-1712649 (TP to TS 38.104: relations between single core and conducted/OTA test requirements). This TP was proposed to address the fact that the relationship among the core and conformance requirements for NR BS will be not be the same as for the legacy BS specifications. Namely, for selected NR BS FR1 requirements, a single core requirements will be translated into two different test requirements, i.e. one for conducted conformance, and the second one for radiated conformance purposes. This is due to the fact, that Measurement Uncertainties for the conducted and radiated test setups are expected to differ. Furthermore, during previous meeting comments were received claiming that the proposed description of relations among single core requirement and the conducted/OTA requirements is not applicable to all requirements. Therefore in this CR appropriate corrections were incorporated to adddress the above comment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subclause 4.2: text update to address relations among single core requirement and two different test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-specific relations among the core requirement and related conducted/OTA test requirements will not be addressed in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Mirror TP to the TR 38.817-02 is provided in R4-1800894. |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 4.2	Relationship between minimum requirements and test requirements

Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.141-1 [5] for conducted testing and in 3GPP TS 38.141-2 [6] for radiated testing.

The minimum requirements given in this specification make no allowance for measurement uncertainty. The test specification 3GPP TS 38.141-1 [5] defines conducted test tolerances and test specification 3GPP TS 38.141-2 [6] defines radiated test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

NOTE:	Selected radiated FR1 requirements are based on the same limits as defined for conducted requirements. Measurement uncertainty values for the conducted test setup and OTA test setup may differ for those requirements due to different test setups. Therefore in case of selected FR1 requirements, a single limit used for both conducted and radiated requirements may result in two different test requirements for conducted and OTA testing, as captured in 3GPP TS 38.141-1 [5] and 3GPP TS 38.141-1 [6].

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in recommendation ITU-R M.1545 [7].

------------------------------ End of modified sections ------------------------------