3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800905

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: correction of the conducted Rx out-of-band blocking requirement (7.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WF agreements on the inband /OOB boundaries are hardly readable when appied in text. Therefore it is proposed to use ΔfOOB offset values, and to describe the conducted OOB blocking requirement based on parametrized approach in the text. This is aligned with the in-band blocking requirement correction. Also correction of the unnecessary “basic limit” approach is proposed, i.e. Based on the AAS discusions, the “basic limit” concept was supposed to be applied only to the requirements, which are subject to the emisisons scaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction of the unnecessary “basic limit” approach - Apply the parametrized description for blocking requirement based on the ΔfOOB offset - Merge the 1-C and 1-H minimum requirements, as those have the same text thanks to the ΔfOOB offset parametrization - Wording alignment with other blocking requirements - Reference corrections, editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification of the blocking requirement for various NR operating bands; wrong references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2, 7.5.3, 7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 7.5	Out-of-band blocking

### 7.5.1	General

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.5.2	Minimum requirement for BS type 1-C and BS type 1-H

Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to BS antenna input using the parameters in table 7.5.2-1. The reference measurement channel for the wanted signal is identified in subclause 7.2.2 for each channel bandwidth and further specified in annex A.

For BS type 1-C and BS type 1-H the out-of-band blocking requirement apply from FUL_low - ΔfOOB to FUL_high + ΔfOOB, including the downlink frequency range of the operating band. ΔfOOB is the maximum offset of the out-of-band emissions mask from the operating band edge and is defined in table 7.4.2.2-1.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

[For a BS capable of multi-band operation, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to subclause 7.4.2.2 shall be excluded from the out-of-band blocking requirement.]

Table 7.5.2-1: Out-of-band blocking performance requirement for NR

| Interfering signal mean power [dBm] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- |
| -15 | PREFSENS +6 dB (Note) | CW carrier |
| NOTE: PREFSENS depends on the channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |

### 7.5.3	Void

### 7.5.4	Void

------------------------------ End of modified sections ------------------------------