3GPPRAN4#AH1801	R4-1800909

San Diego, US, Jan 22nd-26th

Title: 	Interruptions in NSA operation

Source: 	Qualcomm Incorporated

Agenda item:	4.6.13 - Interruption and related requirements (38.133/36.133) [NR_newRAT]

Document for:	Discussion

# Introduction

In Nagoya RAN4 meeting a WF was agreed on interruptions on NSA operation [1]. In this contribution, we provide a proposal on interruptions to be taken on PCell/SCell in MCG and PSCell/SCell in SCG. These interruptions would be included in 36.133 for E-UTRA and 38.133 for NR.

# Discussion

## Interruptions

In this contribution, we provide times on interruptions in various scenarios for both LTE and NR in NSA operation. We split the interruptions into two parts, RRC reconfiguration related interruptions (e.g.: SCell add/release), and transitions between active/non-active related interruptions (e.g.: SCell activation/deactivation). In both scenarios, we will need interruptions on both LTE and NR.

Proposal 1: For RRC reconfiguration following table shows the interruptions to be taken and where to capture them

| Affected Tech | Tech Triggering RRC Reconfiguration | Interruption Duration | Notes |
| --- | --- | --- | --- |
| LTE | LTE | 1ms for inter-band 5ms for intra-band | Captured in 36.133 |
| LTE | NR | 2ms for inter-band 5ms for intra-band | Async operation between NR and LTE Captured in 36.133 |
| NR | LTE | 1ms for inter-band 5ms for intra-band | Async operation between NR and LTE Captured in 38.133 |
| NR | NR | 1ms for inter-band 2ms for intra-band | Captured in 38.133 No MBSFN subframes in NR |

Proposal 2: For scenarios involving transitions between active and non-active states of any configured cell, the following table show the interruption duration that the UE allowed to take.

| Affected Tech | Tech Triggering RRC Reconfiguration | Interruption Duration | Notes |
| --- | --- | --- | --- |
| LTE | LTE | 1ms | Captured in 36.133 |
| LTE | NR | 1ms | Captured in 36.133 |
| NR | LTE | max (0.5ms,1 slot) | Captured in 38.133 |
| NR | NR | max (0.5ms, 1 slot) | Captured in 38.133 |

# Conclusions

In this contribution, we provide the allowable time for interruptions that the UE can take under various scenarios.

# References

[1] R4-1709901, “WF on interruption on NSA operation”, Huawei, HiSilicon