3GPP TSG-RAN WG4 Meeting #AH-1801	R4-1800912

San Diego, CA, USA, 22nd – 26th January 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On OTA methods for spurious measurements of NR BS type 2-O

Agenda item:	4.4.4

Document for:	Discussion

# 1	Introduction

At the RAN4 #85 meeting, we presented a simple OTA procedure [1] (which is based on the proposal in [2]) for measuring spurious emissions in the mmWave for NR base stations. The procedure has been revised according to the comments received during online and offline discussion, in particular, it has been further refined in view of reducing the TRP measurement duration to a reasonable time. The procedure can be used for testing spurious emissions ranging from 30 MHz to the second harmonic of the upper frequency edge of the downlink operating band in one single test setup, only the measurement antenna needs to be changed. The procedure is applicable to other test setup scenarios.

In [3], we have quantitatively shown that the TRP measurement duration of spurious emissions is in the order of months, which is attributed to a very wide range of spurious frequencies, the number of TRP measurement points and the measurement bandwidth.

This document outlines the details of the OTA method for spurious emission measurements.

# Discussion

Figure 1 shows one example of a typical system setup for measuring radiated spurious emissions in an indoor anechoic chamber.

Measurement Distance  r          Type equation here.   Type equation here.

Measurement Antenna

DUT

h2          Type equation here.   Type equation here.

Vertical movement

Styrofoam table

h3          Type equation here.   Type equation here.

h1          Type equation here.   Type equation here.

ϕ

Turntable

LNA

Spectrum Analyzer

Filter

Figure 1: An example of test setup

Using the test setup example above, the OTA method for measuring spurious emissions of NR BS operating in frequency range 2 consists of the following steps.

Calibration steps are not considered herein.

Configuration steps:

- Configure all the transmitters of the NR BS (i.e., DUT) to operate at maximum power such that the BS antenna array radiates maximum power for a single distinct beam.

- If the DUT cannot be configured as described in Step 1 because it can only support multiple beams, then set the DUT to generate several beams at the maximum transmit power levels utilizing all the NR BS transmitters.

- The DUT is placed at [h3 = 1.5 m] above the ground.

- RF filtering (e.g., a notch, low pass or high pass filter) is necessary to increase the dynamic range of measurement.

- Spurious emission power is measured from 30 MHz to the 2nd harmonic of the upper frequency edge of the downlink operating band and use a measurement bandwidth as specified in TS 38.104 (see Table 1). The operating band plus a guard band on either side of the band edge is excluded from the measurement.

Table 1: NR BS radiated Tx spurious emission limits in frequency range 2 (from TS 38.104)

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30MHz - 1GHz | [-13 dBm] | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1 NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. |  |  |  |

- Position the measurement antenna in the test chamber at a fixed distance r from the DUT, where r is in the far field region. However, due to the physical constraints of the test chamber and radiating sources of DUT, it is unlikely that r can satisfy the far field conditions for the entire range of the spurious frequency.

far field conditions  r ≫ 2D2λr ≫ D         r ≫ λ

where D is the largest dimension of the BS antenna array or measurement antenna and λ is the wavelength of the spurious frequency in the measurement. What an optimum value for r is FFS.

- All measured spurious emissions should be far field measurements. However, for the fixed distance r, the far field conditions will not be met for certain chunks of the spurious frequency range as pointed out in Step 6.

- Measurement are made for both polarizations of the DUT antenna.

- Typically, cables attached to the DUT may not radiate in the mmWave range. Therefore, for measuring spurious emissions, the measurement antenna’s 3-dB beamwidth need not cover the power cables.

Pre-scan steps:

- Pre-scan the entire range of the spurious frequencies by rotating the DUT on the turntable through 360° in the azimuth (ϕ) direction with a discrete step of [∆ϕ = 45°]. The total number of ∆ϕ-sectors J= 360°∆ϕ. For high spurious frequencies (e.g., above 18 GHz), the 3-dB beamwidth of the measurement antenna may not completely cover the DUT, then ∆ϕ should be reduced.

  - For each ∆ϕ-sector, measure the emission level by sweeping through the spurious frequency range using a spectrum analyser. In order to find the maximum absolute emission level, the measurement antenna is moved in the vertical direction from [h1 = 1 m] to [h2 = 4 m] in height above the reference ground. Record the strongest measured emission level for the spurious frequency and the corresponding measurement antenna height, sector number and antenna polarity in list L. As the range of spurious frequencies is wide, the choice of measurement antenna and the needed test setup RF filtering will depend on the frequency under measurement

  - From L in Step 10a, those spurious frequencies with the maximum absolute emission levels < ([-13 dBm] – [Y = 20 dB]) or corresponding to the noise floor do not need further measurements; these frequencies can be discarded from L. The purpose of this step is to eliminate the need to perform time-consuming TRP measurements on spurious frequencies with no useful emissions and to identify a list of spurious frequencies with significant emissions for further measurements in Step 11.

- Using the list L in Step 10b, sort the list in descending order of the emission levels; denote the sorted list as Ls.

  - Sweep from the spurious frequency with the highest recorded emissions to the lowest in Ls. Vary the measurement antenna height between h1 and h2 to search for the peak emission level based on the height and sector recorded in Step 10a

  - Measure the peak EIRP for co- and cross-polarization and calculate the root mean square EIRP in linear units

  - If the peak EIRP per spurious frequency ≤ [-13 dBm] then the spurious frequency is compliant, no further measurements are needed and the spurious frequency can be discarded from Ls

TRP measurement steps:

- If Ls ≠ empty then calculate TRP level by using a suitable sampling grid. There are a number of grids which can be used, each with their pros and cons. A comparison of the different TRP sampling grids in terms of measurement uncertainty and duration is FFS. The most basic one is the full-sphere uniform grid. Using this grid, the TRP is approximately given as follows:

TRP ≈ π2NM n=0N-1m=0M-1EIRPθ,ϕsinθn						(1)

where M and N are the number of points in the azimuth and elevation directions, respectively, which in turn determined by the grid resolution (see Annex A.2 in [3] for more details). N and M are defined as

N= π∆θ+1  and M= 2π∆ϕ										(2)

where Δθ = FFS and ∆ϕ = FFS. The total number of measurement points on the surface of the sphere is

T= N × M-2M-1 								                 (3)

- Set the measurement antenna to the first position (out of the T points) on the spherical surface

- Measure and record EIRP at the position by sweeping through the spurious frequencies in Ls using a spectrum analyser.

- Move the measurement antenna to the next measurement point

- Repeat Steps 12.b and 12c until all T points have been covered

- Calculate TRP using equation (1) for each spurious frequency; if TRP ≤ [-13 dBm] then the spurious frequency is compliant, else it is not compliant

# 3	Conclusions

In this document, a test method for measuring spurious emissions of NR BS type 2-O has been discussed and proposed. The proposed test method comprises three stages:

- Configuration

- Pre-scan

- TRP measurements

# References

- R4-1713796, Spurious emission measurement procedures for NR BS type 2-O, Nokia, Nokia Shanghai Bell

- Nokia’s contribution for equivalent isotropically radiated power (EIRP), total radiated power (TRP), and channel edge measurements, 47CFR30

- R4-1713793, On TRP measurement duration of NR base stations, Nokia, Nokia Shanghai Bell