TSG-RAN Working Group 4 (Radio) meeting AH-1801	R4-1800921

San Diego, US; 22 - 26 January, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On introduction of NR to MSR specifications 37.104/141

Agenda item:	4.4.1

Document for:	Approval

# Introduction

During RAN4#85, BS RF requirements for NR have been defined in 38.104. As the scope of NR Work Item includes the introduction of NR+LTE operation in MSR specifications, RAN4 needs to discuss and conclude on 37 series specification changes related to NR operation. This document discusses various aspects which should be considered before CRs to 37.104/141 are provided.

# Discussion

MSR specifications define currently fifteen different capability sets (see Table 1 with capability sets without NB-IoT standalone operation) including GSM, UTRA, E-UTRA, NB-IoT in-band, NB-IoT guard-band and NB-IoT stand-alone operation. Having in mind RAN4 has very limited time to conclude requirements in MSR specification, it is necessary to focus on very limited and essential capability set(s) in Rel’15. MSR specifications impact for NR and NR+LTE operation is included in NR WID, NB-IoT can be considered to be included in Rel’15 new capability set(s) taking into account NB-IoT operation is optional and declared by the manufacturer (as defined currently in Note 3, 4, and 5 in Table 1 below). Table 2 proposes two alternatives of capability set to be introduced in Rel’15 MSR specifications, it should be noted we do not see a need to introduce additional capability set for NR stand-alone operation as SR NR operation is already included in both proposed options.

# Table 1: Capability sets

| Capability Set supported by the BS | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | CS7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supported RATs | UTRA (MC) | E-UTRA (MC), NB-IoT in-band3, NB-IoT guard band4 | UTRA, E-UTRA, NB-IoT in-band3, NB-IoT guard band4 | GSM, UTRA | GSM, E-UTRA, NB-IoT in-band3, NB-IoT guard band4 | GSM, UTRA, E-UTRA | GSM, UTRA, E-UTRA, NB-IoT in-band3, NB-IoT guard band4 |
| Supported configurations | SR UTRA (SC, MC) | SR E-UTRA (SC, MC, CA) E-UTRA + NB-IoT in-band3 E-UTRA + NB-IoT guard band4 E-UTRA + NB-IoT in-band + NB-IoT guard band5 | MR UTRA + E-UTRA SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) E-UTRA + NB-IoT in-band3 E-UTRA + NB-IoT guard band4 MR UTRA + E-UTRA + NB-IoT in-band3 MR UTRA + E-UTRA + NB-IoT guard band4 E-UTRA + NB-IoT in-band + NB-IoT guard band5 MR UTRA + E-UTRA + NB-IoT in-band + NB-IoT guard band5 | MR GSM + UTRA SR GSM (MCBTS) SR UTRA (SC, MC) | MR GSM + E-UTRA SR GSM (MCBTS) SR E-UTRA (SC, MC, CA) E-UTRA + NB-IoT in-band3 E-UTRA + NB-IoT guard band4 MR GSM + E-UTRA + NB-IoT in-band3 MR GSM + E-UTRA + NB-IoT guard band4 E-UTRA + NB-IoT in-band + NB-IoT guard band5 MR GSM + E-UTRA + NB-IoT in-band + NB-IoT guard band5 | MR GSM + UTRA + E-UTRA MR GSM + UTRA MR GSM + E-UTRA MR UTRA + E-UTRA SR GSM (MCBTS) SR UTRA (SC, MC) SR E-UTRA (SC, MC, CA) | MR GSM + UTRA* MR GSM + E-UTRA MR E-UTRA + UTRA* SR UTRA (SC, MC)* SR E-UTRA (SC, MC) E-UTRA + NB-IoT in-band3 E-UTRA + NB-IoT guard band4 MR GSM + E-UTRA + NB-IoT in-band3 MR GSM + E-UTRA + NB-IoT guard band4 MR UTRA + E-UTRA + NB-IoT in-band3 MR UTRA + E-UTRA + NB-IoT guard band4 E-UTRA + NB-IoT in-band + NB-IoT guard band5 MR GSM + E-UTRA + NB-IoT in-band + NB-IoT guard band5 MR UTRA + E-UTRA + NB-IoT in-band + NB-IoT guard band5 |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC1, BC2 or BC3 | BC2 | BC2 | BC2 | BC2 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: For this configuration related to BC2 bands, the support of UTRA in band 3 is declared by the manufacturer. NOTE 3: The support of NB-IoT in-band operation is optional and declared by the manufacturer. NOTE 4: The support of NB-IoT guard band operation is optional and declared by the manufacturer. NOTE 5: The support of both NB-IoT in-band and guard band operation is optional and declared by the manufacturer. |  |  |  |  |  |  |  |

# Table 2: Proposed options for new capability set (NR+LTE) in Rel’15

| Capability Set supported by the BS | CS16 option 1 | CS16 option 2 |
| --- | --- | --- |
| Supported RATs | NR E-UTRA | NR E-UTRA NB-IoT in-band3 NB-IoT guard band4 |
| Supported configurations | MR E-UTRA + NR SR NR (SC, MC, CA) SR E-UTRA (SC, MC, CA) | MR E-UTRA + NR SR NR (SC, MC, CA) SR E-UTRA (SC, MC, CA) E-UTRA + NB-IoT in-band3 E-UTRA + NB-IoT guard band4 MR NR + E-UTRA + NB-IoT in-band3 MR NR + E-UTRA + NB-IoT guard band4 E-UTRA + NB-IoT in-band + NB-IoT guard band5 MR NR + E-UTRA + NB-IoT in-band + NB-IoT guard band5 |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: For this configuration related to BC2 bands, the support of UTRA in band 3 is declared by the manufacturer. NOTE 3: The support of NB-IoT in-band operation is optional and declared by the manufacturer. NOTE 4: The support of NB-IoT guard band operation is optional and declared by the manufacturer. NOTE 5: The support of both NB-IoT in-band and guard band operation is optional and declared by the manufacturer. |  |  |

Proposal 1: Agree to use either option 1 or option 2 (see Table 2 above) for the new LTE+NR capability set in Rel’15.

Existing MSR requirements were defined on the basis of GSM, UTRA and E-UTRA specifications. As NR WID considers NR+LTE operation only, it is proposed to define requirements for new capability set on the basis of NR and LTE requirements only. While most of the MSR requirements need minor updates with respect to the new channel bandwidths for NR, some of them would require additional modifications (new requirement on operating band unwanted emissions on the basis of Cat B option 1, interfering signal for blocking and intermodulation requirements, new in-band/out-of-band boundary for bands >200MHz).

Proposal 2: Requirements for new capability set (NR+LTE) shall be defined on the basis of NR and LTE requirements only.

# Conclusion

On the basis of discussion in this document, it is proposed to agree the following proposals:

Proposal 1: It is proposed to agree on either option 1 or option 2 (see Table 2 above) for the new LTE+NR capability set in Rel’15.

Proposal 2: Requirements for new capability set (NR+LTE) shall be defined on the basis of NR and LTE requirements only.