3GPP TSG RAN WG4 Meeting #AH-1801	R4-1800946

San Diego, US, 22 - 26 January 2018

Source:		Rohde & Schwarz

Title:	Tx radiated spurious emissions measurements with dynamic measurement bandwidth

Agenda Item:		4.3.2.8.2

Document for:	Approval

# Introduction

The OTA spurious emissions limits are specified by [1] for UEs and [2] for BS. As TRP is the selected metric for spurious emissions, test time becomes a very critical issue [6] for FR2 transmitters in particular. Furthermore, OTA testing suffers from frequency-dependent reduction of available signal-to-noise ratio at the measurement receiver due to increasing free space path loss for higher frequencies. Now, the key for reducing test time and to improve the SNR at the measurement receiver is the measurement resolution bandwidth (RBW). Unfortunately, RBW has conflicting impact on both, the test time and the SNR. While an increasing RBW reduces test time on the one hand side, it reduces the SNR on the other hand.

This contribution proposes a spurious emissions test method which addresses this conflict by dynamic selection of the RBW.

# Discussion

## Problem description

According to [4] the frequency range for measurements of unwanted emissions for devices operating in FR2 (fundamental frequency range) is 30 MHz up to the 2nd harmonic.

According to [3] the lowest detectable EIRP emission level can be expressed as

which includes the frequency-dependent path loss Lpath and test setup specific parameters like the measurement antenna gain Grx and the measurement receiver’s noise figure F. While M represents an optional measurement margin, the remaining variable parameter is the bandwidth B, which corresponds to the measurement resolution bandwidth RBW.

Assuming an upper fundamental frequency edge at 43.5 GHz, there is a need to measure spurious emissions up to 87 GHz according to [4]. For that example, Figure 1 depicts the Tx spurious emissions limits for both, UEs (red solid line) and BSs (blue solid line), for the frequency range 30 MHz to 87 GHz (Note: logarithmic scale). With the assumption of an OTA measurement range of 1 m the spurious emissions measurement receiver (e.g. spectrum analyzer) needs to take into account the frequency-dependent free space path loss Lpath(f), accordingly. This is illustrated in Figure 1 by Tx spurious emissions limits which are reduced by Lpath(f) (dashed lines).

Figure 1: 3GPP NR FR2 spurious emissions limits with and without Lpath(f)@ 1m distance in the frequency range 30 MHz to 87 GHz

The green dashed line depicts the Displayed Average Noise Level (DANL) which represents e.g. the noise floor of a spectrum analyzer at the specified RBW, typically -105 dBm/100 kHz and -95 dBm/MHz, respectively, for a high performance spectrum analyzer [5]. Now, the distance between the “OTA” spurious emissions limit lines and the DANL line provides the available measurement margin or signal-to-noise ratio (SNR), which obviously decreases with the measurement frequency. Consequently, to maintain a minimum SNR with increasing measurement frequency, the RBW needs to be reduced accordingly.

Note: Test setup specific cabling losses and measurement antenna gain will contribute to the available SNR, too. However, this is not relevant for the considerations of this contribution.

Observation 1:

There is a need for reducing the RBW with increasing measurement frequency in order to maintain a minimum SNR (measurement margin). Thus, constant RBWs as in legacy spurious emission measurements are not optimal.

However, for the sake of minimizing the measurement time, it is desirable to use the maximum possible RBW at any measurement frequency. For a given maximum noise level (e.g. Tx spurious emission limit with a certain measurement margin M and a known noise floor of the measurement setup, the optimum RBW can be calculated according to [7] as

RBWHz=10Maximum Noise Level dBm-Noise Floor (dBm)10

Observation 2:

The optimum RBW with respect to test time is a function of the Tx spurious emission limit and the measurement receiver’s noise floor. Thus, fixed RBWs are very inefficient.

## Measurement method using dynamic RBW

According to observations 1 and 2, it is desirable to select the RBW dynamically throughout the Tx spurious emissions measurements in order minimize the test time and to maintain a minimum SNR.

Proposal:

In order to minimize the test time of Tx spurious emissions measurement, use the following method based on dynamic RBW selection:

- Fast sweep to determine the optimum measurement bandwidth based on the maximum allowed spur level and the required signal-to-noise ratio (margin vs. noise floor estimate).

- Sweep to detect possible spurs using the optimum measurement bandwidth determined in step 1.

- Final measurement of in step 2 detected spurs with desired SNR (margin) incl. standard limit check.

# Conclusion

Observation 1:

There is a need for reducing the RBW with increasing measurement frequency, in order to maintain a minimum SNR (measurement margin). Thus, constant RBWs as in legacy spurious emission measurements are not optimal.

Observation 2:

The optimum RBW with respect to test time is a function of the Tx spurious emission limit and the measurement receiver’s noise floor. Thus, fixed RBWs are very inefficient.

Proposal:

In order to minimize the test time of Tx spurious emissions measurement, use the following method based on dynamic RBW selection:

- Fast sweep to determine the optimum measurement bandwidth based on the maximum allowed spur level and the required signal-to-noise ratio (margin vs. noise floor estimate).

- Sweep to detect possible spurs using the optimum measurement bandwidth determined in step 1.

- Final measurement of in step 2 detected spurs with desired SNR (margin) incl. standard limit check.

# References

- 3GPP TS 38.101-2 V1.0.0 (2017-12) sub clause 6.5.3. “Spurious emissions”

- 3GPP TS 38.104 V1.0.0 (2017-12) sub clause 9.7.5. “OTA transmitter spurious emissions”

- 3GPP RAN4#84 R4-1708097, “Lowest detectable level for OTA unwanted emission”, Ericsson

- ITU-R SM.329-12

- R&S white paper, “Advanced Techniques for Spurious Measurements with R&S FSW-K50”, https://www.rohde-schwarz.com/appnote/1EF97

- 3GPP RAN4#84 R4-1707230, “Estimation of general spurious emission test time for mmWave UE”, Anritsu Corporation

- “Fundamentals of spectrum analysis”, Christoph Rauscher, Rohde & Schwarz