3GPP TSG-RAN WG4 NR AH-1801	R4-1800953

San Diego, USA, January 22th-26th, 2018

Agenda item:	 4.6.5.1

Source: 	Qualcomm Incorporated

Title: 	PBCH Performance and SSB Index Acquisition

Document for:	Approval

# 1. 	Introduction

During the last RAN4#85 meeting a way forward for SSB index acquisition time has been agreed [1], together with revised PBCH reading simulation assumptions [2]. In this paper we present updated simulation results for sub-6 and mmWave scenarios under various propagation conditions and propose a requirement for SSB index acquisition time.

# 2. 	Assumptions

PBCH performance simulations have been performed according to the assumptions in [2] for these particular cases:

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated)

- Carrier Frequency 30 GHz: channel models AWGN, CDL-A/B/C, TDL-A/B/C/D

- Carrier Frequency 2 GHz: channel models AWGN, EPA5, ETU30, ETU70, TDL-A/B/C/D

- One-shot detection: no combining of different PBCHs

In the agreed way forward [1], it is suggested that the SS/PBCH index acquisition time will be derived as PBCH-DMRS sequence acquisition time for FR1(sub-6) and as PBCH decoding (NR-MIB acquisition) time for FR2 (mmWave). Thus, mmWave results are reported for Case 1 (PBCH reading), whilst sub-6 results are reported for Case 2 (PBCH-DMRS time index reading without decoding PBCH data), for SNR of -6, -8 and -10 dB.

For Case 1, the basic SI reading time is defined as the number of SS-blocks required for 99% to successfully decode the PBCH. For Case 2, DMRS time index reading time is defined as the number of SS-blocks required for 99% to successfully reading DMRS time index.

# 3.		Simulation Results

# 3.1 Results for 30 GHz – Case 1 PBCH SI Reading Performance

It can be observed that max. 3 independent decoding attempts are required to guarantee 99% detection probability at - 6 dB SNR in fading channel conditions.

Table 1. PBCH SI reading time at 30 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | CDL-A |  | CDL-B |  | CDL-C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 3 | 2 | 2 | 2 | 2 | 2 |
| -8 | 1 | 1 | 4 | 4 | 3 | 3 | 4 | 4 |
| -10 | 3 | 3 | 7 | 6 | 7 | 8 | 7 | 6 |

| SNR [dB] | TDL-A |  | TDL-B |  | TDL-C |  | TDL-D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| -8 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| -10 | 8 | 8 | 7 | 7 | 7 | 7 | 7 | 7 |

3.2 Results for 2 GHz – Case 2 DMRS Time Index Detection Performance

It can be observed 1 detection attempts is sufficient to guarantee 99% detection probability at - 6 dB SNR in fading channel conditions.

Table 2. DMRS time index reading time at 2 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | EPA5 |  | ETU30 |  | ETU70 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| -8 | 1 | 1 | 2 | 2 | 1 | 2 | 2 | 2 |
| -10 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 |

| SNR [dB] | TDL-A |  | TDL-B |  | TDL-C |  | TDL-D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| -8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| -10 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

# 4. 	Discussion and Proposals

In the agreed way forward [1], it is suggested that a side condition of SNR = -6 dB and 99% decoding success rate could be used for deriving the requirements. Taking the corresponding results from section 3 and considering one extra attempt as margin for implementation, we recommend the following:

Proposal 1: The SSB Index acquisition time TSSB_time_index for FR1 corresponds to 2 attempts of DMRS acquisition.

Proposal 2:  The SSB Index acquisition time TSSB_time_index for FR2 corresponds to 4 attempts of PBCH SI reading.

# 5. 	Conclusions

In this paper, we provided simulation results according to the simulation assumptions agreed in [2] and discussed the requirements following the agreed way forward in [1]. As outcome the following in proposed:

Proposal 1: The SSB Index acquisition time TSSB_time_index for FR1 corresponds to 2 attempts of DMRS acquisition.

Proposal 2:  The SSB Index acquisition time TSSB_time_index for FR2 corresponds to 4 attempts of PBCH SI reading.

A text for CR to TS 38.133 capturing these proposals in included in the Appendix.

# 6. 	References

- R4-1714251, “Way forward on the SSB time index acquisition time”, Huawei, HiSilicon, Ericsson, Intel, Mediatek, Samsung, Qualcomm

- R4-1713437, “Updated PBCH Simulation assumption after redesign”, Huawei, HiSilicon

# Appendix

The following is a text proposal for CR to TS 38.133 reflecting the conclusion above, should those be agreeable.

------------------------------------------------------------------------------------------------------------------------------------------------

#### 9.2.5.1	Intrafrequency cell identification

Editor’s Note : The requirements below have been derived so far assuming no configured Scell or E-UTRA SCell. The requirements when one or more SCells or E-UTRA SCells are configured is for further study. The requirements below have been derived without considering gap sharing when all SMTC occasion are fully overlapping with measurement gaps.

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if the UE has already identified an SS block with different time index and the same physical cell ID, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index

Tidentify_intra_without_index = TPSS/SSS_sync + T SSB_measurement_period  ms

Tidentify_intra_with_index = TPSS/SSS_sync + T SSB_measurement_period + TSSB_time_index

Where:

TPSS/SSS_sync: it is the time period used in PSS/SSS detection given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-5 (deactivated Scell) or 9.2.5.1-6 (deactivated SCell)

TSSB_time_index: it is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3 or 9.2.5.1-4 or  9.2.5.1-7 (deactivated SCell) or  9.2.5.1-8 (deactivated SCell)

T SSB_measurement_period: equal to a measurement period of SSB based measurement given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated Scell) or 9.2.5.2-4(deactivated SCell)

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | max(600ms,[5 or 6]x SMTC period) Note 1 |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD Note 1 |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.11-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | max(40ms, [2] x SMTC period)Note 1 |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-4: Time period for time index detection (Frequency range FR2)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBDNote 1 |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |

Table 9.2.5.1-6: Time period for PSS/SSS detection, deactivated SCell (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |

Table 9.2.5.11-7: Time period for time index detection, deactivated SCell (Frequency range FR1)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |

Table 9.2.5.1-8 Time period for time index detection, deactivated SCell (Frequency range FR2)

| DRX cycle | TSSB_time_index |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD |
| DRX1 < DRX cycle≤ DRX2 | TBD |
| … | … |
| … | … |