3GPP TSG-RAN WG4 Meeting 2018-AH-01                             R4-1800990

San Diego, CA, USA, 22 – 26 January, 2018

Agenda Item:	4.3.2.1.4

Source:	Samsung

Title:	  Further efforts on EIRP CDF for spherical coverage

Document for:	Discussion

- Background

As stipulated in [1] and [2], RAN4 has started to consider more aspects of real-product implementations for the simulation analysis for Spherical coverage EIRP. These aspects significantly impact the achievable EIRP results, and thus should be gathered more data for CDF calculations. In this regard, the way forward was approved in RAN4#85 which has the harmonized assumptions for an adequate requirement of spherical coverage EIRP with the clear understanding of UE implementation and design aspect [3]. The WF also encourages UE vendors to submit their simulation results that shall include all the six assumptions as follows:

In this contribution, we would like to provide the additional EIRP CDF data to follow the latest agreements.

- Simulation results

Table 1 summarizes the CDF based EIRP results for the spherical coverage carried out the six simulation. It is shown that the EIRP degradation at each %-tile, 50-20%, from the 100%-tile value.

Table 1: Summary of simulation results with assumptions in [3]

|  |  | Assumptions1 | Assumptions2 | Assumptions3 | Assumptions4 | Assumptions5 | Assumptions6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range |  | n257 | n257 | n257 | n257 | n257 | n257 |
| # of antenna module/set in total |  | 1 | 1 | 2 | 2 | 3 | 3 |
| Finite UV test points |  |  |  |  |  |  |  |
| Beam phase shifter controller | degree | 45 | 45 | 45 | 45 | 45 | 45 |
| Antenna module/set location |  | Top / Bottom | Top / Bottom | Top & Bottom | Top & Bottom | Left & Right & Bottom | Left & Right & Bottom |
| Front/Back cover |  | Glass | Glass | Glass | Glass | Glass | Plastic |
| Side cover / Frame |  | Metal | Plastic | Metal | Metal | Metal | Plastic |
| Device size (WxHxD) | cm3 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 | 66.6 |
| Display panel | Y/N | Y | Y | Y | N | Y | Y |
| Bezel Margin | mm | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| %-tile point |  | Decreased by (from 100%-tile) |  |  |  |  |  |
| @ 50% | dB | 8.6 | 6.4 | 7 | - | 6 | 4.8 |
| @ 40% | dB | 10.4 | 7.4 | 9.4 | - | 8.4 | 5.8 |
| @ 30% | dB | 12.4 | 9 | 11.4 | - | 10.6 | 7.4 |
| @ 20% | dB | 15 | 11.8 | 13.8 | - | 13 | 9 |

First of all, the impact of the UE cover material can be analysed by comparing the assumption 1 and 5 to 2 and 6, respectively. The metal frame makes its EIRP value decrease up to 4 dB at each point, and the degradations worsen with the lower percentile point.

Observation 1: Differences between metal and plastic frame make the EIRP at each point decrease by up to 4 dB. The gap is bigger with the lower CDF point.

Moreover, completely different with previous contributions from some companies, adding more antenna module increases just about 1 dB by comparing the simulation results of the assumption 1, 3 and 5. When the other conditions for the practical UE are the same, increasing the number of chipsets is not a good option to have better spherical coverage. Furthermore, this option will also restrict incumbent features and legacy antennas, and may interfere with them and vice versa.

Observation 2: Adding more antenna module increases about 1 dB in the practical UE assumptions, and that will restrict incumbent features and legacy antennas.

- Conclusion

In this contribution, we provide our simulation results to follow the agreement in RAN4#85. Through comparing the six results, two important observations can be derived as below.

Observation 1: Differences between metal and plastic frame make the EIRP at each point decrease by up to 4 dB. The gap is bigger with the lower CDF point.

Observation 2: Adding more antenna module increases about 1 dB in the practical UE assumptions, and that will restrict incumbent features and legacy antennas.

- Reference

[1] 	R4-1711036, “Consideration of EIRP spherical coverage requirement,” Samsung, Apple, Intel, 3GPP RAN4#84Bis, Oct 2017

[2] R4-1712377, “Further consideration of EIRP spherical coverage,” Samsung, 3GPP RAN4#85, Nov 2017

[3] R4-1714455, “WF on spherical coverage in FR2,” Samsung, Apple, LGE, Intel, Oppo, Xiaomi, Vivo, MediaTek, 3GPP RAN4#85, Nov 2017