3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801037

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: correction of the OTA Rx in-band blocking requirement (10.5.2.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WF agreements on the inband /OOB boundaries are hardly readable when appied in text. Therefore it is proposed to use ΔfOOB offset values, and to describe the OTA in-band blocking requirement based on parametrized approach in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New table 10.5.2.2-0 with the ΔfOOB values for 1-O - Apply the parametrized description for blocking requirement based on the ΔfOOB offset - Wording alignment with other blocking requirements - Reference corrections, editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification of the blocking requirement for various NR operating bands; wrong references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 10.5.2	OTA in-band blocking

#### 10.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 10.5.2.2	Minimum requirement for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are are within the within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with

OTA wanted and OTA interfering signal specified in tables 10.5.2.2-1, 10.5.2.2-2 and 10.5.2.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclase 10.3.2 for each channel bandwidth and further specified in annex A.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For BS type 1-O the OTA in-band blocking requirement apply from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the operating band. The ΔfOOB for BS type 1-O is defined in table 10.5.2.2-0.

Table 10.5.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB [MHz] |
| --- | --- | --- |
| BS type 1-O | 100 MHz > FUL_high – FUL_low | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

[For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3.]

# Table 10.5.2.2-1: General OTA blocking requirement for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] | OTA Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | EISREFSENS + 6 dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 7.5 | 5 MHz NR signal SCS: 15 kHz |
|  | EISMINSENS + 6 dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 7.5 | 5 MHz NR signal SCS: 15 kHz |
| 25,30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 30 | 20 MHz NR signal SCS: 15 kHz |
|  | EISMINSENS + 6 dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 30 | 20 MHz NR signal SCS: 15 kHz |

# Table 10.5.2.2-2: OTA narrowband blocking requirement for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 | EISREFSENS + 6 dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISMINSENS + 6 dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | EISREFSENS + 6 dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISMINSENS + 6 dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| NOTE: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth |  |  |

# Table 10.5.2.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| NR channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342,5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz NR signal, 1 RB SCS: 15 kHz |
| 10 | [347.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | [352.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20 MHz NR signal, 1 RB SCS: 15 kHz |
| 30 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 40 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 50 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 60 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 70 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 80 | [552.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 90 | [562.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 100 | [557.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |

------------------------------ End of modified sections ------------------------------