3GPP TSG-RAN WG4 AH meeting	R4-1801108

San Diego, USA, 22 - 26 Jan 2018

Agenda Item:	4.3.2.1.5

Souce:		Samsung (offline discussion moderator)

Title:	Offline discussion on network performance analysis for spherical coverage

Document for:	Discussion and decision

# Introduction

The outcome of Tuesday morning coffee break offline discussion on network performance analysis for spherical coverage is presented in this document.

# Offline discussion outcome

NW simulation scenarios

Offline agreement 1-1: At least indoor hotspots scenario as described in TR 38.803 should be simulated.

Modified Proposal 1-2: For dense urban scenarios, decide from the following options:

- Option 1: Use the scenario as described in TR 38.803

  - Support: Huawei, OPPO, Apple

  - Discussion:

    - Qualcomm: Issue is that inter-BS distance can be very small (e.g. 3m)

- Option 2: Modify urban macro scenario to resemble a “dense urban” scenario (e.g. ISD, BS/UE height, channel model, indoor/outdoor ratio, etc)

  - Support: LGE, Qualcomm, Samsung

  - Discussion:

    - OPPO: ISD should be FFS (<200m)

    - Huawei: Channel model should not be UMa

Recommendation: Continue discussion to down-select between Option 1 and Option 2

Modified Proposal 1-3: For urban macro scenario, decide from the following options

- Option 1: As described in TR 38.803 (ISD=200m)

  - Discussion

    - LGE: Issue is there is outage

    - Qualcomm: Have a case where there is 0% indoor

    - DCM: How about 500m ISD?

- Option 2: Not simulated

- Possible agreement: Option 1 with 0% indoor as the baseline.

  - Other ISD FFS (e.g. 500m).

Proposal 1-4 (not discussed in offline): Other scenarios to be considered:

- “Dense urban macro” (details FFS)

- “Suburban street micros” (details FFS)

- Recommendation: Discuss the need for additional scenarios in the Jan meeting, and make a decision if possible.

NW simulation assumptions

Offline agreement 2-1: UE elevation distribution is modified to be uniform from 0 to 180 degrees.

Modified Proposal 2-2: For UE resource allocation (UL), assume:

- 20MHz for outage evaluation

- 200MHz for mean throughput evaluation

  - FFS how to exclude cell edge UEs in outage, e.g. 0 throughput UEs are excluded from mean throughput calculation)

- Other details FFS

Offline agreement 2-3: For indoor/outdoor UE ratios, assume:

- For urban macro: 0%, 20%, 100% indoor

- For dense urban: 0%, 20%, 100% indoor

- Note: this is only for initial simulation comparison and NOT for performance requirement conclusion. Further down selection on the ratio to be done for performance requirement conclusion

Offline agreement 2-4: No body blockage and hand grip modelling as baseline. Companies are encouraged to provide results with blockage

Proposal 2-5 (not discussed in offline): For UE antenna pattern modeling (that parameterizes (percentile, EIRP) the spherical coverage requirements), consider the following options:

- Option 1: Use 38.803 UE 2x2 antenna configuration with implementation loss

- Option 2: Up to each company. Companies do not need to disclose the UE antenna pattern assumed in simulation but need to provide the resulting EIRP CDF.

# Reference

- R4-1714455	WF on spherical coverage in FR2,	Samsung, Apple, LGE, Intel, oppo, Xiaomi, Vivo, MediaTek

- R4-1800207	Email discussion on network performance analysis for spherical coverage, Samsung