3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801226

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: correction of the OTA Rx out-of-band blocking requirement (10.6.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WF agreements on the inband /OOB boundaries are hardly readable when appied in text. Therefore it is proposed to use ΔfOOB offset values, and to describe the OTA OOB blocking for general and co-location requirement based on parametrized approach in the text. This is aligned with the in-band blocking requirement correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Apply the parametrized description for blocking requirement based on the ΔfOOB offset, for general and co-location requirements - Wording alignment with other blocking requirements - Reference corrections, editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification of the blocking requirement for various NR operating bands; wrong references. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.1, 10.6.2.1, 10.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 10.6	OTA out-of-band blocking

### 10.6.1	General

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

### 10.6.2	Minimum requirement for BS type 1-O

#### 10.6.2.1	General minimum requirement

The requirement applies at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted signal applies to all supported polarizations, under the assumption of polarization matching. The interferer shall be polarization matched in-band and the position maintained for OOB measurements.

For a BS type 1-O with cross polarized antennas the single polarized interferer signal power shall be doubled such that each polarization of the BS receives half the interferer signal power.

For OTA wanted and OTA interfering signals coupled to the RIB using the parameters in table 10.6.2.1-1, the following requirements shall be met:

-	Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for OTA wanted signal is identified in subclause 10.3.2 for each channel bandwidth and further specified in annex A.

[For a multi-band RIB, the OTA out-of-band requirement applies for each supported operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to subclause 7.4.2.2 shall be excluded from the OTA out-of-band blocking requirement].

For BS type 1-O the OTA out-of-band blocking requirement apply from 1 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the operating band. The ΔfOOB for BS type 1-O is defined in table 10.5.2.2-0.

# Table 10.6.2.1-1: OTA out-of-band blocking performance requirement

| Interfering signal mean power [dBm] | Interfering signal distance [m] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| [36] | [30] | EISminSENS + 6 dB (Note) | [CW carrier] |
| NOTE: EISminSENS depends on the channel bandwidth as specified in subclause 9.2. |  |  |  |

#### 10.6.2.2	Co-location minimum requirement

This additional OTA out-of-band blocking requirement may be applied for the protection of BS receivers when NR, E-UTRA BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with a BS.

The requirement is a co-location requirement, the interferer power levels specified at the co-location reference antenna conducted input.

The requirement is valid over the minSENS RoAoA.

Interfering signal shall be applied to the co-location reference antenna. The interfering power is specified per polarization.

When the wanted and an interfering signal using the parameters in table 10.6.2.2-1, the following requirements shall be met:

-	Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for OTA wanted signal is identified in subclause 10.3.2 for each channel bandwidth and further specified in annex A.

For BS type 1-O the OTA blocking requirement for co-location with BS in other frequency bands is applied for all operating bands for which co-location protection is provided.

# Table 10.6.2.2-1: OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Interfering signal mean power for WA BS [dBm] |  | Interfering signal mean power for MR BS [dBm] |  | Interfering signal mean power for LA BS [dBm] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | +46 |  | +38 |  | +24 | EISminSENS + 6 dB (note) | [CW carrier] |
| NOTE: EISminSENS depends on the BS class and on the channel bandwidth, see subclause and 10.2. |  |  |  |  |  |  |  |

------------------------------ End of modified sections ------------------------------