3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801241

San Diego, California, USA, 22 – 26 January 2018

Agenda Item:	4.4.1.1

Source: 	Nokia, Nokia Shanghai Bell, Ericsson, Huawei

Title: 	TP to TR 38.817-02: NR BS in-band and out-of-band boundaries for FR1 (5.9)

Document for:	Approval

# 1.	Introduction

The way forward on in-band and out-of-band boundaries for FR1 was agreed in RAN4#84bis [1]. The agreements have not been recorded in the NR BS TR 38.817-02 [2], although some of them have been implemented in the NR BS TS 38.104 [3].

This contribution provides a text proposal to record the agreements, which cover both conducted and radiated transmitter and receiver characteristics, into a new sub-clause in the General clause in TR 38.817-02 [2] for future reference.

# 2.	Text Proposal

<New clause>

## 5.9	In-band and out-of-band boundaries for FR1

Agreements of the boundary between OBUE and spurious emissions are summarized in tables 5.9-1 and 5.9-2. The boundary is valid regardless of whether it is single-RAT case or MSR case, and NR new band or refarming band (except Band 46). If wider band(s) than current maximum bandwidth is specified (i.e. wider than 900MHz for NR), the values of the boundary should be discussed.

# Table 5.9-1: ΔfOBUE (NR single-RAT case)

| Applicable specification |  | 3GPP TS 38.104 3GPP TS 38.141 | 3GPP TS 38.104 3GPP TS 38.141 |
| --- | --- | --- | --- |
| Applicable BS type |  | BS type 1-C | BS type 1-H BS type 1-O |
| ΔfOBUE | Operating band < 100 MHz | 10 MHz | 10 MHz |
|  | 100 MHz ≦ Operating band ≦ 200MHz | 10 MHz | 40 MHz |
|  | 200 MHz < Operating band ≦ 900 MHz | 40 MHz | 40 MHz |

# Table 5.9-2: ΔfOBUE (NR MSR case)

| Applicable specification |  | 3GPP TS 37.104 3GPP TS 37.141 | 3GPP TS 37.105 3GPP TS 37.145-1 3GPP TS 37.145-2 |
| --- | --- | --- | --- |
| Applicable BS type |  | BS type 1-C | BS type 1-H BS type 1-O |
| ΔfOBUE | Operating band < 100 MHz | 10 MHz | 10 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 10 MHz | 40 MHz |
|  | 200 MHz < Operating band ≦ 900 MHz | 40 MHz | 40 MHz |

Agreements of the in-band / out-of-band boundary on receiver characteristics are summarized in tables 5.9-3 and 5.9-4. The boundary is valid regardless of whether it is single-RAT case or MSR case, and NR new band or refarming band (except Band 46). If wider band(s) than current maximum bandwidth of band is specified (i.e. wider than 900MHz for NR), the values of the boundary should be discussed.

# Table 5.9-3: ΔfOOB (NR single-RAT case)

| Applicable specification |  | 3GPP TS 38.104 3GPP TS 38.141 | 3GPP TS 38.104 3GPP TS 38.141 |
| --- | --- | --- | --- |
| Applicable BS type |  | BS type 1-C | BS type 1-H BS type 1-O |
| ΔfOOB | Operating band < 100 MHz | 20 MHz | 20 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 20 MHz | 60 MHz |
|  | 200 MHz < Operating band ≦ 900 MHz | 60 MHz | 60 MHz |

# Table 5.9-4: ΔfOOB (NR MSR case)

| Applicable specification |  | 3GPP TS 37.104 3GPP TS 37.141 | 3GPP TS 37.105 3GPP TS 37.145-1 3GPP TS 37.145-2 |
| --- | --- | --- | --- |
| Applicable BS type |  | BS type 1-C | BS type 1-H BS type 1-O |
| ΔfOOB | Operating band < 100 MHz | 20 MHz | 20 MHz |
|  | 100 MHz ≦ Operating band ≦ 200 MHz | 20 MHz | 60 MHz |
|  | 200 MHz < Operating band ≦ 900 MHz | 60 MHz | 60 MHz |

<Next changed clause>

#### 6.6.4.1	General

A set of band-centric masks are defined for NR in Frequency Range 1, based on the same principles as for LTE. Details of the masks are as follows:

-	The UEM should be defined up to an offset ΔfUEM from the band edge, where ΔfUEM is specified in tables 5.9-1 and 5.9-2. This serves as a baseline for the boundary between UEM and spurious emission for both Category A and Category B spurious emission limits.

-	The emission levels for the UEM will be the same as for LTE (5 MHz and larger channel BW) in the full frequency range (FDL_low– ΔfUEM) ~ (FDL_high+ΔfUEM).

-	The same UEM tables will be used for the different ΔfUEM.  For Category B spurious emissions, the limits will be taken from LTE Category B Option 1.

-	The following variations for the Unwanted Emissions Mask (UEM) tables should be used for sub-6 GHz:

–	BS classes

–	Category of emission (A/B)

–	Frequency range (above/below 1 GHz)

–	Power levels (for some BS classes)

The value of ΔfUEM determines the extent of the mask outside the operating band, but it also defines the spurious emissions requirements. It is a parameter that will vary between bands and is therefore best documented in a table, listing its value for different bands. Since it is common to spurious emissions and UEM requirements, the table should be placed in the general section of Unwanted emissions (6.6.1).

Specification of UEM relies on a few common parameters defining the frequency offset. These can largely be re-used from LTE:

-	Since the limits will be the same as for LTE, we also need to consider re-using the concept of two definitions of offset from the channel edge: Δf defining offset to the edge of the measurement filter and f_offset defining the offset to the centre of the filter. Both should be kept, since the difference (half the measurement bandwidth) is not negligible.

-	Keeping both Δf and f_offset, we also need to define two maximum values setting the maximum offset to which the UEM extends: fmax and f_offsetmax.

For Wide Area BS, the UEM level in the spurious domain is aligned with ITU-R recommendation SM.329. Consequently, there is a need to distinguish between Category A and B emission limits and also between frequencies below and above 1 GHz, since they have different limits and/or measurement bandwidths. For this reason, Category A and B should be in separate subclauses:

Wide Area BS, Category A (subclause)

Table: Category A (<1 GHz)

Table: Category A (1 GHz < f < 6 GHz)

Wide Area BS, Category B (subclause)

Table: Category B (<1 GHz)

Table: Category B (1 GHz < f < 6 GHz)

Medium Range BS (subclause)

Table: Category A & B

Local Area BS (subclause)

Table: Category A & B

Editor’s note: Operation in Non-contiguous spectrum or Multi-band BS are both integral part of the LTE specifications. If no additional aspect to such operation is discovered during the NR Work Item, it would be possible to re-use the LTE text regarding non-contiguous spectrum. Initially, text concerning these two features can be introduced in TS 38.104 in brackets and be for further study.

<End of change>

# References

[1]	R4-1711779, “WF on in-band and out-of-band boundary”, NTT DOCOMO, INC., Nokia, Ericsson, Huawei, ZTE.

[2]	R4-1714547, “TR 38.817-02 v0.5.0: General aspects for BS RF for NR”, Ericsson.

[3]	3GPP TR 38.104 v15.0.0, “NR; Base Station (BS) radio transmission and reception”.