3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801280

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: correction of the TX transient time figure (9.5.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was observed that the figure for the relations of transmitter ON period, transmitter OFF period and transmitter transient period for OTA transmitter ON/OFF power requirement was reused from E-UTRA and needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alignment among the conducted and OTA requirements for the transmitter ON/OFF power: - 6.4.2: Application of NR terminology, editorial corrections - 9.5.2, 9.5.3: correction of the E-UTRA figure, editorial corrections, ETSI styles application |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect figure for the OTA transmitter ON/OFF power requirement would exist in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 9.5.2, 9.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The OFF power levels for 1-O and 2-O were agreed in R4-1714125, as already captured in the baseline TS 38.104. |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 6.4.2	Transmitter transient period

#### 6.4.2.1	General

The transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.4.2.1-1 Example of relations among transmitter ON period, transmitter OFF period and transmitter transient period

#### 6.4.2.2	Minimum requirement for BS type 1-C and BS type 1-H

For BS type 1-C and BS type 1-H, the transmitter transient period shall be shorter than the values listed in the minimum requirement table 6.4.2.2-1.

Table 6.4.2.2-1: Minimum requirement for the transmitter transient period for for BS type 1-C and BS type 1-H

| Transition | Transient period length [µs] |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

For BS type 1-C, the transmitter transient period shall be shorter than the values listed in the minimum requirement table 6.4.2.2-1.

#### 6.4.2.3	Void

------------------------------ Next modified section ------------------------------

### 9.5.2	OTA transmitter OFF power

#### 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70/N µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the BS (BWConfig) centred on the assigned channel frequency during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz. For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location reference antenna conducted output(s). For BS type 2-O the transmitter OFF power is defined as TRP.

[For BS capable of multi-band operation, the requirement is only applicable during the transmitter OFF period in all supported operating bands.]

#### 9.5.2.2	Minimum requirement for type BS type 1-O

The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz.

#### 9.5.2.3	Minimum requirement for type BS type 2-O

The requirements for OTA transmitter OFF power spectral density for BS type 2-O should be less than [-36 dBm/MHz].

NOTE: The OTA transmitter OFF power is measured together with the OTA transmitter transient period. As the OTA transmitter OFF power for BS type 2-O is defined as TRP, the measurement procedure for the OTA transmient period is FFS.

### 9.5.3	OTA transient period

#### 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

is FFS

#### 9.5.3.1	Minimum requirement for BS type 1-O

For BS type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.1-1.

Table 9.5.3.1-1: Minimum requirement for OTA transmitter transient period for BS type 1-O

| Transition | Transient period length [µs] |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

#### 9.5.3.2	Minimum requirement for BS type 2-O

For BS type 2-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1.

Table 9.5.3.2-1: Minimum requirement for OTA transmitter transient period for BS type 2-O

| Transition | Transient period length [µs] |
| --- | --- |
| OFF to ON | 3 |
| ON to OFF | 3 |

------------------------------ End of modified sections ------------------------------