3GPP TSG-RAN WG4 Meeting AH-1801	R4-1801283

## San Diego, CA, USA, 22 – 26 Jan, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.104 | CR | CRNum | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104: consideration of regional requirements (4.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-01-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following arrangements are proposed for the regional requirements consideration in TS 38.104: - Additional regional requirements in subclause 4.5 (Regional requirements) are proposed to be kept as a placeholder for the time being; - Align the regional requirements in table 4.5-1 for the conducted and radiated requirements, where possible; - For the emission limits subject to the FCC Title 47 ruling, the regional requirement is proposed in [] until final conclusion is reached in the emissions scaling consideration across regions; - For the unwanted emissions, Category A and Category B limits are recognized as regional requirements; - Further regional requirements are not precluded. The above proposal is to avoid the case where the additional regional requirements must be considered in all core requirement’s sections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.5: inputs for the regional requirements table collected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of regional requirements in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Inputs from R4-1800284 were considred in the revised version. |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 4.5	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.5-1: List of regional requirements

| Subclause number(s) | Requirement | Comments |
| --- | --- | --- |
| 5.2 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.6.2 9.7.2 | Occupied bandwidth, OTA occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 6.6.3.4, 9.7.3.2 | Absolute ACLR, OTA absolute ACLR, | The emission limits specified as the basic limit + X [dB] is applicable, unless stated differently in regional regulation. |
| 6.6.4.2.5.1 | Limits in FCC Title 47 | The BS may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.4.4 9.7.4.2 | Operating band unwanted emissions, OTA out-of-band emissions | The emission limits specified as the basic limit + X [dB] is applicable, unless stated differently in regional regulation. |
| 6.6.5.2.1 | Tx spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [2], may apply regionally. |
| 6.6.5.2.3 9.7.5.3.3 | Tx spurious emissions: additional requirements OTA Tx spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the BS operating band. |
| 7.6.4, 10.7.2 | Rx spurious emissions, OTA Rx spurious emissions | The emission limits specified as the basic limit + X [dB] is applicable, unless stated differently in regional regulation. |
| 9.7.5.2 | OTA Tx spurious emissions | The emission limits specified as the basic limit + X [dB] is applicable, unless stated differently in regional regulation. |

------------------------------ End of modified sections ------------------------------