3GPP TSG-RAN WG4 #AH-1807 Meeting                                R4-1808618

Montreal, Canada, 2 to 6 July, 2018

Agenda item:	5.4.2.3

Source: 	ZTE

Title: 	On NR BS PUCCH demodulation performance requirements

Document for:	Discussion

Introduction

During RAN4#87 meeting in May two WFs [1][2] relevant for NR BS PUCCH demodulation performance requirements and simulation assumptions for alignment purpose have been approved. This contribution focuses on the initial simulation results of SNR at which the ACK missed detection probability does not exceed 1% for FR1 conducted single user PUCCH formats without DTX detection.

Initial simulation assumptions and results for FR1 conducted

Here are the simulation assumptions for FR1 conducted:

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FR1 |  |  |
| Number of Tx | 1 |  |  |
| Number of Rx | 2, 4, 8 |  |  |
| Carrier frequency (GHz) | 4 |  |  |
| Propagation channel | AWGN |  |  |
| Waveform | CP-OFDM |  |  |
| SCS@CBW | 30 kHz @ 20&40 MHz |  |  |
| Modulation | QPSK |  |  |
| Cyclic prefix | Normal |  |  |

The initial simulation results of SNR at which the ACK missed detection probability does not exceed 1% are summarized in table 1 below for FR1 conducted single user PUCCH format 0: 1Tx, 1 OFDM symbol, without DTX detection.

Table 1 Initial simulation results for FR1 conducted single user PUCCH format 0:

1Tx, 1 OFDM symbol, without DTX detection

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 30 kHz@20 MHz | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  | -1.6 | -1.6 |  |  |  |
|  | 4 | Normal |  | AWGN |  | -3.5 | -3.5 |  |  |  |
|  | 8 | Normal | AWGN |  |  | -5.5 | -5.5 |  |  |  |

The initial simulation results of SNR at which the ACK missed detection probability does not exceed 1% are summarized in table 2 below for FR1 conducted single user PUCCH format 1: 1Tx, 14 OFDM symbol, without DTX detection.

Table 2 Simulation results for FR1 conducted single user PUCCH format 1:

1Tx, 14 OFDM symbol, without DTX detection

| Number of Tx | Number of Rx | Cyclic Prefix |  | Propagation conditions | SCS@Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 30 kHz@20 MHz | 30 kHz@40 MHz |  |  |  |
| 1 | 2 | Normal |  | AWGN |  | -9.0 | -9.0 |  |  |  |
|  | 4 | Normal |  | AWGN |  | -12.2 | -12.2 |  |  |  |
|  | 8 | Normal | AWGN |  |  | -14.4 | -14.4 |  |  |  |

From the above simulation results we have the following observation and proposal:

Observation 1: For FR1 conducted single user PUCCH formats 0 and 1, for SCS 30 kHz the SNR simulation results are the same for different channel bandwidths due to short PUCCH information.

Proposal 1: In Rel-15 defining the PUCCH requirements only for one moderate channel bandwidth is enough for a certain SCS to reduce the specification and test efforts. FFS the exact moderate channel bandwidth from 20, 40 or 50 MHz for SCS 30 kHz.

Conclusions

Based on the discussions above, we have the following observation and proposal:

Observation 1: For FR1 conducted single user PUCCH formats 0 and 1, for SCS 30 kHz the SNR simulation results are the same for different channel bandwidths due to short PUCCH information.

Proposal 1: In Rel-15 defining the PUCCH requirements only for one moderate channel bandwidth is enough for a certain SCS to reduce the specification and test efforts. FFS the exact moderate channel bandwidth from 20, 40 or 50 MHz for SCS 30 kHz.

# References

- R4-1808022 Way forward on General part of NR BS demodulation performance, May 2018.

- R4-1808025 Way forward on NR PUCCH demodulation performance in Rel-15.