3GPP TSG-RAN WG4 #AH-1807 Meeting                                R4-1808619

Montreal, Canada, 2 to 6 July, 2018

Agenda item:	5.4.2.4

Source: 	ZTE

Title: 	On NR BS PRACH performance requirements

Document for:	Discussion

Introduction

During RAN4#87 meeting in May two WFs [1][2] relevant for NR PRACH performance requirements and simulation assumptions for alignment purpose have been approved. This contribution focuses on the initial simulation results of SNR for 1% missed preamble detection probability for FR1 conducted case.

Simulation parameters and initial results for FR1 conducted

Here are the simulation parameters for FR1 conducted:

| Parameter (FR1 conducted) | Value |
| --- | --- |
| Format/SCS/CBW combinations | Format 0: SCS 1.25 kHz Format A3: SCS 15 & 30 kHz Format B4: SCS 15 & 30 kHz Format C2: SCS 15 & 30 kHz |
| Tx | 1 |
| Rx | Conducted: 2, 4 and 8Rx |
| Channel condition | AWGN |
| Frequency offset | 0 |
| NCS | Format 0: Ncs = 13 Format A3: Ncs = 12 Format B4: Ncs = 12 Format C2: Ncs = 12 |

The initial simulation results of SNR for missed detection probability < 1% for FR1 conducted are summarized in Table 1 below. It could be observed that for AWGN channel the simulation results are the same for SCS 15 kHz and SCS 30 kHz for the same preamble format.

Observation 1: For FR1 conducted with AWGN channel, the simulation results of SNR for missed detection probability < 1% are the same for SCS 15 kHz and SCS 30 kHz for the same preamble format.

Table 1: Simulation results for FR1 conducted: SNR (dB) for missed detection probability < 1%

| Number of TX | Number of RX | Propagation conditions | Frequency offset | SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Format 0 (SCS 1.25 kHz， NCS 13) | Format A3 (SCS 15 or 30 kHz， NCS 12) | Format B4 (SCS 15 or 30 kHz，NCS 12) | Format C2 (SCS 15 or 30 kHz，NCS 12) |  |
| 1 | 2 | AWGN | 0 | -13.6 | -13.5 | -16.5 | -11.7 |  |
|  | 4 | AWGN | 0 | -16.1 | -15.9 | -18.9 | -14.3 |  |
|  | 8 | AWGN | 0 | -18 | -18 | -21 | -16.2 |  |

Conclusions

Based on the simulation results above, we have the following observation:

Observation 1: For FR1 conducted with AWGN channel, the simulation results of SNR for missed detection probability < 1% are the same for SCS 15 kHz and SCS 30 kHz for the same preamble format.

# References

- R4-1808022 Way forward on General part of NR BS demodulation performance, May 2018.

- R4-1808026 Way forward on NR PRACH demodulation requirements, May 2018.