TSG-RAN Working Group 4 (Radio) meeting AH1807  	R4-1808644

Montreal, Canada, 2nd July – 6th July 2018

Source:	Ericsson

Title:	On measurement uncertainty for OTA Adjacent channel selectivity, in-channel selectivity, narrowband blocking, In-Band Blocking, RX Intermodulation for eAAS

Agenda item:	4.2.1.1.3.1

Document for:	Discussion

# Introduction

This contribution presents a further discussion for measurement uncertainty calculation of OTA Adjacent channel selectivity in sub-clause 4.1.2.3 of TS 37.145-2[3]. This contribution is based on R4-1807086 [9] presented in RAN4 meeting #87 and the WF R4-1808366 [11] agreed in the same meeting. We are glad to encourage feedback from other companies on this topic.

# Discussion

According to the R4-1808366 [11], we can combinate the uncertainty factors at the receiver as following formula (taking ACS as an example):

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect         (2-1)

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2                                                             (2-2)

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2                                                          (2-3)

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2                                                 (2-4)

We can discuss these factors one by one.

- MUwanted_conducted  and MUinterfer_conducted

We can use the MU values in the TS 36.141[6] because the setup in conducted test is the same as it in OTA test. The results are listed in the Table 2.1.

Table 2.1  measurement uncertainty values for the conducted ACS test

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |
| --- | --- | --- | --- |
| ICS, ACS and NBB | MUwanted_conducted 1.96σ | ±0.7 dB | ±1.0 dB |
|  | MUinterfer_conducted 1.96σ | ±0.7 dB | ±1.0 dB |
| IBB | MUwanted_conducted 1.96σ | ±0.7 dB | ±1.0 dB |
|  | MUinterfer_conducted 1.96σ | ±1.0 dB | ±1.2 dB |
| RX IM | MUwanted_conducted 1.96σ | ±0.7 dB | ±1.0 dB |
|  | MUCWinterfer_conducted 1.96σ | ±0.5 dB | ±0.7 dB |
|  | MUMODinterfer_conducted 1.96σ | ±0.7 dB | ±1.0 dB |

- MUEIS_OTA

We take the OTA reference sensitivity calculation budget table and values in TS 37.842[2]. The results are listed in the Table 2.2 below.

## Table 2.2   measurement uncertainty values for the OTA EIS test

| Frequency range |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- |
| Indoor Anechoic Chamber | Expanded uncertainty 1.96σ | 1.22 dB | 1.25 dB |
| Compact Antenna Test Range | Expanded uncertainty 1.96σ | 1.33 dB | 1.40 dB |
| MUEIS_OTA 1.96σ |  | 1.3 dB | 1.4 dB |

- MUmatching_conducted and MUmatching_OTA

We assume the MUmatching_conducted= MUmatching_OTA because the MUmatching_conducted is not quite clear in specifications. The MUmatching_OTA can take the relevant contributions from the budget table of OTA EIS in TS 37.842 [2], and then multiply 1.96 with it to get expand uncertainty. The calculations are listed as follows:

## Table 2.3 Indoor Anechoic Chamber uncertainty assessment for matching part in ACS

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Impedance mismatch in the transmitting chain | 0.14 | 0.23 | U-shaped | 1.41 | 1 | 0.10 | 0.16 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.1 | 0.16 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.196 | 0.314 |

## Table 2.4 CATR uncertainty assessment for matching part in ACS

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.15 | 0.15 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.294 | 0.294 |

- MUTestEquipment_OTA

The test equipment part can also take the relevant contribution from the budget table of OTA EIS in TS 37.842 [2], and then multiply 1.96 with it to get expand uncertainty. The calculations are listed as follows:

## Table 2.5 Indoor Anechoic Chamber uncertainty assessment for test equipment in ACS

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.46 | 0.46 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.9 | 0.9 |

## Table 2.6 CATR uncertainty assessment for test equipment in ACS

| UID | Uncertainty source | f ≦ 3GHz | 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.46 | 0.46 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.9 | 0.9 |

- ACLReffect

Estimate ACLR effect according to degradation assumption. We can take 0.4dB as an acceptable value for ACLR effect in FR1 OTA test.

Now we put all results above into formula (2-1) and get (Note that all uncertainties in table are 1.96σ values)

## Table 2.7 Indoor Anechoic Chamber Measurement Uncertainty for OTA receiver tests

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| ICS, ACS and narrow band blocking | MUwanted_conducted | ±0.7 dB | ±1 dB |  |
|  | MUinterfer_conducted | ±0.7 dB | ±1 dB |  |
|  | MUmatching_conducted | ±0.196 dB | ±0.314 dB |  |
|  | MUwanted | ±0.672 dB | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.672 dB | ±0.949 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.22 dB | 1.25 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.196 dB | ±0.314 dB |  |
|  | MUchamber_OTA | ±0.8 dB | ±0.807 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA | 1.66 dB | 2 dB |  |

| IBB | MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| --- | --- | --- | --- | --- |
|  | MUinterfer_conducted | ±1.0 dB | ±1.2 dB |  |
|  | MUmatching_conducted | ±0.196 dB | ±0.314 dB |  |
|  | MUwanted | ±0.672dB | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.981 dB | ±1.158 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.22 dB | 1.25 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.196 dB | ±0.314 dB |  |
|  | MUchamber_OTA | ±0.8 dB | ±0.807 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  |  |
|  | MUIBB_OTA | 1.85 dB | 2.13 dB | MUIBB_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |

| RX IM | MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| --- | --- | --- | --- | --- |
|  | MUCWinterfer_conducted | ±0.5 dB | ±0.7 dB |  |
|  | MUMODinterfer_conducted | ±0.7 dB | ±1.0 dB |  |
|  | MUmatching_conducted | ±0.196 dB | ±0.314 dB |  |
|  | MUwanted | ±0.672dB | ±0.949 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUCWinterfer | ±0.460 dB | ±0.626 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUMODinterfer | ±0.672dB | ±0.949 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.22 dB | 1.25 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.196 dB | ±0.314 dB |  |
|  | MUchamber_OTA | ±0.8 dB | ±0.804 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  |  |
|  | MURXIM_OTA | 1.96 dB | 2.43 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

## Table 2.8 CATR Measurement Uncertainty for OTA receiver tests

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| ACS and narrow band blocking | MUwanted_conducted | ±0.7 dB | ±1 dB |  |
|  | MUinterfer_conducted | ±0.7 dB | ±1 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.635 dB | ±0.956 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA 1.96σ | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA | 1.73 dB | 2.12 dB |  |

| IBB | MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| --- | --- | --- | --- | --- |
|  | MUinterfer_conducted | ±1.0 dB | ±1.2 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635 dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.956 dB | ±1.163 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  |  |
|  | MUIBB_OTA | 1.91 dB | 2.25 dB | MUIBB_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |

| RX IM | MUwanted_conducted | ±0.7 dB | ±1.0 dB |  |
| --- | --- | --- | --- | --- |
|  | MUCWinterfer_conducted | ±0.5 dB | ±0.7 dB |  |
|  | MUMODinterfer_conducted | ±0.7 dB | ±1.0 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635 dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUCWinterfer | ±0.404 dB | ±0.635 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUMODinterfer | ±0.635 dB | ±0.956 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  |  |
|  | MURXIM_OTA | 1.95 dB | 2.54 dB | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

- Consideration of PA

Based on the analysis in our paper [10], we might not need PA during OTA ACS and NBB measurement for eAAS in FR1.

# Conclusion

.	The measurement uncertainty of  OTA ACS and NBB test for eAAS in FR1 can use the values in Table 3.1, IBB in table 3.2 and RX IM in table 3.3.

## Table 3.1   measurement uncertainty values for the OTA ICS, ACS and NBB test for eAAS in FR1

| Frequency range |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- |
| Indoor Anechoic Chamber | Expanded uncertainty 1.96σ | 1.66 dB | 2 dB |
| Compact Antenna Test Range | Expanded uncertainty 1.96σ | 1.73 dB | 2.12 dB |

## Table 3.2   measurement uncertainty values for the OTA IBB test for eAAS in FR1

| Frequency range |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- |
| Indoor Anechoic Chamber | Expanded uncertainty 1.96σ | 1.85 dB | 2.13 dB |
| Compact Antenna Test Range | Expanded uncertainty 1.96σ | 1.91 dB | 2.25 dB |

## Table 3.3   measurement uncertainty values for the OTA RX IM test for eAAS in FR1

| Frequency range |  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- |
| Indoor Anechoic Chamber | Expanded uncertainty 1.96σ | 1.96 dB | 2.43 dB |
| Compact Antenna Test Range | Expanded uncertainty 1.96σ | 1.95 dB | 2.54 dB |

# References

[1]	3GPP TS 37.105 V15.0.0 (2017-12)

[2]	3GPP TR 37.842 V15.0.0 (2017-12)

[3]	3GPP TS 37.145-2 V14.3.0 (2017-12)

[4]	3GPP TS 37.145-1 V14.2.0 (2017-09)

[5]	3GPP TS 37.104 V15.0.0 (2017-09)

[6]	3GPP TS 36.141 V15.0.0 (2017-09)

[7]	R4-1805154, “RX directional requirements - MU estimations”

[8]	R4-1805349, “Draft CR to TR 37.843 Indoor anechoic chamber test method MU for ACS and NBB”

[9]	R$-1807076, “On measurement uncertainty for OTA Adjacent channel selectivity and narrowband blocking for eAAS”，Ericsson

[10]	R4-18xxyyzz,  “Necessity of PA for OTA Adjacent channel selectivity and narrowband blocking measurement for eAAS”, Ericsson

[11]       R4-1808366, “ WF on MU and TT for receiver requirements”, Ericsson