3GPP TSG- RAN4 Meeting AH#2	R4-1808656

# Montreal, Canada, 2nd to 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR37.843 Compact Antenna Test Range for OTA adjacent channel selectivity, narrow-band blocking, and general blocking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The text capturing detailed procedures in CATR for adjacent channel selectivity, narrow band blocking and in-band blocking. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All changes highlighted in yellow. Conformance test procedure and MU budget are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Detailed test procedure descriptions not fully clear and correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.3.5.3	CATR

##### 10.3.5.3.1	General

This method measures the adjacent channel selectivity, narrow-band blocking, and general blocking in compact antenna test range with a sufficient separation between reference point of the AAS BS and the transmitting antenna by the range antenna reflector. The measurement system setup is as depicted in figure 10.3.5.3.1-1.

##### Figure 10.3.5.3.1-1: CATR measurement system setup for adjacent channel selectivity, general blocking and narrowband blocking

##### 10.3.5.3.2 	Calibration

Calibration shall be done with the procedure shown in 10.3.2.3.2 for the frequencies of wanted and interfering signals with power combiner/coupler. Calculate the calibration value for wanted and interfering signal from A to B as well as from A to E.

LWanted_cal, A→B:  Calibration value for wanted signal between A and B in figure 10.3.5.3.1-1.

Linterferer_cal, A→E:  Calibration value for interfering signal between A and E in figure 10.3.5.3.1-1.

##### 10.3.5.3.3 	Procedure

- Set up AAS BS in place of SGH from calibration stage. Align AAS BS with boresight of transmitting antenna.

- Configure RF signal generator for wanted signal to set the signal at the required level plus LA→B and at reference measurement channel.

- Configure RF signal generator for interferer to set the signal at the required level plus LA→E and at reference measurement channel. The interferer’s center frequency offset should be swept as required.

- Measure the throughput of the wanted signal according to annex E of the 3GPP TS 36.141 [??].

- Repeat steps 2-4 for all conformance test directions.

- Repeat steps above per involved band for multi-band RIB(s).

##### 10.3.5.3.4 	MU assessment

###### 10.3.5.3.4.1 	MU Budget

The MU is estimated based on MU in the conducted specification 36.141, the MU for EIS in section 10.3.3 and an estimated uncertainty for matching of test equipment and DUT or test antenna.

Table 10.3.5.3.4.1-1: CATR uncertainty budget format for mismatch error

| UID | Description of uncertainty contribution | Details in anex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 2 | Standing wave between DUT and test range antenna | C2-2 |

###### 10.3.5.3.4.2 	MU Value

# Table 10.3.5.3.4.2-1: CATR uncertainty assessment for OTA mismatching error

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.15 | 0.15 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 0.294 | 0.294 |

Test system uncertainty is obtained by follwing equations:

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

Where

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

And we assume  MUmatching_conducted= MUmatching_OTA

Test system uncertainty for ACS, narrow band blocking, and general band blocking is shown in Table 10.3.5.3.4.2-4. Note that all uncertainties in table are 1.96σ values.

Table 10.3.5.3.4.2-3 Test system uncertainty of ACS, narrow band blocking, and general band blocking for CATR.

| Frequency range |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz |  |
| --- | --- | --- | --- | --- |
| ACS and narrow band blocking | MUwanted_conducted | ±0.7 dB | ±1 dB |  |
|  | MUinterfer_conducted | ±0.7 dB | ±1 dB |  |
|  | MUmatching_conducted | ±0.294 dB | ±0.294 dB |  |
|  | MUwanted | ±0.635dB | ±0.956 dB | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer | ±0.635 dB | ±0.956 dB | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA | 1.33 dB | 1.40 dB |  |
|  | MUTestEquipiment_OTA | 0.9 dB | 0.9 dB |  |
|  | MUmatching_OTA | ±0.294 dB | ±0.294 dB |  |
|  | MUchamber_OTA | ±0.933 dB | ±1.03 dB | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | ACLR effect | 0.4 dB |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA | 1.73 dB | 2.12 dB |  |

#### 10.3.5.x	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}