3GPP TSG- RAN WG4 Meeting AH-1807 LTE/NR perf 	R4-1808676

# Montreal, Canada, 2th – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.812-02: Measurement step size for BS type 2-O out-of-band blocking conformance testing (new clause 10.6.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The measurement step size of NR BS type 2-O out-of-band blocking conformance testing was agreed in RAN4#87, while the corresponding details in TR 37.812-02 have yet to be filled in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fill in the details for the measurement step size of NR BS type 2-O out-of-band blocking conformance testing in TR 37.812-02, based on the agreement in RAN4#87, and a small refinement in the agreed measurement step size for 400MHz BS channel bandwidth of wanted signal. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details for the measurement step size of NR BS type 2-O out-of-band blocking conformance testing in TR 37.812-02 remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 10.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 10.6.2	FR2 OTA out-of-band blocking

The BS out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal.

To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal at frequency above 6 GHz should be set as shown in Table 10.6.2-1 in the NR BS type 2-O OTA out-of-band blocking conformance testing.

Table 10.6.2-1: Measurement step size for out-of-band blocking conformance testing

| BS Channel bandwidth of wanted signal [MHz] | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
| Measurement step size for interfering signal [MHz] | 15 | 30 | 60 | 120 |

<End of change>