3GPP TSG-RAN WG4 Meeting AH1807	R4-1808812

Montreal, Canada, 2 – 6 July 2018

Agenda Item:	4.1.2

Source:	Ericsson

Title:	Update of simulation assumption for sTTI UE demodulation and CSI requirements

Document for:	Discussion

# 1	General

## 1.1	Reference receiver

LMMSE receiver is baseline receiver for UE demodulation and CSI requirements for both subslot-based and slot-based frame structure. Note the simulation assumption does not model the interfering cells.

# 2	UE demodulation requirements

## 2.1	Simulation assumption for SPDCCH

### 2.1.1	Test parameters

| Test number | Test 1 | Test 2 |
| --- | --- | --- |
| Description | CRS-based, Localized | DMRS-based, Distributed |
| Bandwidth | 10MHz | 10MHz |
| Number of PDCCH symbols (CFI) | 2 | 2 |
| Cyclic prefix | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN |
| Propagation condition | EVA30 | EPA5 |
| Antenna configuration | 4x2 Low | 2x2 Low |
| Antenna ports | 0-3 | 107 |
| Precoder update granularity | N/A | 2 SREGs every 1ms |
| Beamforming precoder | N/A | Random precoder (TS36.101 B.4.4) (Note 4) |
| Short TTI length (Note 1) | Slot-based / subslot-based | Slot-based / Subslot-based |
| Configured SPDCCH-PRB-sets | {Set 1} | {Set 2} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 | Set 2 |
| Aggregation level | 2 SCCE | 8 SCCE |
| DCI format | 7-1C | 7-1F |
| Number of bits for DCI (without CRC) | 32 (FDD) 34 (TDD) | 28 (FDD) 30 (TDD) |
| PDSCH TM | TM4 | TM9 |
| Duplex modes (Note 2) | FDD / TDD | FDD / TDD |
| Evaluation criteria (Note 5) | 1% Pm-dsg | 1% Pm-dsg |
| Note 1: Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based SPDCCH demodulation only. Note 3: For slot-based transmission, DCI 7-1x is transmitted only on SPDCCH in slot 1. For subslot-based transmission, DCI 7-1x is transmitted on SPDCCH in all the subslots. Note 4: No CSI feedback is necessary for simulation. Note 5: Pm-dsg for SPDCCH is calculated acorss all the short TTIs. |  |  |

### 2.1.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 |
| --- | --- | --- |
| Reference symbol | CRS | DMRS |
| Transmission type | Localized | Distributed |
| Number of OFDM symbols | 1 | 2 |
| Number of PRBs | 16 | 16 |
| Allocated PRBs | {0, 1, … 15} | {1, 2, 7, 8, 13, 14, 19, 20, 29, 30, 35, 36, 41, 42, 47, 48} |
| Total number of SCCEs | 4 (16 SREGs) | 8 (32 SREGs) |

## 2.2	Simulation assumption for Slot-PDSCH/Subslot-PDSCH

### 2.2.1	Minimum processing time

- Subslot-based transmission:

  - UE can choose set1 with n+4 or set 2 with n+6 according to UE capability.

- Slot-based transmission:

  - Minimum processing time for: n+4

### 2.3.1	Test parameters

|  | Test 1 | Test 2 |
| --- | --- | --- |
| Bandwidth | 10MHz | 10MHz |
| Number of PDCCH symbols (CFI) | 2 | 2 |
| Cyclic prefix | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN |
| DMRS sharing (applicable only for subslot TTI) | N/A | Enabled (Note 4) |
| Propagation channel | EVA30 | EPA5 |
| Antenna configuration | 4x2 Low | 2x2 Low |
| Transmission modes | TM3 dual layer | TM9 dual layer |
| Antenna ports | 0-3 | 7 and 8 for dual layer |
| Precoder update granularity | N/A | 1 PRG every 1ms |
| Beamforming precoder | N/A | Random precoder (TS36.101 B.4.1 for single layer, B.4.2 for dual layer) (Note 5) |
| TTI length (Note 1) | Slot-based / Subslot-based | Slot-based / Subslot-based |
| MCS | 16QAM 1/2 | QPSK 1/3 |
| Duplex mode (Note 2) | FDD / TDD | FDD / TDD |
| Evaluation criteria | 70% of Max Tput | 70% of Max Tput |
| CSI-RS signals | N/A | Antenna ports 15, 16, 17, 18 |
| CSI-RS periodicity and subframe offset | N/A | 5/ 2 |
| CSI-RS configuration | N/A | 0 |
| ZP-CSI-RS configuration | N/A | Not configured |
| v_shift (mod(cell_ID, 6)) | 0 | 0 |
| Associated SPDCCH transmission | CRS-based, Localized | CRS-based, Localized |
| Aggregation level | [2 SCCE] | [4 SCCE] |
| Used/Unused SPDCCH resource indication (Note 6) | ’10’ | ’10’ |
| DCI format | 7-1B | 7-1F |
| Configured SPDCCH-PRB-sets | {Set 1, Set 2} | {Set 3, Set 4} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 for slot-based Set 2 for subslot-based | Set 3 for slot-based Set 4 for subslot-based |
| Note 1: sTTI PDSCH demodulation requirements should be specified for both slot-based and subslot-based. Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based PDSCH demodulation only. Note 3: Void. Note 4: DMRS is transmitted in subslot #1, #3, and #5. DMRS sharing is enabled in subslots #2 and #4. Note 5: No CSI feedback is necessary for simulation. Note 6: Set when SPDCCH L1 Reuse Indication is configured. Otherwise it is not signaled. |  |  |

### 2.3.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 | Set 3 | Set 4 |
| --- | --- | --- | --- | --- |
| Reference symbol | CRS | CRS | CRS | CRS |
| Transmission type | Localized | Localized | Localized | Localized |
| Number of OFDM symbols | 1 | 1 | 2 | 2 |
| Number of PRBs | 16 | 16 | 8 | 8 |
| Allocated PRBs | {0, 1, …, 15} | {34, 35, …, 49} | {0, 1, …, 7} | {8, 9, …, 15} |
| Rate matching mode | Mode 1 | Not configured | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured | {2, 0} | {1, 1} | Not configured |
| Total number of SCCEs | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) |

### 2.4.3	FRC tables (For information)

#### 2.4.3.1	CRS-based PDSCH

##### 2.4.3.1.1	Slot-based transmission

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | TBD |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot (two codewords) |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  |  |
| Slot index 0 | Bits | 9912 |
| Slot index 1 | Bits | 12576 |
| Binary channel bits per slot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 11200 |
| Slot index 1 | Bits | 14114 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  |  |
| Slot index 0 | Bits | 10264 |
| Slot index 1 | Bits | 13184 |
| Note: Given per component carrier |  |  |

##### 2.4.3.1.2	Subslot-based transmission

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 8 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslot (Two codewords) |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |
| Subslot index 5 | Bits | TBD |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  |  |
| Subslot index 1 | Bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |
| Subslot index 5 | Bits | TBD |
| Binary channel bits per subslot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 6016 |
| Subslot index 2 | Bits | 4416 |
| Subslot index 3 | Bits | 2944 |
| Subslot index 4 | Bits | 4416 |
| Subslot index 5 | Bits | 6144 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frame 0 |  |  |
| Subslot index 1 | Bits | 6016 |
| Subslot index 2 | Bits | 3840 |
| Subslot index 3 | Bits | 2560 |
| Subslot index 4 | Bits | 3840 |
| Subslot index 5 | Bits | 6144 |
| Note: Given per component carrier |  |  |

#### 2.4.3.2	DMRS-based PDSCH

##### 2.4.3.2.1	Slot-based transmission

|  | Unit | Test 2 (Ref. R.66) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Number of HARQ processes |  | [8] |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot (One codeword) |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 1800 |
| Slot index 1 | Bits | 2216 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 1416 |
| Slot index 1 | Bits | 2216 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frames 0 (RB0–RB20 and RB30–RB49 => 41RBs) |  |  |
| Slot index 0 | Bits | 1480 |
| Slot index 1 | Bits | 1800 |
| Binary channel bits per slot |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 5000 |
| Slot index 1 | Bits | 6680 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 4800 |
| Slot index 1 | Bits | 6680 |
| For Sub-Frame 5 |  | N/A |
| For Sub-Frames 0 (RB0–RB20 and RB30–RB49 => 41RBs) |  |  |
| Slot index 0 | Bits | 4100 |
| Slot index 1 | Bits | 5420 |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier |  |  |

##### 2.4.3.2.2	Subslot-based transmission

|  | Unit | Test 2 (Ref. R.44) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | FFS |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots (One codeword) |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 744 |
| Subslot index 2 | bits | 600 |
| Subslot index 3 | Bits | 376 |
| Subslot index 4 | Bits | 600 |
| Subslot index 5 | Bits | 872 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 744 |
| Subslot index 2 | bits | 600 |
| Subslot index 3 | Bits | 376 |
| Subslot index 4 | Bits | 600 |
| Subslot index 5 | Bits | 872 |
| For Sub-Frame 5 |  |  |
| For Sub-Frames 0 (41PRB) |  |  |
| Subslot index 1 | Bits | 600 |
| Subslot index 2 | bits | 504 |
| Subslot index 3 | Bits | 296 |
| Subslot index 4 | Bits | 504 |
| Subslot index 5 | Bits | 600 |
| Binary channel bits per subslots |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 2480 |
| Subslot index 2 | bits | 2016 |
| Subslot index 3 | Bits | 1344 |
| Subslot index 4 | Bits | 2016 |
| Subslot index 5 | Bits | 2544 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 2480 |
| Subslot index 2 | bits | 1848 |
| Subslot index 3 | Bits | 1344 |
| Subslot index 4 | Bits | 2016 |
| Subslot index 5 | Bits | 2544 |
| For Sub-FFrame 5 |  | N/A |
| For Sub-Frames 0 (41PRB) |  |  |
| Subslot index 1 | Bits | 1976 |
| Subslot index 2 | bits | 1584 |
| Subslot index 3 | Bits | 1056 |
| Subslot index 4 | Bits | 1584 |
| Subslot index 5 | Bits | 2040 |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier |  |  |

# 3	CSI reporting test

## 3.1	Minimum processing time

- Subslot-based transmission:

  - UE can choose set1 with n+4 or set 2 with n+6 according to UE capability.

- Slot-based transmission:

  - Minimum processing time for: n+4

## 3.2	CQI reporting under fading conditions with PUSCH

### 3.2.1	Aperiodic reporting mode with cell-specific reference signals

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02]

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2x2) |
| Transmission modes | TM4 |
| CodeBookSubsetRestrction bitmap | 000001 |
| Reporting interval | 5ms |
| CQI delay | n+4 slots for slot-based n+4 or n+6 subslots for subslot-based |
| Reporting mode | PUSCH 1-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD (slot-based only) |

SPDCCH parameters for Wideband CQI test.

|  | SPDCCH parameters |
| --- | --- |
| Reference symbol | CRS |
| Transmission type | Localized |
| Number of OFDM symbols | 1 |
| Number of PRBs | 16 |
| Allocated PRBs | {0, 1, … 15} |
| Rate matching mode | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured |
| Total number of SCCEs | 4 (16 SREGs) |

CQI-to-MCS tables used for aperiodic reporting mode with cell-specific reference signals

|  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 | Slot 1 | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 2800 | 3672 | 1408 | 1008 | 872 | 1008 | 1472 |
| CQI index | Code rate | Modulation |  |  |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 2 | 3 | 3 | 2 | 1 | 2 | 3 |
| 4 | 0.3 | 2 | 4 | 5 | 6 | 4 | 3 | 4 | 6 |
| 5 | 0.44 | 2 | 6 | 7 | 8 | 6 | 5 | 6 | 9 |
| 6 | 0.59 | 2 | 7 | 9 | 9 | 8 | 7 | 8 | 9 |
| 7 | 0.37 | 4 | 10 | 13 | 14 | 11 | 10 | 11 | 14 |
| 8 | 0.48 | 4 | 12 | 15 | 16 | 13 | 12 | 13 | 16 |
| 9 | 0.6 | 4 | 14 | 16 | 16 | 15 | 14 | 15 | 16 |
| 10 | 0.46 | 6 | 17 | 20 | 22 | 18 | 17 | 18 | 22 |
| 11 | 0.55 | 6 | 19 | 23 | 24 | 20 | 18 | 20 | 25 |
| 12 | 0.65 | 6 | 21 | 25 | 27 | 22 | 20 | 22 | 27 |
| 13 | 0.75 | 6 | 23 | 27 | 28 | 24 | 22 | 24 | 28 |
| 14 | 0.85 | 6 | 25 | 28 | 28 | 26 | 24 | 26 | 28 |
| 15 | 0.93 | 6 | 26 | 28 | 28 | 27 | 25 | 27 | 28 |

### 3.2.2	Aperiodic reporting mode with CSI reference signals

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02]

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2 x 2) |
| Transmission modes | TM9 |
| Beamforming model | TS 36.101 B.4.3 |
| CRS reference signals | Antenna port 0 |
| CSI reference signals | Antenna ports 15 and 16 |
| CSI-RS periodicity and subframe offset | 5ms / 1 |
| CSI-RS reference signal configuration | 0 |
| CodeBookSubsetRestrction bitmap | 000001 |
| Reporting interval | 5ms |
| CQI delay | n+4 slots for slot-based n+4 or n+6 subslots for subslot-based |
| Reporting mode | PUSCH 1-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD (slot-based only) |

SPDCCH parameters for Wideband CQI test.

|  | SPDCCH parameters |
| --- | --- |
| Reference symbol | DMRS |
| Transmission type | Localized |
| Number of OFDM symbols | 2 |
| Number of PRBs | 16 |
| Allocated PRBs | {0, 1, … 15} |
| Rate matching mode | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured |
| Total number of SCCEs | 8 (32 SREGs) |

CQI-to-MCS tables used for aperiodic reporting mode with CSI reference signals

|  |  |  | Non CSI-RS |  | 2 CRS-RS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 | Slot 1 | Slot 0 | Slot 1 |
|  |  | PRB | 50 | 50 | 50 | 50 |
|  |  | Available REs | 2600 | 3348 | 2500 | 3348 |
| CQI index | Code rate | Modulation |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 |
| 3 | 0.19 | 2 | 1 | 2 | 1 | 2 |
| 4 | 0.3 | 2 | 3 | 4 | 3 | 4 |
| 5 | 0.44 | 2 | 5 | 7 | 5 | 7 |
| 6 | 0.59 | 2 | 7 | 9 | 7 | 9 |
| 7 | 0.37 | 4 | 10 | 12 | 10 | 12 |
| 8 | 0.48 | 4 | 12 | 14 | 12 | 14 |
| 9 | 0.6 | 4 | 14 | 16 | 13 | 16 |
| 10 | 0.46 | 6 | 17 | 19 | 17 | 19 |
| 11 | 0.55 | 6 | 18 | 21 | 18 | 21 |
| 12 | 0.65 | 6 | 20 | 23 | 20 | 23 |
| 13 | 0.75 | 6 | 22 | 26 | 22 | 26 |
| 14 | 0.85 | 6 | 24 | 27 | 23 | 27 |
| 15 | 0.93 | 6 | 25 | 28 | 24 | 28 |

# References

- R4-1803103, “Way forward on sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1805560, “Way forward on sTTI DMRS pattern and CQI2MCS mapping table”, Huawei, Ericsson, Qualcomm

- R4-1808475, “Way forward on sTTI UE demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

# Information

## sTTI slot-index and subslot-index

Figure 1 Slot index for slot-based PDSCH and Subslot index for the subslot-based OFDM symbols.

## DCI Payload size (without CRC)

| Parameter | Format 7-1C | Format 7-1F | Notes |
| --- | --- | --- | --- |
| UL/DL differentiation flag | 1 | 1 |  |
| Resource block assignment | 8 | 8 | PDSCH allocation type 0 with 10MHz (RBG=6) |
| Modulation and coding scheme | 5 | 5 |  |
| HARQ process number | 4 | 4 |  |
| New data indicator | 1 | 1 |  |
| Redundancy version | 2 | 2 |  |
| TPC command for PUCCH | 2 | 2 |  |
| DMRS position indicator | 0 | 0 | DMRS sharing |
| Downlink assignment index | 0 (FDD) 2 (TDD) | 0 (FDD) 2 (TDD) |  |
| SRS request | 0 | 0 | SRS triggering not configured |
| Used/Unused SPDCCH resource indication | 0 | 0 | L1-based reuse indication not configured |
| SPUCCH resource indication | 2 | 2 |  |
| Precoding information | 4 (2 CRS ports) 7 (4 CRS ports) | N/A |  |
| Antenna port(s), scrambling identity and number of layers | N/A | 3 |  |
| Total (bits) | 29 (FDD, 2 ports) 32 (FDD, 4 ports) 31 (TDD, 2 ports) 34 (TDD, 4 ports) | 28 (FDD) 30 (TDD) |  |

## RRC-based rate-matching mode

Figure 2	RRC-based SPDCCH rate matching mode.

## L1-based rate-matching mode

Figure 3	L1-based SPDCCH rate matching mode.