3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf	R4-1808895

# Montreal, Canada, 2th – 6th July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.145-2: adding TRP measurement grids (Annex F) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-xx |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no TRP measurement grid in the conformance test specification for measuring radiated power of BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The TRP measurement grid is added to the conformance test specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not implemented, there is no means to test TRP requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

BeWθ 	The Beam width in θ

BeWϕ	The Beam width in ϕ

BeWθ,REFSENS 	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.

BWChannel	Channel bandwidth (for E-UTRA)

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink.

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation

f	Frequency

f	Separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

ΔminSENS 	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS 	Difference between conducted reference sensitivity and OTA REFSENS

EISminsens		The EIS declared for the minSENS RoAoA

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset 	Separation between the Base Station RF bandwidth edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

FBW RF,high 	Upper RF bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low 	Lower RF bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC,high		Centre frequency of the highest transmitted/received carrier.

FC,low		Centre frequency of the lowest transmitted/received carrier.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset.

Fedge_high 	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset. Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT.

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fuw	Frequency offset of unwanted signal

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all transceiver units supporting the operating band.

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

Pmax,c,EIRP 	The maximum carrier EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP)

Pmax,c,TRP 	The maximum carrier output TRP per cell

Pmax,t,TRP 	The maximum total output power per cell

PRated,c,EIRP 	The rated carrier output EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP)

PRated,c,TRP 	The rated carrier output TRP

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

The angle in the reference coordinate system between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

PREFSENS	Conducted reference Sensitivity power level

TRPEstimate	Numerically approximated TRP

<End of modified section>

<Start of next modified section>

Annex F (normative): 
 TRP measurement grids

F.1		General

F.2	Spherical equal angle grid

F.2.2	Rayleigh sampling criteria

If DUT is mounted along the yz plane as shown in Figure F2.2-1, the sampling interval in the elevation and azimuth angle θgrid and φgrid via Rayleigh sampling criteria should be defined as following

where θr and φr are Rayleigh resolution angles in the elevation and azimuth plane. The Rayleigh resolution angles can be determined by

Where Dy is the diameter of radiated  parts of antenna along y-axis, Dz is the diameter of radiated parts of antenna along the z-axis and  is wavelength for the measured frequency.

If the antenna array are rectangular, the Rayleigh resolution angles could also be determined by measuring the first null beamwidth (FNBW) in the elevation and azimuth planes after the elements within the antenna array are calibrated. The Rayleigh resolution angle is  half of the FNBW in the elevation and azimuth planes.

Figure F2.2-1. Spherical coordinate for OTA conformance testing of DUT

F.3	Spherical equal area grid

TRPEstimate  shall be calculated as FFS

F.4 	Spherical Fibonacci grid

TRPEstimate  shall be calculated as FFS

F.5 Orthogonal 2 cut grid

TRPEstimate  shall be calculated as FFS

F.6 Wave vector space grid

If DUT is mounted along the yz plane as shown in Figure F2.2-1, the sampling interval in wave vector space should be defined as following:

where kyr and kzr are Rayleigh resolutions in y and z-axes in wave vector space. The Rayleigh resolutions can be determined by

where Dy is diameter of radiated  parts of antenna along  y-axis, Dz is diameter of radiated parts of antenna along the z-axis.

The sampling points in the wave vector space (Ky,m, Kz,n) should satisfy |Km,n|<1, where Km,n=yKy,m+zKz,n are the vectors of the sampling points in the wave vector space. According to the relationship between the wave vector space (Ky, Kz) and spherical coordinate  (φ,θ) as following, the sampling points in the spherical coordinate (φm,n,θn) could be derived.

Then, the total radiated power (TRP) in the wave vector space is determined by

where kgrid,y and kgrid,z are the sampling intervals in the wave vector space,  φm,n and θn are the azimuth and elevation angles of the sampling points  in the spherical coordinate system, Km,n=yKy,m+zKz,n are the vectors of the sampling points in the wave vector space and  |Km,n|<1 is to pick out points representing the radiated modes.

If the antenna array are rectangular, the Rayleigh resolution angles could also be determined by measuring the first null beamwidth (FNBW) in the elevation and azimuth planes after the elements within the antenna array are calibrated. The Rayleigh resolution angle is half of the FNBW in the elevation and azimuth planes.

<End of modified section>

<Start of next modified section>

Annex G (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-02 | RAN4#78 | R4-161118 |  |  |  | Specification structure | 0.1.0 |
| 2016-04 | RAN4#78bis | R4-162524 |  |  |  | R4-161370 - TP for TS 37.145-2: Adding Annex for relations between core and conformance requirements | 0.2.0 |
| 2016-05 | RAN4#79 | R4-164927 |  |  |  | R4-164717 - TP to TS 37.145 (part 2) sections 1-5 R4-164718 - TP for TS 37.145-2: Addition of radiated transmit power conformance test requirement in clause 6 R4-164719 - TP to TS 37.145 (part 2) section 7 - Radiated receiver characteristics | 0.3.0 |
| 2016-08 | RAN4#80 | R4-167179 |  |  |  | R4-166422 - TP to 3GPP TS 37.145-2 - clean up R4-166218 - TP for TS 37.145-2: Editorial correction on table numbers R4-166567 - TP to TS 37.145-2: Manufacturer declarations consistency improvements R4-166938 - TP for TS 37.145-2: Improvements of text in subclause 4.8 R4-166940 TP for TS 37.145-2: Improvements on text relating to the reference coordinate system R4-166939 TP to TR 37.145-2: Test Requirements derivation: Annexes A, B, C, E R4-166929 TP to TS 37.145-2: Measurement uncertainties and TT values R4-166931 TP for TS 37.145-2 - On OTA RX sensitivity requirements for AAS R4-167191 TP to TR 37.145-2: definition and test procedure clarifications for EIRP and EIS | 0.4.0 |
| 2016-09 | RP-73 | RP-161450 |  |  |  | Editorial corrections after review by ETSI editHelp | 1.0.0 |
| 2016-09 | RP-73 |  |  |  |  | TS was approved by RAN plenary | 13.0.0 |
| 2016-12 | RP-74 | RP-162422 | 0003 | - | A | Correction of Manufacturer declaration description list in TS 37.145-2 | 13.1.0 |
| 2017-03 | RP-75 | RP-170586 | 0005 | 1 | F | TS 37.145-2: Clarification of test procedure for radiated transmit power | 13.2.0 |
| 2017-03 | RP-75 | RP-170586 | 0006 | 1 | F | TS 37.145-2: Corrections | 13.2.0 |
| 2017-03 | RP-75 | - | - | - | - | Update to Rel-14 version (MCC) | 14.0.0 |
| 2017-06 | RP-76 | RP-171306 | 0011 |  | A | CR to TS 37.145-2: Isolation of Band 46 from the AAS BS specification | 14.1.0 |
| 2017-09 | RP-77 | RP-171968 | 0013 |  | A | CR to TS 37.145-2: Editorial corrections, Rel-14 | 14.2.0 |
| 2017-12 | RAN#78 | RP-172608 | 0015 | 1 | A | Correction of maximum steering directions declarations | 14.3.0 |
| Editors note: change history captured below is for the drafting of the big CR for the OTA updates – it will not be included in the final CR |  |  |  |  |  |  |  |
| 2018-02 | RAN4#86bis | R4-1805160 |  |  |  | R4-1802963 - Draft CR to TS 37.145-2 – Skeleton for OTA updates R4-1803385 - Draft CR to TS 37.145-2: Addition of description of co-location referenec antenna in sub-clause 4.15 |  |
| 2018-04 | RAN4#86bis | R4-1806003 |  |  |  | R4-1805163 - Draft CR to TS 37.145-2 – adding sections 6.2 R4-1805834 - draft CR for TS 37.145-2 - sections 1-5 R4-1805989 - TP for TS 37.145-2: Addition of TRP in sub-clause 3.1, 3.2, 3.3, 6.1 R4-1805831 - Draft CR for TS 37.145-2: Introduction of RIB interface in Figure 4.2-1 and addition of Figure 4.2-2 in sub-clause 4.2 R4-1805832 - Draft CR for TS 37.145-2: Update of transmit and receive configurations in sub-clause 4.8 with the co-location concept R4-1805853 - draft CR for TS 37.145-2 - sections 6.1 R4-1805854 - draft CR for TS 37.145-2 - 6.3, 6.4 R4-1805855 - Draft CR to TS 37.145-2 – OTA transmit ON/OFF power (6.5) R4-1805859 - Draft CR to TS 37.145-2 – adding a section 6.7.2 OTA occupied bandwidth R4-1805861 - draft CR for TS 37.145-2 - updating section 7.1 R4-1805862 - draft CR for TS 37.145-2 - 7.4, 7.5 R4-1805864 - draft CR for TS 37.145-2 - 7.8, 7.9 R4-1805990 - Draft CR to TS 37.145-2 – OTA frequency error (6.6.2) R4-1805991 - Draft CR to TS 37.145-2 – OTA time alignment error (6.6.3) R4-1805992 - Draft CR to TS 37.145-2 – OTA modulation quality (6.6.4) R4-1805914 - Draft CR on AAS BS radiated OTA REFSENS conformance test (7.3) |  |

<End of modified section>